3GPP TSG-CT6 Meeting #89	C6-180380

- Sophia-Antipolis, France, 10th Jul 2018 - 13th Jul 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 31.124 | CR | 0495 | rev | 1 | Current version: | 14.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | x | ME | x | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test cases for 3GPP PS Data Off |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Comprion GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  |  | Date: |  | 2018-07-12 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No tests defined for handling BIP when 3GPP PS Data Off is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test cases for E-UTRAN to cl. 27.22.4.27 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature is not tested |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 3.4, 27.22.4.27 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- 3.3	Table of optional features

Support of USIM Application Toolkit is optional for Mobile Equipment. However, if an ME states conformance with a specific 3GPP release, it is mandatory for the ME to support all functions of that release, as stated in table B.1, with the exception of the functions:

-	"Alpha identifier in REFRESH command supported by terminal";

-	"Event Language Selection";

-	"Proactive UICC: PROVIDE LOCAL INFORMATION (language)"; and

-	"Proactive UICC: LANGUAGE NOTIFICATION".

The support of letter classes, which specify mainly ME hardware dependent features, is optional for the ME and may supplement the USIM Application Toolkit functionality described in the present document. If an ME states conformance to a letter class, it is mandatory to support all functions within the respective letter class.

The supplier of the implementation shall state the support of possible options in table A.1.

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
|  | ………… |  |  |  |
| 132 | Class E: Terminal does support eFDD | O |  | pc_BIP_eFDD |
| 133 | Class E: Terminal does support eTDD | O |  | pc_BIP_eTDD |
|  | ………. |  |  |  |
| xxx | Terminal supports 3GPP PS data off | O |  | O_PS_Data_Off |

- 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| Item | Description | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | [………..] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | OPEN CHANNEL 27.22.4.27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | […………………] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Immediate link establishment, E-UTRAN, bearer type '03', default EPS bearer | Rel-8 | 6.5 |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C223 | C223 | E.1/89 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | OPEN CHANNEL, BIP is not a 3GPP PS data off exempt service | Rel-14 | 6.X |  |  |  |  |  |  |  |  |  |  |  | CXXB | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | OPEN CHANNEL, BIP is a 3GPP PS data off exempt service | Rel-14 | 6.Y |  |  |  |  |  |  |  |  |  |  |  | CXXB | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | [……] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  | [……………..] |  |
| --- | --- | --- |
| C226 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/181 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_DEAC_UICC |
| C227 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/183 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_eDRX_ SUSPEND_UICC |
| CXXB | IF ((A.1/132 OR A.1/133) AND A.1/152) AND A.1/xxx THEN M ELSE N/A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND O_SMS_SGs_MT AND O_PS_Data_Off |
| [………….] |  |  |
| TCEP001 | IF NOT A.1/84 THEN during the test execution, the display or the non-display of any alpha identifier, text string or icon shall be treated as successfully verified. |  |

[……]

- 27.22.4.27	OPEN CHANNEL

[…..]

- 27.22.4.27.6	Open Channel (related to E-UTRAN)

27.22.4.27.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.6.2	Conformance requirements

The ME shall support the class "e" commands and E-UTRAN as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 9.2, clause 8.2, clause 8.15, clause 8.52, clause 8.59, clause 8.61 and Annex S

-  	TS 23.107 [30], cl 9.1.2.2, clause 9.1.2.3,

-  	TS 23.203 [31], cl 6.1.7.2,

-  	TS 24.301 [32], cl 9.9.4.3,

-  	TS 36.508 [33], cl 6.6.1.

- 	TS 24.011 [11], cl 10.

- 	TS 31.102 [14], cl 4.2.109

27.22.4.27.6.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (OK); or

-	TERMINAL RESPONSE (Command performed with modification); or

-	TERMINAL RESPONSE (User did not accept the proactive command);

-	TERMINAL RESPONSE (ME currently unable to process command);

to the UICC after the ME receives the OPEN CHANNEL proactive command while accressing E-UTRAN/EPC. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

To verify that the ME sets up a PDN connection with the Access Point Name (APN) indicated in the Open Channel command which differs from the default APN.

To verify that the ME uses the Default EPS bearer when Bearer Type 3 is indicated in the Open Channel command.

To verify that the ME does not disconnect the Deafult EPS bearer when the user rejects the user confirmation of the Open Channel command.

To verify that the ME sends the TERMINAL RESPONSE (ME currently unable to process command) if the 3GPP PS data off status is "active" and the UE is not configured with indication that Bearer Independent Protocol is a 3GPP PS data off exempt service.

To verify that the ME sends the TERMINAL RESPONSE (OK) if the 3GPP PS data off status is "active" and the UE is configured with indication that Bearer Independent Protocol is a 3GPP PS data off exempt service.

27.22.4.27.6.4	Method of test

27.22.4.27.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name: 			Any value other than TestGp.rs or Test12.rs

User login:						UserLog

User password:					UserPwd

UICC/ME interface transport level

Transport format:			TCP

Port number: 				44444

Data destination address:	01.01.01.01 (as an example)

Note: 	If a data destination address different to 01.01.01.01 is used then the same value is used in the content of the affected Open Channel commands and the network simulator setup and related UE settings might require a corresponding adaptation.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME’s default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

For sequence 6.1, 6.2 and 6.3 the E-USS shall be able to support 2 active PDN connections at the same time.

In case the ME supports A.1/173 AND A.1/174 AND A.1/178, for sequence 6.1 and 6.3 the NB-SS shall be able to support 2 active PDN connections at the same time.

For sequence 6.X service n°117 is "available" in EFUST and the Bearer Independent Protocol is not marked in EF3GPPPSDATAOFF as a 3GPP PS data off exempt service. Also SMS over SGs is used to send and receive short messages.

For sequence 6.Y service n°117 is "available" in EFUST and the Bearer Independent Protocol is marked in EF3GPPPSDATAOFF as a 3GPP PS data off exempt service. Also SMS over SGs is used to send and receive short messages.

27.22.4.27.6.4.2	Method of test

Expected Sequence 6.1 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '02')

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER  ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 7. |
| 2 | UICC  ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 3 | ME  UICC | FETCH |  |
| 4 | UICC  ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 5 | ME  USER | The ME may display channel opening information |  |
| 6 | ME  E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | E-USS/NB-SS  ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 8 | ME  E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME  UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |

PROACTIVE COMMAND: OPEN CHANNEL 6.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name: 			TestGp.rs

Text String:					"UserLog" (User login)

Text String:					"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number: 	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class: 	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

[….]

Expected Sequence 6.X (OPEN CHANNEL, BIP is not a 3GPP PS data off exempt service)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER  ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 2 | USER  ME | Set 3GPP PS data off status is "active" |  |
| 3 | ME  E-USS | Send a Request Bearer Resource Modification message | The ME indicates the change of 3GPP PS Data Off Status to the PDN GW via the PCO (Protocol Configuration Options) |
| 4 | E-USS  ME | SMS-PP Data Download 6.X.1 | Send SMS over SGs |
| 5 | ME  UICC | ENVELOPE: SMS-PP DOWNLOAD 6.X.1 |  |
| 6 | UICC  ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 7 | ME  E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 8 | UICC  ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 9 | ME  UICC | FETCH |  |
| 10 | UICC  ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 11 | ME  UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.X.1 | [ME currently unable to process command] |

SMS-PP (Data Download) Message 6.X.1

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 53 | 68 | 6F | 72 | 74 | 20 | 4D | 65 | 73 |
|  | 73 | 61 | 67 | 65 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 6.X.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI		"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP		TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON		International number

NPI		"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 21 | 43 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 53 | 68 |
|  | 6F | 72 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.X.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 6.Y (OPEN CHANNEL, BIP is a 3GPP PS data off exempt service)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER  ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 2 | USER  ME | Set 3GPP PS data off status is "active" |  |
| 3 | ME  E-USS | Send a Request Bearer Resource Modification message | The ME indicates the change of 3GPP PS Data Off Status to the PDN GW via the PCO (Protocol Configuration Options) |
| 4 | E-USS  ME | SMS-PP Data Download 6.X.1 | Send SMS over SGs |
| 5 | ME  UICC | ENVELOPE: SMS-PP DOWNLOAD 6.X.1 |  |
| 6 | UICC  ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 7 | ME  E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 8 | UICC  ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 9 | ME  UICC | FETCH |  |
| 10 | UICC  ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 11 | ME  USER | The ME may display channel opening information |  |
| 12 | ME  E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 13 | E-USS/NB-SS  ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 14 | ME  E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 15 | ME  UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |

27.22.4.27.6.5	Test requirement

The ME shall operate in the manner defined in expected sequences 6.1 to 6.5.