NB M2M - Channel Mapping

Introduction

The typical functionalities of MAC layer for Cellular IoT were introduced in GERAN#63 [1]. This paper introduces the channel mapping function for S1-based architecture and Gb-based architecture.

This paper is an update of GP-140850 proposed in GERAN #64.

Channel mapping

- 2.1 Channel mapping for Gb-based architecture

As the Gb-based interface is connectionless, it is natural that data and signalling would terminate at MAC layer in radio interface. Therefore there is no need to have additional logical channels, and MAC layer has no channel mapping function. The channel mapping between transport channels and physical channels is supported by PHY layer as shown in Figure 1.

Figure 1. Channel mapping for Gb-based architecture

In the above figure, there are two types of channels as explained below:

- physical channels

The physical channels are listed in Table 1. Each physical channel type is defined by different requirement of physical layer performance.

Table 1. Physical channels

| PHY channel name | Acronym | Downlink | Uplink | Description |
| --- | --- | --- | --- | --- |
| Physical Uplink Shared Channel | PUSCH |  | X | uplink data and uplink control signalling are carried |
| Physical Downlink Shared Channel | PDSCH | X |  | downlink data and downlink control signalling are carried |
| Physical Broadcast and Synchronization Channel | PBSCH | X |  | basic system information and synchronization information are carried |
| Physical Broadcast Channel | EPBCH | X |  | extended system information is carried |

- transport channels

The transport channels are listed in Table 2. Each transport channel type is defined by the transport manner on radio interface.

Table 2. Transport channels

| Transport channel name | Acronym | Downlink | Uplink | Description |
| --- | --- | --- | --- | --- |
| Broadcast Channel | BCH | X |  | Basic system information is carried |
| Extend Broadcast Channel | E-BCH | X |  | Other system information are carried |
| Downlink Shared Channel | DL-SCH | X |  | Downlink data and control message are carried |
| Paging Channel | PCH | X |  | Paging messages are carried |
| Uplink Shared Channel | UL-SCH |  | X | Uplink data and control message are carried |
| Random Access Channel | RACH |  | X | Random access message is carried |

Proposal 1: It is proposed to adopt the channel mapping for Gb-based architecture into the TR.

- 2.2 Channel mapping for S1-based architecture

As the S1 interface is connection oriented, the PDCP and RRC layer have to be maintained to support S1 interface without modifications on MME side. In this case, the channel mapping would have three layers as shown in Figure 2. The channel mapping between logical channels and transport channels is supported by MAC layer. The channel mapping between transport channels and physical channels is supported by PHY layer same as for Gb-based architecture.

The uplink mapping is shown as red lines, and the downlink mapping is shown as blue lines. Most channels would have mapping on each layer with the only exception on RACH channel, as random access procedure is terminated in MAC layer.

Figure 2. Channel mapping for S1-based architecture

In the above figure, the definitions of transport channels and physical channels are the same as in Sec 2.1. The logical channels are explained below:

- logical channels

The MAC layer provides data transfer services on logical channels. Each logical channel type is defined by what type of information is transferred.

The logical channels are listed in Table 3.

Table 3. Logical channels

| Logical channel name | Acronym | Control channel | Traffic channel | Description |
| --- | --- | --- | --- | --- |
| Broadcast Control Channel | BCCH | X |  | System information is carried |
| Paging Control Channel | PCCH | X |  | Paging message is carried |
| Common Control Channel | CCCH | X |  | RRC signalling on SRB0 is carried [2] |
| Dedicated Control Channel | DCCH | X |  | RRC signalling on SRB1 and SRB2 are carried [2] |
| Dedicated Traffic Channel | DTCH |  | X | User data is carried |

Proposal 2: It is proposed to adopt the channel mapping for S1-based architecture into the TR.

Conclusion

In order to support data transmission on radio interface, mapping of different channels is designed in NB M2M solution. It is proposed to agree proposal 1 and proposal 2.

References

  - GP-140571, “Typical Functionalities of MAC layer”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN#63.

  - 3GPP TS 36.331, “Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC)”.