Proposed Text for the TR on NB M2M Channel Mapping

Introduction

A new study item named Cellular IoT was approved at GERAN#62 (see [1]). Channel mapping is the basic function for the whole system design which has been proposed in [2]. This document contains a text proposal for the Cellular IoT TR on the channel mapping function of the NB M2M solution which is the result of consolidating [2].

Proposed text for the TR

First Change

- 7.1	Narrow Band M2M (NB M2M)

- 7.1.1	Link layer aspects

- 7.1.1.x 	Channel mapping

- 7.1.1.x.1 Channel mapping for Gb-based architecture

As the Gb-based interface is connectionless, it is natural that data and signalling would terminate at MAC layer in radio interface. Therefore there is no need to have additional logical channels, and MAC layer has no channel mapping function. The channel mapping between transport channels and physical channels is supported by PHY layer as shown in Figure 7.1.1.x-1.

Figure 7.1.1.x-1. Channel mapping for Gb-based architecture

- physical channels

See the Sec 7.1.2 and Sec 7.1.3 as references.

- transport channels

The transport channels are listed in Table 7.1.1.x-1. Each transport channel type is defined by the transport manner on radio interface.

Table 7.1.1.x-1. Transport channels

| Transport channel name | Acronym | Downlink | Uplink | Description |
| --- | --- | --- | --- | --- |
| Broadcast Channel | BCH | X |  | Basic system information is carried |
| Extend Broadcast Channel | E-BCH | X |  | Other system information are carried |
| Downlink Shared Channel | DL-SCH | X |  | Downlink data and control message are carried |
| Paging Channel | PCH | X |  | Paging messages are carried |
| Uplink Shared Channel | UL-SCH |  | X | Uplink data and control message are carried |
| Random Access Channel | RACH |  | X | Random access message is carried |

- 7.1.1.x.2 Channel mapping for S1-based architecture

As the S1 interface is connection oriented, the PDCP and RRC layer have to be maintained to support S1 interface without modifications on MME side. In this case, the channel mapping would have three layers as shown in Figure 7.1.1.x-2. The channel mapping between logical channels and transport channels is supported by MAC layer. The channel mapping between transport channels and physical channels is supported by PHY layer.

The uplink mapping is shown as red lines, and the downlink mapping is shown as blue lines. Most channels would have mapping on each layer with the only exception on RACH channel, as random access procedure is terminated in MAC layer.

Figure 7.1.1.x-2. Channel mapping for S1-based architecture

The definitions of transport channels and physical channels are the same as in Sec 7.1.1.x.1.

- logical channels

The MAC layer provides data transfer services on logical channels. Each logical channel type is defined by what type of information is transferred.

The logical channels are listed in Table 7.1.1.x-2.

Table 7.1.1.x-2. Logical channels

| Logical channel name | Acronym | Control channel | Traffic channel | Description |
| --- | --- | --- | --- | --- |
| Broadcast Control Channel | BCCH | X |  | System information is carried |
| Paging Control Channel | PCCH | X |  | Paging message is carried |
| Common Control Channel | CCCH | X |  | RRC signalling on SRB0 is carried (TS 36.331 for reference) |
| Dedicated Control Channel | DCCH | X |  | RRC signalling on SRB1 and SRB2 are carried (TS 36.331 for reference) |
| Dedicated Traffic Channel | DTCH |  | X | User data is carried |

End of Changes

References

  - GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, VODAFONE Group Plc., GERAN#62.

  - GPC150038, “NB M2M - Channel Mapping”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd., GERAN Adhoc#1 on FS_IoT_LC.