NB M2M - Paging for Cellular IoT

Introduction

At GERAN#62 a new SI was agreed to study Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things. An NB M2M radio access technology was introduced and to support NB M2M access, the Paging mechanism should be considered to match the M2M requirement. In this contribution, the Paging procedure based on NB M2M access for Cellular IoT will be discussed.

This contribution is an update of [6] presented at GERAN#64. Major updates have been highlighted in red colour.

Discussion

Paging from Core Network

Paging from MME

In S1 based architecture, the MME initiates the paging procedure by sending the PAGING message to the BS. Currently, a Paging message sent on S1 is as follows.

Table 1 Paging on S1 interface [1]

| IE/Group Name | Presence | Comments |
| --- | --- | --- |
| Message Type | M |  |
| UE Identity Index value | M | It equals to IMSI mod 1024, used for calculation of DRX cycle for Paging |
| UE Paging Identity | M | It’s always S-TMSI (40bits) to identify the UE paged in the radio interface. In abnormal case, IMSI will be used. |
| Paging DRX | O | If it appears, the Min {paging DRX, default DRX value broadcast in system information} will be used for the paging distribution in the radio interface. |
| CN Domain | M | Domain Indication, PS or CS. For BS supporting NB M2M, this indication may be discarded. Irrelevant with cellular IoT |
| List of TAIs |  | Paging will be sent in cells which belong to tracking areas as indicated in this IE |
| >TAI List Item |  |  |
| >>TAI | M |  |
| CSG Id List |  | CSG related Irrelevant with cellular IoT |
| >CSG Id | O |  |
| Paging Priority | O | Paging priority indication is included only if MME receives a Downlink Data Notification with ARP associated with MPS. Irrelevant with cellular IoT |

According to the PAGING message from the MME, the BS can distribute the paging over the radio interface with the UE identity in the tracking areas included in the TAI list. The Paging DRX will be used by the BS to calculate when the UE will wake up for paging.

Paging from SGSN

On Gb based architecture, When an SGSN initiates the paging procedure for GPRS services as defined in 3GPP TS 24.008, it shall send one or more PAGING-PS PDUs to the BSS

Table 2 Paging on Gb interface [2]

| Information elements | Presence | Comments |
| --- | --- | --- |
| PDU type | M |  |
| IMSI | M | Paging Identity, 5-10bytes |
| DRX Parameters | O | The UE shall also indicate within the DRX parameters whether it supports the split pg cycle option on CCCH. |
| BVCI a) | C | One and only one of the conditional IEs shall be present, indication of the cells within which the BS shall page the UE |
| Location Area (note) | C |  |
| Routeing Area (note) | C |  |
| BSS Area Indication (note) | C |  |
| PFI | O | UE specific information, enabling a BS to execute the paging procedure in an UE specific manner Irrelevant with cellular IoT |
| ABQP | O |  |
| QoS Profile | M |  |
| P-TMSI | O | Paging Identity |

The content of Paging on Gb interface is almost the same with S1 interface except the DRX parameters. For GPRS, the DRX parameter indicates the supporting of the split pg cycle. However, the DRX parameter also includes the CN Specific DRX cycle length coefficient and DRX value for S1 mode as in Table 3. In other words, when the DRX parameter is sent to BS of NB M2M, the BS can only identify the DRX value field in this parameter and neglect the other info. By the way, the SGSN only keeps the DRX parameter the same as it is sent from UE to SGSN by ATTACH/RAU and transfers it to BS without any change. In such a way, the DRX parameter can still be used by BS to calculate when the UE will wake up for paging as mentioned in 2.1.2.

Table 3 Content of DRX Parameter [3]

| 8 | 7 | 6 | 5 |  | 4 | 3 |  | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRX parameter IEI |  |  |  |  |  |  |  |  |  | octet 1 |
| SPLIT PG CYCLE CODE |  |  |  |  |  |  |  |  |  | octet 2 |
| CN Specific DRX cycle length coefficient and DRX value for S1 mode |  |  |  | SPLIT on CCCH |  |  | non-DRX timer |  |  | octet 3 |

Requirement for Core Network in NB M2M

Considering the requirement of coverage enhancement in Cellular IoT, Coverage Class specific paging channel are planned in NB M2M access for power saving of the UEs in good coverage condition and resource utilization. In this case, the coverage class for each paged UE should be provided by core network in PAGING message to BS.

Besides that, the core network should store and maintain the coverage class information for a specific UE. The maintenance procedure is assisted by both UE and BS which is described in section 2.2.3.

Paging over the radio interface

Paging Channel Configuration

In NB M2M, The Paging message is carried on the Paging Control Channel (PCCH). Such a PCCH will be mapped via PCH on the Physical Downlink Shared Channel (PDSCH) as shown in Figure 1.

Figure 1 Channel Mapping for PCCH

The PCCH is coverage class [4] specific to separate resources for UEs in different coverage conditions. Therefore the resource utilization can be enhanced compared with multiplexing UEs in different coverage conditions. If the paging for UEs in different coverage conditions is multiplexed in the same resource, the PAGING messages will always to be repeated the same number of times as the worse coverage class, which will significantly decrease the paging capacity.

The presence of PCCH for a specific coverage class is scheduled by the corresponding DCI for the same coverage condition. When a PCCH is scheduled, the information below is present in the DCI with a specific value for P-RNTI, from which the UE will know the exact location of the PCCH and perform paging monitoring.

Table 4 PCCH Allocation in DCI

| Information Element | length | Description |
| --- | --- | --- |
| P-RNTI | 20bits | This identifier indicates the subsequent resource allocation for PCCH. A fixed value will be used for all UEs in the cell. |
| Channel ID | 4bits | Indicate the physical location of the PCH channel |
| Start Indicator | 10bit | The start of the resource allocation on the corresponding physical channel, the value means the Nth slot in current DCI interval |
| Duration | 7 bit | the duration of the resource allocation for PCCH, the value indicates how long a UE should keep receiving |
| MCS | 3bit | Modulation and scheme for the PCCH |

Consequently, when the UE identifies the P-RNTI from the DCI monitoring, the UE will listen to the corresponding PCCH for PAGING message. An example is given in Figure 2, where the DCI indicates that the PCCH will be transmitted on the same channel than the DCI by Channel ID and will start from the 15th slot in the current DCI interval and last for 12 slots. The MCS is also indicated in the DCI. Such a DCI monitoring for PCCH is doing periodically for a specific UE.

Figure 2 PCCH allocation

Reception of paging

Similarly as legacy mechanism, the UE also needs to use Discontinuous Reception (DRX) in idle mode in order to reduce power consumption. One Paging Occasion (PO) for NB M2M access is a DCI interval where P-RNTI is included in the DCI. One Paging Frame (PF) is one Super Frame containing one or multiple Paging Occasion(s). When DRX is used the UE needs only to monitor one PO per DRX cycle.

PF and PO is determined by following formulae using the DRX parameters provided in System Information:

PF is given by following equation:

SFN mod T= (T div N)*(UE_ID mod N)

Index i_s pointing to PO will be derived from following calculation:

i_s = floor (UE_ID/N) mod Ns

The following Parameters are used for the calculation of the PF and i_s:

-	T: DRX cycle of the UE. T is determined as min [value in system information, value via CN indication]. If UE specific DRX is not configured by upper layers, the default value broadcast in the system information is applied.

-	N: N is used to group the super frame where the DCI for PCCH can only appear in PF. It can be selected from {T, T/2, T/4, T/8}.

For example,

when N= T, it means the DCI for PCCH could be appeared in each super frame;

when N= T/2, it means the DCI for PCCH could be appeared in each two super frame;

-	Ns: The DCI is separated into Ns groups for each coverage class. The UE can select one of the Ns groups based on its IMSI for the DCI monitoring for PCH. Ns is calculated by the following equation where the unit of DCI interval is number of frames.

Ns= 64/DCI interval

-	UE_ID: IMSI mod 1024 (for S1 based architecture) or IMSI (for Gb based).

When the PF and PO is identified, the UE will perform discontinuous reception for paging as below.

Figure 3 PO Locations example for Paging, Ns=16

The value of T is the number of super frame. It could be from 0 to 65535 where there are 65536 super frames in NB M2M access. The length of one super frame is 5.12s. Examples are listed in Table 5 for paging cycle setting.

Table 5 Examples of Paging Cycle Setting

| T | DRX (Second) | DRX (Hour: min: Second) |
| --- | --- | --- |
| 1 | 5.12 | NA |
| 2 | 10.24 | NA |
| 4 | 20.48 | NA |
| 8 | 40.96 | NA |
| 16 | 81.92 | 1min21.92s |
| 32 | 163.84 | 2min43.84s |

Coverage Class Maintenance

In coverage class specific paging, the Core Network is required to provide the corresponding coverage class for each paged UE.

The initial coverage class could be provided to CN by ATTACH procedure.

The coverage class is maintained during the following conditions in Gb architecture,

- When the UE is in the transfer mode, the UE specific coverage class is always piggybacked in the UL-UNITDATA PDU containing the last UE’s LLC PDU as in Figure 4.

Figure 4 Transfer mode

- When the UE is in the idle mode and there is no data to be sent, the detection of coverage class change will trigger a RACH procedure with the cause value as coverage class change. With this cause value, the BS will inform the SGSN also by UL-UNITDATA PDU with the coverage class information as in Figure 5. However, since there is no real LLC PDU sent from UE, the BS will set the Length Indicator of LLC-PDU IE in the UL-UNITDATA to zero, which has been allowed by TS 48.018.

Figure 5 Idle mode without data sending

Similarly, in S1 based architecture,

- When the UE is in connected mode, the new IE can be carried by UE CONTEXT RELEASE COMPLETE from BS to MME.

- When the UE is in idle mode, if the Coverage Class Change has been detected, the UE will ask for a RRC connection setup without any real NAS signalling transaction and the BS will follow the value cause of Coverage Class Change in Channel Request to inform the MME about Coverage Class also in UE CONTEXT RELEASE COMPLETE message.

The CN should replace the UE specific Coverage Class with the latest one and send to BS in PAGING message

Compared with the way to update the coverage class always by RAU, it obviously avoids the countable NAS signalling and longer transmission time in radio interface. It is benefit for power saving.

With this maintenance, it also avoids the blind paging sending in all coverage classes, which has more efficient utilization of resource.

Paging Message via radio interface

In one unit of downlink paging resource, one Paging Message will be carried, containing paging for multiple UEs as showed in Table 6.

The maximum number of UEs which can be paged in one Paging Message is limited by the resource allocation. For example, if the allocation is limited to 60 bytes because of the MCS and the length of time of the allocation, then there can be up to 12 UEs paged by S-TMSI at the same time.

Table 6 Example of Paging Message in radio interface

| IE/Group Name | Presence | Range | Semantics description |
| --- | --- | --- | --- |
| UE identity List |  | 1 to <maxNrOfPagingRec> |  |
| >UE identity | M | S1: S-TMSI 5 bytes Gb: IMSI 5-10 bytes | Provides the NAS identity of the UE that is being paged. S-TMSI or IMSI |

Paging Capacity Evaluation

According to the requirement in the traffic model, the requirement of PAGING capacity is about 4905 users per hour per cell.

An example of PAGING capacity for NB M2M is listed in Table 7 when assuming N=T.

Table 7 Example of Paging Capacity

| DCI interval (ms) | Duration of Paging message(ms) | Payload(including 6 byte cost for header and CRC) [5] | Max no. of UE paged within a PAGING message (S-TMSI is used) | i_s | Appearance of Paging during an hour | Capacity for Paging per hour | Occupation on one PDSCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 320 | 30 | 18 (MCS 5) | 2 | 16 | 11250 | 22500 | 9.37% |
| 320 | 30 | 18 (MCS 5) | 2 | 2 | 1406 | 2812 | 1.17% |
| 320 | 30 | 18 (MCS 5) | 2 | 1 | 703 | 1406 | 0.58% |
| 5120 | 480 (30*16) | 18 (MCS 1) | 2 | 1 | 703 | 1406 | 9.37% |
| 5120 | 640 (40*16) | 24 (MCS 1) | 3 | 1 | 703 | 2109 | 12.5% |
| 5120 | 800 (50*16) | 30 (MCS 1) | 4 | 1 | 703 | 2812 | 15.6% |
| 5120 | 960 (60*16) | 36(MCS1) | 6 | 1 | 703 | 4218 | 18.75% |
| 5120 | 1120 (70*16) | 42 (MCS1) | 7 | 1 | 703 | 4921 | 21.88% |

Assuming there are two PDSCHs in a cell,

- When the paging is always sent on the worst coverage, according to Table 7, at least 4921 users can be supported with cost of 10.94% of all PDSCH resource

- When the paging is sent per coverage class and assuming 50% users (2453) can be served by normal coverage and the other 50% users can be served by enhanced coverage. According to Table 7, 2812 users of normal coverage can be supported by 1.17% of one PDSCH and 2812 users of enhanced coverage can be supported by 15.6 % of one PDSCH. Then (1.17% + 15.6%)/2=8.39 % of all PDSCH is enough for the 4921 users.

It is conclued that when the paging is sent according to different coverage classes, less resources is consumed compared with no distinguishing of coverage class.

Conclusions

This contribution has discussed the paging mechanism for NB M2M access, and proposes to adopt this mechanism into the TR.

References

  - TS 36.413, “3GPP TS E-UTRAN: S1 Application Protocol (S1AP)”.

  - TS 48.018, “3GPP Base Station System (BSS) -Serving GPRS Support Node (SGSN);BSS GPRS Protocol (BSSGP)”.

  - TS 24.008, “Technical Specification Group Core Network and Terminals; Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”.

  - GPC150037, “NB M2M - Overview of Coverage Class”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. , GERAN Adhoc#1 on FS_IoT_LC.

  - GP-140870, “Proposed Text for the TR on Refinement of the Clean-slate Physical Layer Design”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. , GERAN#64.

  - GP-140854, “NB M2M - Paging for Cellular IoT”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd. , GERAN#64.