Cooperative Ultra Narrow Band technology for Cellular IoT

# Scope

This document is an overview of a new long-range and low-power radio technology that addresses the connectivity for devices of the Internet of Things (IoT). Already up and running in several countries within stand-alone networks, this technology can be easily adapted to meet Cellular IoT requirements, listed in [1].

# Introduction

Many studies forecast the rapid growth of machine-to-machine (M2M) connectivity and its low-cost and low-power side, the Internet of Things (IoT). Answering these needs is a significant opportunity for the 3GPP ecosystem but also a major challenge if we consider the key requirements for IoT connectivity:

improved coverage compared to legacy GSM/EDGE,

ultra low power consumption (battery operated devices),

ultra low cost.

Different solutions can be designed to meet these requirements, based on the evolution of existing GSM/EDGE or based on brand new radio implementation. In this paper, we present a disruptive approach, which we consider to be a good candidate for Cellular IoT. It uses well-known ultra narrow band (UNB) radio technology but in a new network-oriented implementation. We call this approach "Cooperative UNB" (C-UNB) in this paper.

The following section 3 describes the key features of the C-UNB technology. Section 4 gives indications about its implementation in the case of Cellular IoT.

# Overall description

## Radio transmission

The requirements for improved coverage and ultra low power devices imply many tradeoffs when selecting a radio technology. In C-UNB technology, the tradeoff consists in using extremely low data rates. These data rates give ultra narrow band transmission, and hence very good reception sensitivity. High reception sensitivity is a key advantage when deploying a large network, because it reduces the number of base stations and it helps to get good indoor coverage. Ultra narrow band transmission, obtained with ultra low data rates, is also a key feature to get an energy-efficient medium access control, as explained in section 3.2 hereunder.

C-UNB networks, which are already deployed, use the sub-gigaHertz ISM spectrum. The values in table 1 are computed with the link budget elements depicted in figure 1. These values are compliant with ISM requirements and give the reader a flavor of the C-UNB radio technology. Equivalent values are achievable in GSM/EDGE sub-gigaHertz band.

Figure 1: Link budget elements

Table 1: Tx and Rx Maximum Coupling Loss of C-UNB networks using ISM sub-gigaHertz band.

|  | Modulation | Data rate (bps) | Tx Power | Compound TX Antenna Gain | Compound RX antenna gain | Rx sensitivity | MCL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| upstream | BPSK | 100 | +14dBm | 0dB | +6dB | -142dBm | +162dB |
| downstream | GFSK | 600 | +27dBm | +6dB | 0dB | -130dBm | +163dB |

Note 1:

Rx sensitivity values given in table 1 are those of actual implementation:

in device, -130dBm is achievable with common sub-gigaHertz chipsets

in base station, -142dBm is achieved through advanced processing techniques.

Note 2:

BPSK modulation is not a standard feature in many radio chipsets, but its implementation is easily manageable in most radio chipsets available on the market.

## Unsynchronized devices

In previous section, we have seen that UNB radio transmission gives very good MCL -and hence very good coverage- even with power transmission as low as 25mW. This feature is key for reducing power consumption within devices. To go a step further in power saving, medium access in C-UNB technology is made without any kind of synchronization or signaling messages.

A device does not listen for the network when it has nothing to transmit or receive. When it is ready for transmission, it transmits its radio packet on three different frequencies (see figure 2) without prior carrier sense or equivalent Listen Before Talk (LBT) techniques. The three frequencies are chosen pseudo-randomly in the allocated frequency band by the transmitting device. They are different for each message.

This medium access, equivalent to the well-known ALOHA, may lead to many collisions of radio packets unless the network load is kept low (i.e. 15%). In the case of C-UNB technology, the radio access load is kept low because:

IoT devices transmit short messages

IoT devices have a low daily throughput

thousands of UNB radio channels are available even in a spectrum bandwidth of a few hundreds of kHz.

Downlink transmission uses piggy-backing: a device willing to receive a radio packet from the network opens a reception window at a given time after its upstream transmission. In case of multiple receptions (see section 3.3), the network selects the best base station for transmitting the downlink message.

Figure 2: Pseudo-random selection of frequencies during transmission in C-UNB technology

## Cooperative reception

In common cellular networks, traffic in a given area is split between base stations in the communication range of the mobile devices. This is done by means of cell and/or frequency selection made by the network and the devices. This process is very efficient to get high network capacity, especially in the case of quite long session. Its drawback, in term of power consumption, relies in the periodic exchange of signaling messages between the network and the devices.

C-UNB technology target IoT where each single device transmits small messages (i.e. 10-20 bytes of application data). In such case, it is more energy efficient to let devices transmitting their radio frames when needed (i.e. randomly) and to increase the listening capability of the network.

Figure 3: Cooperative reception in C-UNB radio access technology

C-UNB base station receivers listen the full-allocated spectrum continuously. Each received radio packet is decoded and transmitted for back-end processing. As UNB transmission gives very high MCL and hence very long-range communication, each single transmission of a given device may be received by several base stations (see figure 3). For the sake of simplicity, these multiple receptions are not managed as coherent decoding but as several receptions of the same radio packet, which is demodulated by several C-UNB base stations and then de-duplicated in the back-end servers. This innovative approach is called "cooperative reception".

## Radio packet format

Radio packet format in present C-UNB networks is a tradeoff between payload size, data rate and maximum transmission time, limited by ISM band regulations. Figure 4 depicts the radio packet format, which is used in present deployment in various European countries. In the case of Cellular IoT, this format can be easily adapted to meet new radio constraints.

Figure 4: radio packet format in C-UNB upstream (from [2])

One can notice that each radio packet is tagged with a unique device identifier only. This unique identifier (i.e. serial number) is used for identifying the source of each radio packet in upstream and the destination in downstream. The limited size of the identifier (i.e. 32 bits) avoids the need of short temporary identifiers, as we have in GSM/EDGE with the TMSI.

# Implementation in 3GPP ecosystem

C-UNB technology is currently implemented in stand-alone networks, operating in sub-gigaHertz ISM bands. These networks have a specific architecture [3] made of dedicated base-stations connected to back-end servers, which give access to the Internet. In the case of Cellular IoT, the C-UNB technology has to be compliant with the 3GPP ecosystem. Many points of compliance have to be addressed; we give hints for two ones: frequency bands and base station implementation.

The C-UNB radio technology can cope with very limited spectrum bands, i.e. a few hundreds of kHz. The frequency band can be in one chunk or split in several sub-bands. If we consider the 200kHz frequency bands available in the GSM/EDGE legacy spectrum, it is proposed to allocate one or several channels to carry C-UNB technology in the case of Cellular IoT.

When we consider the deployment of C-UNB base stations, they can be co-located with BTS with separated antenna or integrated in BTS, when they are built with software defined radio (SDR) equipments. Connectivity with the core network could reuse already defined interfaces such as Gb or S1.

# Conclusion

This paper gives an overview of an innovative low-cost, low-power, long-range radio technology for nation-wide access networks devoted to the connectivity of IoT devices. This technology is named "Cooperative Ultra Narrow Band" in this paper. As such, it is already deployed in stand-alone networks, but it can be adapted to the 3GPP ecosystem. It meets or exceeds many requirements defined for the Cellular IoT. Therefore, it is proposed to have it as a possible candidate for Cellular IoT.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62

[2]	ETSI GS LTN 003 V1.1.1 (2014-09), Low Throughput Networks (LTN); Protocols and Interfaces

[3]	ETSI GS LTN 002 V1.1.1 (2014-09), Low Throughput Networks (LTN); Functional Architecture