# EC-GSM – Current vs vs compact burst mapping

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One of the main objectives in FS_IoT_LC is to increase the coverage compared to existing GPRS services. In case of evolving GSM, the most straight forward way is to make use of blind repetitions. This is what has been proposed for the Extended Coverage GSM (EC-GSM) concept (earlier known as GSM Evolution), see [2].

In this contribution, the burst mapping of traffic channels and their associated control channels are considered. It is concluded that the current burst mapping over a block is maintained on the DL while a more compact burst mapping is used for UL transmissions.

# Compact vs current burst mapping

As discussed during GERAN#63 and in [5], it is possible to use a more compact burst mapping than used today, in case of repetitions over multiple TTIs.

The separation between instances of the same burst is with the current burst mapping separated by four TDMA frames. In a more compact burst mapping, the bursts could instead be mapped on consecutive TDMA frames. Both mapping options are shown in Figure 1 for eight times repetition

In the normal burst mapping the separation in time between the first and last transmission of the same burst is roughly 20 ms, while for the more compact burst mapping illustrated to the right in Figure 1 the separation between the first and last burst repetition is only around 7 ms, i.e. a reduction of 65%. When increasing the repetition interval to 16 the difference gets closer to 75%.

Figure 1. Current burst mapping (left) and more compact burst mapping (right).

This improves the time coherency of the transmission and hence the performance can be improved.

As is shown in Figure 1, the TTI is effectively prolonged by the use of compact burst mapping, and hence it is not beneficial to be used on the DL where devices of different coverage will monitor the same DL channel. A shorter monitoring time will allow a more energy efficient operation. Also, to allow for backwards compatibility, the change in block structure should preferably be minimized compared to the legacy mapping.

Hence, the compact burst mapping is only used on the UL, and is always applied. In a frequency hopping scenario, the use of compact mapping is not expected to improve performance since time coherency is lost between TDMA frames (hopping on different frequencies), but on the other hand, the performance is not degraded compared to using current burst mapping.

Hence the principle is followed that current burst mapping for traffic channels and their associated control channels are followed on the DL while compact burst mapping is always applied on the UL. This applies regardless of FH configuration.

# Simulations

## Simulation assumptions

Simulations have been run for MCS-1 in both a non-frequency hopping and a frequency hopping scenario using the highest coverage class of 16 blind transmissions, see [7].

The simulation assumptions from [6] have been followed applying the agreed frequency offset model to EC-GSM according to [4], and the candidate specific initial drift from [8].

Some of the parameters are listed in Table 1.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| MCS | MCS-1 |
| Channel | TU |
| Direction | UL |
| Mobile speed | 1.2 km/h |
| Frequency hopping | iFH, noFH |
| Band | GSM 900 |
| Impairments | Typical RX and TX. |
| AFC | Range of possible frequency offsets detected [-100,100] Hz |
| Number of transmissions | 16 |

## Results

Figure 2. Performance of compact burst mapping compared to current burst mapping on a no FH channel and iFH channel.

As can be seen, the compact burst mapping is superior compared to the current burst mapping in a low diversity scenario while in a high diversity scenario there is no difference in performance. Hence, for simplicity the compact burst mapping is always applied on UL.

# Conclusions

Based on the results and reasoning provided in the contribution current burst mapping is maintained on the DL for CIoT for traffic channels and their associated control channels, while a compact burst mapping is used on the UL.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

3GPP TS 45.002, “Multiplexing and multiple access on the radio path”

GP-140885, “Frequency offset model for cellular IoT”, source Ericsson LM. GERAN#64.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

GPC150009, “Technical Report Update”, source Rapporteur. GERAN Ad Hoc#1 on FS_IoT_LC.

GPC150055, “EC-GSM, Mapping of logical channels onto physical channels”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

GPC150064, “EC-GSM, EC-SCH design, performance and mapping”, source Ericsson LM. GERAN Ad Hoc#1 FS_IoT_LC.