# EC-GSM, EC-RACH design, performance and mapping

# Update of GP-140886

# Introduction

In this contribution the EC-RACH channel performance is evaluated in extended coverage under initial assumptions on mobile autonomous and network triggered access load. It is shown that the capacity of a dedicated RACH channel on TS1 is sufficient to handle the load from Cellular IoT, given preliminary assumptions made in this paper. It is shown that the negative performance impact by using a conventional receiver is limited at the investigated arrival rates, and that performance gains are possible if using an overlaid CDMA technique.

In Section 2 simulation parameters, models as well as assumptions are described.

In Section 3, the results are presented.

# Simulation Settings

## Basic Simulation Properties

The basic simulation properties are summarized in Table 1. Notable is that the normal burst (NB) was used for the RACH, as described in the concept of Accelerated Access System Procedure (ASAP), see [1]. ASAP allows more efficient signaling which is important for Cellular IoT, and facilitated the investigation since the multiple TSCs used for the different coverage classes are already available for the NB. It is expected that similar conclusions would be drawn if access bursts (ABs) are instead used, as long as the coding rate is similar and the bursts are aligned similarly in time, but this is not investigated in this contribution.

Table 1: Basic Simulation Properties.

| Parameter | Setting |
| --- | --- |
| Propagation model | Typical Urban |
| MS speed | 1.2 km/h |
| Frequency hopping | No |
| Receiver mode | 2 Rx antennas, IRC |
| Repetition combining | I/Q accumulation with frequency offset compensation |
| Logical channel | RACH normal burst with 48 bit payload [1] |
| TSC | A unique TSC was configured per coverage class. |
| Number of unique bursts | 100000 (Given k repetitions, k x 100000 bursts are transmitted) |
| Interfering bursts | Mix of repeated GMSK normal bursts, see Section 2.3, unless otherwise stated. |
| Frequency error | Drawn from a normal distribution with mean 0Hz and standard deviation 2 Hz independent for carrier and all interferers. New realization after each completed repetition interval. The frequency is drifting with 22.5 Hz/s during transmissions and 9 Hz/s when not transmitting, cf. [2]. |

## Coverage Classes and Signal Levels

Six coverage classes are assumed using 1, 2, 4, 8, 16 and 32 repetitions (transmissions of the same burst). The average relative powers between bursts of the coverage classes are 0dB, -3dB, -6dB, -9dB, -12dB and -15dB for coverage class 1, 2, 3, 4, 5 and 6 respectively, which maps well to the expected 3 dB gain per doubling of repetitions. This will not take into consideration the fact that coverage class 1 devices generally are more than 3dB stronger than coverage class 2 devices. The model will however be valid for devices near the cell border. It can be noted that there is a coarse power control on the RACH available in GSM today (RACH power reduction), but this has also not been taken into account.

The accuracy of the estimation of a device’s coverage class is not covered in this contribution.

Each coverage class has one assigned training sequence code (TSC). All users in the simulation use the TSC associated with its class. If two users are of the same class they will use the same TSC, this is also true for the desired signal. It is assumed that the repetition intervals are fixed in time, i.e. class 1 can transmit every burst, class two can start transmitting every other burst, class three can start transmitting every 4th burst etc., see Figure 1.

Class 1

Class 2

Class 3

Class 4

Class 5

Slot available to start transmission

Slot for repetition

Consecutive TS1 slots

Figure 1: Illustration of coverage class repetition mapping

The first RACH burst in a full cycle of RACH bursts, that spans a full hyperframe, corresponds to TS1 of the TDMA frame for which FN = 0. It is assumed that all coverage classes transmit on the same timeslot. Collisions are possible with both interferers of the same class and with interferers of all other classes.

## Interference model

RACH collisions are modeled with repeated interfering normal bursts with an arrival intensity modeled by a Poisson process. When an interferer of coverage class X is born, that same burst is repeated 2(X-1) times with TSC according to Section 2.2. The distribution between classes is assumed to be 0.6, 0.2, 0.1, 0.05, 0.025, 0.025 for class 1, 2, 3, 4, 5, 6 respectively. This is an initial assumption that needs to be updated once a complete path loss model has been agreed.

The total arrival intensity for RACH, i.e. access attempts without repetitions, is assumed to be 4.49 events/s [3].

## Overlaid CDMA

The RACH performance can be improved by using for example an overlaid CDMA technique, [4]. The performance of overlaid CDMA have been simulated using the same framework as described in Section 2.1, Section 2.2 and Section 2.3, with the exception that colliding RACH burst always are in the same coverage class as the desired signal. Note that codes can be designed to maintain orthogonality also for the case of mixed coverage classes, but that has not been evaluated in this contribution.

# GSM Evolution RACH Performance

## Conventional RACH Receiver

The SNR degradation for a conventional receiver for different coverage classes due to RACH collisions when all coverage classes use TS1 are shown in Figure 2. As a reference it can be noted that at 10% BLER the SNR for 32 repetitions is below -12 dB when no collisions occur roughly corresponding to an MCL of 162 dB, i.e. a 18 dB coverage improvement compared to a reference MCL for GPRS of 144 dB. Figure 2 shows that the SNR degradation is low for the relevant interferer intensities assumed. The RACH collisions in Figure 2 and Figure 3 are modelled according to Section 2.3.

Figure 2: SNR degradation for different coverage classes

The BLER degradation for different coverage classes due to RACH collisions when all coverage classes use TS1 are shown in Figure 3. Figure 3 shows that the BLER degradation is low for the relevant interferer intensities assumed.

Figure 4 and Figure 5 show the SNR degradation and BLER degradation when excluding coverage class 1 collisions, for example if coverage class 1 use TS0. As seen in Figure 4 and Figure 5 the SNR degradation is in this case reduced by roughly 0.2dB and the BLER degradation is reduced by roughly 0.005 at intensity 4.49 events/s.

Figure 3: BLER degradation compared to no collisions at 10% BLER

Figure 4: SNR degradation for different coverage classes (excluding cc1 collisions)

Figure 5: BLER degradation compared to no collisions at 10% BLER (excluding cc1 collisions)

## Overlaid CDMA

Figure 6 and Figure 7 show the performance gains of using overlaid CDMA compared to the same conventional receiver used in Section 3.1. The RACH collisions in Figure 6 and Figure 7 are always of the same coverage class as the desired signal, hence direct comparison to Figure 2 and Figure 3 are not possible and the relevant intensity here is not necessarily 4.49 events/s. The improvements brought by OLCDMA are not surprisingly increasing with increasing coverage class.

Figure 6: SNR degradation improvement with OLCDMA

Figure 7: BLER degradation improvement with OLCDMA at 10% BLER

# Conclusions

In this contribution the performance of the RACH channel is investigated for GSM Evolution given preliminary assumptions for Cellular IoT.

Given these assumptions the results indicate that GSM Evolution can cater for the assumed range of access loads with a low SNR and BLER degradation, assuming a conventional RACH receiver. If using an overlaid CDMA technique performance improvements are possible.

# References

GPC150062, EC-GSM, Accelerated System Access Procedure, 3GPP TSG GERAN Ad Hoc#1 on FS_IoT_LC , source Ericsson

GP-140885, Frequency offset model for Cellular IoT, GERAN#64, source Ericsson

GP-140909, Traffic models for the Cellular IoT study, GERAN#64, source Ericsson

GP-140883, GSM Evolution for cellular IoT – Increasing UL capacity, GERAN#64, source Ericsson