EC-GSM - Realizing eDRX Using TDMA FN Information

# 1.	Introduction

A device that wants to support downlink reachability with a periodicity ranging from about 1 to 30 minutes will choose to operate using eDRX based reachability since  PSM based reachability will not allow for sufficient extension of battery lifetimes to be realized within this range (see Figure 1 below and GP-140910).

Figure 1: Battery lifetime versus reachability using the shorter sync.

A SGSN that has received an IP packet (e.g. containing a trigger) for delivery to a target device that makes use of eDRX will, based on legacy procedures, respond by immediately sending a corresponding paging request to a BSS. When this occurs there is the possibility that the next paging based instance of reachability for the target device may occur well beyond the point in time when the BSS receives the paging request. This can be problematic considering that (a) the BSS may be limited regarding how long it can buffer a paging request while waiting for the next paging opportunity for a target device operating in eDRX mode and (b) the SGSN may consider the paging request to have failed or the device to be unreachable if it does not receive a corresponding page response within a relatively short time frame.

As such, as a first step towards realizing a complete solution for eDRX, a basic working assumption can be adopted whereby a SGSN will have the ability to buffer downlink IP packets for extended time periods (e.g. up to 1 hour). This leaves open the challenge of how the SGSN can determine an appropriate time for taking further steps towards delivering a buffered IP packet to the corresponding target device.

Working Assumption 1: An SGSN will have the ability to buffer a downlink IP packet for an extended time period prior to delivering it to the target device.

# 2.	Synchronization and TDMA FN Information

In practice it can be expected that all cells managed by any given BSS will be synchronized from a TDMA FN perspective to a high degree such that the transmission of TDMA Frame Number sequences sent over the radio interface of these cells will at most differ by a few milliseconds. As part of Gb interface management procedures, knowledge of BSS specific TDMA FN information can be conveyed from the BSS to the SGSN thereby providing it with near real time knowledge (accurate within a few milliseconds) of ongoing TDMA frame transmissions within the overall TDMA FN space for all cells managed by any given BSS. This near real time knowledge of ongoing TDMA frame transmissions combined with knowing the IMSI, eDRX cycle and coverage class associated with a device allows a SGSN to calculate the next occurrence of the nominal paging group for that device (i.e. an IMSI based hashing algorithm that takes into account the eDRX cycle length and DL coverage class can be used to determine the nominal paging group of a device – see [2]).

Working Assumption 2: The frame structure in all cells served by a BSC belonging to any given vendor will be aligned on a ±5 or ±10 milliseconds level.

Working Assumption 3: Gb interface management procedures will be enhanced to support the transmission of BSS specific TDMA FN information from the BSS to the SGSN which is then maintained by the SGSN indefinitely.

Working Assumption 4: A SGSN uses the IMSI, eDRX cycle and DL coverage class of a device to determine the ongoing occurrences of the nominal paging group for that device within the context of each BSS managing cells in the corresponding paging area.

# 3.	Complete Solution for eDRX in GERAN

The 4 working assumptions listed above can be used as a basis for further developing a full solution for eDRX for EC-GSM as described below.

## 3.1	Selecting an eDRX Cycle

A cell that supports eDRX shall support a set of eDRX cycle lengths subject to the requirement that each member of the set shall occur an integral number of times within the full TDMA FN space (see [1]). This set is shown in Table 3.1-1 and shall be common to all GERAN cells that support EC-GSM since it is based on parsing the GERAN TDMA FN space in a RAT specific manner.

Table 3.1-1. Set of eDRX Cycle Lengths

| eDRX Cycle Value (EXTENDED_DRX) | Target eDRX Cycle Length | Number of 51-MF per eDRX Cycle (EXTENDED_DRX_MFRMS) | eDRX Cycles per TDMA FN Space |
| --- | --- | --- | --- |
| 0 | ~30 seconds | 128 | 416 |
| 1 | ~60 seconds | 256 | 208 |
| 2 | ~2 minutes | 512 | 104 |
| 3 | ~4 minutes | 1024 | 52 |
| 4 | ~6.5 minutes | 1664 | 32 |
| 5 | ~8 minutes | 2048 | 26 |
| 6 | ~13 minutes | 3328 | 16 |
| 7 | ~16 minutes | 4096 | 13 |
| 8 | ~26 minutes | 6656 | 8 |
| 9 | ~52 minutes | 13312 | 4 |
| Note 1: 53248 51-multiframes occur with the TDMA FN space (2715648 TDMA frames) Note 2: All remaining EXTENDED_DRX values are reserved |  |  |  |

If the maximum amount of time that any given SGSN implementation supports for buffering a downlink IP Packet before sending it to a BSS is X minutes then devices in the RAI managed by that SGSN should not be allowed to use an eDRX cycle length that exceeds X.

- The RAU Request shall be modified to include an indication of the eDRX cycle length selected by a device and its estimated DL coverage class. The SGSN decides which eDRX cycle length to use based on the proposed eDRX cycle length from the device and the maximum eDRX cycle length allowed. The RAU Accept shall be modified to include an indication of the actual eDRX cycle the device is to use.

- After a device registers (performs a RAU) the serving SGSN knows (a) its paging area, (b) its eDRX cycle length (EXTENDED_DRX value of Table 3.1-1 included in the RAU Accept) and (c) its estimated DL coverage class (also included in the RAU Request) and therefore knows how to determine the ongoing occurrences of the device’s nominal paging group as they occur within the context of each BSS managing cells in the corresponding paging area.

Working Assumption 5: Each member of the set of eDRX cycle lengths supported in a cell occurs an integral number of times within the TDMA FN space where the set of possible eDRX cycle lengths shall be common to all GERAN cells that support eDRX.

Working Assumption 6: The RAU Request shall be modified to include an indication of the eDRX cycle length selected by a device and its estimated downlink coverage class.

Working Assumption 7: The RAU Accept shall be modified to include an indication of the actual eDRX cycle length allowed by the SGSN for the device.

## 3.2	Managing Downlink IP Packets

When a SGSN first determines it has downlink payload to deliver for a given device and the corresponding Ready timer is not running, it calculates the next occurrence of the device’s nominal paging group by using the TDMA FN information it has for each BSS managing one or more cells in the corresponding paging area, combined with the knowledge of the IMSI, eDRX cycle and DL coverage class associated with the device.

- The SGSN sends a paging request to each BSS in the paging area shortly before the next occurrence of the device’s nominal paging group in the cells managed by that BSS.

- Each paging request contains information that allows a BSS to determine the next occurrence of the device’s nominal paging group (i.e. eDRX cycle length, downlink coverage class and IMSI).

- Upon receiving a paging response from a BSS the SGSN forwards the available IP packet to that BSS for delivery to the target device

Working Assumption 8: Paging requests sent from the SGSN to a BSS shall include an indication of the eDRX cycle length and the estimated downlink coverage class last indicated by the target device (e.g. in a RAU Request).

Working Assumption 9: Upon receiving a downlink IP Packet the SGSN assumes it can buffer it for an extended time period (e.g. up to 1 hour) with no concern for potential impacts of this buffering on the application server from which it received the IP packet.

# 4.	Proposal

It is proposed that the working assumptions listed above be used as the basis for supporting eDRX within EC-GSM.

# 5.	References

GPC150081, “EC-GSM - eDRX and TDMA FN Space Wrap Around”, source Ericsson, CIoT AdHoc #1

GP-140605, “GSM Evolution for cellular IoT – PCH Overview’, source Ericsson, TSG GERAN #63