C-UNB technology for Cellular IoT

-

Link Layer aspects

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. Various radio access technology proposals are candidate for this WI. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB), for which key features are:

ultra-narrow band transmission,

random access,

cooperative reception.

Overall description of the C-UNB radio access technology can be found in [2]. The present document details the link layers aspects of the C-UNB radio access technology. It is an updated extract of document [3], with more inputs on LLC.

# Downstream link layer aspects

## Format of downlink MAC-PDUs

In C-UNB, downlink MAC-PDUs are optimized considering the fact that they are transmitted in reponse to up link transmission. Figure 1 illustrates the MAC-PDU format in C-UNB downlink. The various fields are detailed in the following subclauses.

Figure 1: Format of a MAC-PDU frame in C-UNB downlink transmission

### Header

Downlink header is 56 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a payload length,

acknowledgement bits.

### Payload

The payload of a MAC-PDU is 0-34 byte length. It carries application data and, when necessary, LLC header for segmentation/re-assembly.

### Authentication

The authentication field is a hash, computed with an adaptation of the common C-MAC algorithm. It uses the 128-bit secrete key of the device plus:

the device identifier received in corresponding uplink packet,

the sequence number in the corresponding uplink packet,

the payload field.

This field is used to sign a downlink packet and also to identify the destination device implicitly: the probability to have many devices listening on a given frequency at a given time is very limited, hence a 16-bit hash is enough to authenticate the destination.

### FCS

The frame check sequence in C-UNB downlink consists in a CRC-8. It checks the MAC-PDU after error correction, before further decoding.

### ECC

This field is a 32-bit Error-Correction Code. It uses BCH code applied to the four first fields: header (except preamble), Payload, Authentication and FCS. Given the length of DL MAC-PDU, the ECC is expected to add 1 dB of processing gain in the DL MCL.

## Segmentation & reassembly

Document [3] defines the maximum application packet to 2 000 bytes, with addition of a protocol header of 65 bytes (extended) or 29 bytes (compressed). Given the small size of C-UNB radio packets, it is proposed to have a segmentation/re-assembly mechanism with 32-bytes of application data and 2 bytes for segment numbering (see figure 2).

Figure 2: segmentation in C-UNB downlink

This implementation gives a maximum size of the application data of 4 096 bytes, which is enough for the data packets expected in CIoT.

## DL Mean Throughput

At the SNDCP interface, the transmission of long application blocks requires the maximum throughput. The average throughput of C-UNB radio access technology at SNDCP interface is given by the formula (1):

Given the data rate of 600bps and the sizes of MAC and LLC headers, the DL throughput is about 400bps, which is above the 160bps required by [3].

# Upstream link layer aspects

## Uplink MAC-PDU

Figure 3 illustrates the MAC-PDU format in C-UNB uplink. The various fields are detailed in the following subclauses.

Figure 3: Format of a MAC-PDU frame in C-UNB uplink transmission

### Header

Uplink header is 32 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a frame length,

acknowledgement flags,

a repetition counter (value 1, 2 or 3) tagging the actual repetition number of MAC-PDUs.

### Sequence counter

The uplink sequence counter is 12 bits. It is incremented for each new LLC-PDU sent by a device. The sequence counter is used for authentication purpose (see subclause 3.1.5).

### Identifier

This field is 40 bits long. It contains the device unique identifier set during personalization of the device. Each identifier is unique in a C-UNB system.

### Payload

The payload is 0-32 bytes. Payload length is defined by a 5-bit length field in the MAC-PDU header. Empty payload corresponds to ack packet; they have a specific frame type.

### Authentication

The authentication field is a 16 bits hash, used to authenticate the MAC-PDUs received by the networks. The 16 bits of the authentication field are generated using a proprietary implementation of the well-know C-MAC algorithm. It uses a 128 secrete key. Authentication check is performed in the core network and not in the base stations to avoid secrete key transmission on the back-haul network.

### FCS

The frame check sequence in C-UNB uplink consists in a CRC-8. In reception, it is used to check MAC-PDU consistency before transmission to the core network.

### ECC

The last field of an uplink MAC-PDU is an Error-Correcting Code of 16 bits. It used Reed-Solomon algorithm and is evaluated over the whole MAC-PDU except the preambule.

## Segmentation & reassembly

The CIoT requirements define Mobile Autonomous Reports with 20-200 byte length. Given the fact that only small radio packets are affordable for the C-UNB radio access technology, it is proposed to have a segmentation/re-assembly mechanism based on 31 blocks of data with a LLC header of 8 bits: 4 bits for segment numbering and 4 bits for total number of segments (see figure 4). These values set the maximum block size of application data to 496 bytes.

Figure 4: LLC header in C-UNB uplink

## UL mean throughput

Given the data rate of 250bps in UL and the sizes of the MAC and LLC headers, the formula (1) gives a mean throughput of 160bps in uplink.

# Conclusion

This contribution gives details on the LLC and MAC layer frame formats in C-UNB radio access technology and shows how they fulfill the requirements set by WI FS_IoT_LC [1].

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT", Sigfox, Ad'hoc meeting, 02-05 Feb'15, Sophia Antipolis

[3]	TR 45.820 v0.3.1 (2015-02), "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support Ultra Low Complexity and Low Throughput Internet of Things; (Release 13)"