# EC-GSM – Exception Report Latency Performance Evaluation

# (Update of GP-150269)

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things  [1] (WI code: FS_IoT_LC)  was approved.

One of the main objectives of this study is to increase the coverage compared to existing GPRS services.

In this contribution, calculations of the exception report latency according to the agreed framework [2] at normal, +10 dB and +20 dB extended coverage are presented.

This is an update of GP-150269 presented at GERAN#65 to include the agreements for how to derive the coverage extension on packet data traffic channels when HARQ retransmissions are used in combination with blind repetitions [4]. Updates are marked in red.

# Calculations

## Latency calculation assumptions

Calculations have been performed according to the agreed framework for latency analysis where:

- The latency is measured from when a device first decides to send a report (UL application layer payload) and includes the time to synchronize to the network, the time to perform an access attempt and the time for a BSS to successfully detect reception of the UL application layer payload at the base i.e. this last point in time occurs at the point where the BSS determines that all UL RLC data blocks have been received.

- The worst case uplink payload being  (without header compression)

  - 95 bytes = 20 bytes (application) +  65 bytes (Header above SNDCP) + 4 bytes (SNDCP) + 6 bytes (LLC)

- The latency should be reported at three different coverage conditions: GPRS reference MCL+0dB, GPRS reference MCL+10 dB, and GPRS reference MCL+20dB.

In addition to the payload 4 bytes of TLLI per RLC data block also needs to be included for the contention resolution process. Assuming that data will be transferred using MCS-1 (22 bytes payload) the total number of RLC data blocks to transfer is 6.

Latency is evaluated according to the formula agreed at Cellular IoT telco#8:

Latency for DATA transmission = (1) T Synchronization+ (2) T Transmission + (3) T Receiving + (4)T Wait

- T Synchronization: The time for UE to synchronize to the network. T Transmission: The transmitting time for any signaling and data.

- T Receiving: The receiving time for any signaling and data. It should be highlighted that the valid scheduling information receiving such as USF belonging to the UE which is not included in an obvious signaling should be taken into account.

- T Wait: The time between any transmission and receiving, and also the time between two consecutive transmission or receiving. The waiting time is relevant with the scheduling mechanism for a specific solution.

## Latency evaluations

### General

In the context of this paper the network synchronization time is assumed to be the time it takes for a device to synchronize to the same cell (i.e. confirm suitability) as it was previously camped on.  It is more over assumed to include reading both the FCCH and EC-SCH channel.

Furthermore, as discussed in [2], EC-GSM will use the concept of fixed uplink allocation (FUA) of radio blocks on UL PDTCH resources in the interest of avoiding USF based uplink transmissions for devices operating in extended coverage, and to reduce RX monitoring time for the sake of battery savings.

### GPRS reference case

For an EC-GSM device in normal GPRS reference coverage the evaluation of average network synchronization is fairly straight forward as the FCCH and SCH channels are mapped in pairs occurring every 10th TDMA frame on TS0.  This implies that the average acquisition time of the legacy FCCH and SCH channels are 5.5 TDMA frames or approximately 26 ms (i.e. 5.5 TDMA frames). In addition, as an EC-GSM device also needs to read the EC-SCH before system access an additional average acquisition time of 20 TDMA  frames or 92 ms needs to be added. This results in a total average synchronization time of 118 ms.

The sequence of signaling events for a device at GPRS reference coverage using FUA is shown in Figure 1, where, in order not to underestimate the total latency, it has been assumed that one re-transmission of an RLC data block is needed.

### Figure 1, Illustration of signaling events for transmission of data using fixed allocation.

Furthermore, it is assumed that the time an MS needs to wait for before sending a channel Request tMS-channel req. is equal to 1 TDMA frame and that the average response time in the BSS before transmitting an Immediate Assignment message tBSS-Imm. Assign. is 80 ms. The reaction times in the BSS and MS during packet transfer, tMS and tBSS, are assumed to be equal to one BTTI i.e. 20 ms.

Using the above assumptions the total latency for transmitting data can be evaluated. The latency is calculated in Table 1.

Table 1 Latency calculation for GPRS reference case with fixed allocation

|  |  | Number of occurrences | Time per occurrence (ms) | Total time (ms) |
| --- | --- | --- | --- | --- |
| 1 | Network synchronization | 1 | 118 | 118 |
| 2 | tMS-Channel Req. | 1 | 4.651 | 4.651 |
| 3 | Channel request | 1 | 0.577 | 0.577 |
| 4 | tBSS-Imm. Assign. | 1 | 80 | 80 |
| 5 | Immediate Assignment | 1 | 20 | 20 |
| 6 | tMS | 1 | 20 | 20 |
| 7 | Transmission of data blocks | 6 | 20 | 120 |
| 8 | tBSS | 1 | 20 | 20 |
| 9 | PUAN | 1 | 20 | 20 |
| 10 | tMS | 1 | 20 | 20 |
| 11 | Re-transmission of 1 data block | 1 | 20 | 20 |
|  | Total time |  |  | 443 ms |

### GPRS reference case + 10 dB

For a device at 10 dB extended coverage average network synchronization time is ca 365 ms[3].

The sequence of signaling events at 10 dB extended coverage assuming a total of up to 4 HARQ transmissions (1 initial transmission + up to 3 retransmissions) for successful transfer of an exception report, where each uplink RLC Data block is transmitted with 2 blind repetitions, is illustrated in Figure 2. Simulations have shown [4] that in + 10 dB extended coverage case, 99% of the exception reports are transferred to the BSS within less than 0.6 seconds, referred to as t4 HARQ  in Figure 2. The average resource utilization factor is 1.4, expressed as number of radio blocks transmitted on average for each planned radio block.

Figure 2, Illustration of the sequence of signaling events for transmission of data with a total of 4 HARQ transmissions (Note:  This figure is purely an illustration where e.g. retransmissions of failed PACCH control signaling is not included).

The time an MS needs to wait for sending a channel Request, tMS-channel req.,is equal to 1 TDMA frames[2] (from the last EC-SCH frame to first possible EC-RACH occurrence for a coverage class 3 device). Moreover, in order not to underestimate the response time in the BSS before transmitting an Immediate Assignment message, tBSS-Imm. Assign. , this delay is assumed to be 80 ms (as in the reference case) plus the delay to send one EC-AGCH message to a coverage class 3 device (8 TDMA frames), this results in total BSS response time, tBSS-Imm. Assign. , of around 117 ms. Furthermore, the wait times in the MS during packet transfer, tMS is assumed to be mainly due to scheduling delay and are assumed to be equal to one BTTI of 20 ms.

The time it takes to complete the transmission of a radio block on a specific logical channel is defined in section 6.2.4.2 “Mapping of logical channels onto physical channels” in [2].  For a coverage class 3 device the EC-RACH channel is mapped over 4 TDMA frames. Similarly, for a coverage class 3 device the EC-AGCH is mapped over 8 TDMA frames.

Using the above assumptions the total latency for transmitting data at 10 dB extended coverage can be evaluated – the latency is calculated in Table 2

Table 2 Latency calculation for GPRS +10 dB extended coverage

|  |  | Number of occurrences | Time (ms) | Total time (ms) |
| --- | --- | --- | --- | --- |
| 1 | Network synchronization | 1 | 365 | 365 |
| 2 | Wait channel Request (1 TDMA frame) | 1 | 4.6 | 4.6 |
| 3 | Channel request (4 TDMA frames) | 1 | 18.4 | 18.4 |
| 4 | tBSS-Imm. Assign. 80 ms + 8 TDMA frames | 1 | 117 | 117 |
| 5 | Immediate Assignment (8 TDMA frames) | 1 | 36.9 | 36.9 |
| 6 | tMS | 1 | 20 | 20 |
| 7 | t4 HARQ | 1 | 600 | 600 |
|  | Total time |  |  | 1162 |

### GPRS reference MCL + 20 dB

For a device at 20 dB extended coverage the time average time for network synchronization is 690 ms [3].

The sequence of signaling events at 20 dB extended coverage assuming a total of up to 4 HARQ transmissions (1 initial transmission + up to 3 retransmissions) for successful transfer of an exception report, where each uplink RLC Data block is transmitted with 16 blind repetitions, is illustrated in Figure 3.  Simulations have shown [4] that in + 20 dB extended coverage case, 99% of the exception reports are transferred to the BSS within less than 2.4 seconds, referred to as t4 HARQ  in Figure 3. The average resource utilization factor is 1.7, expressed as number of radio blocks transmitted on average for each planned radio block.

Figure 3, Illustration of the sequence of signaling events for transmission of data with a total of 4 HARQ transmissions (Note:  This figure is purely an illustration where e.g. retransmissions of failed PACCH control signaling is not included)).

Furthermore, the time an MS needs to wait for sending a channel Request,  tMS-channel req.,is equal to 44 TDMA frames, see [2], from the last EC-SCH frame to first possible EC-RACH occurrence for a coverage class 6 device. Moreover, the scheduling delay in the BSS before transmitting an Immediate Assignment message tBSS-Imm. Assign. is on average 105 TDMA frames (((35+2*51+19)+(35+19))/2=105) or  484 ms. Furthermore, the wait times in the MS during packet transfer, tMS is assumed to be mainly due to scheduling delay and are assumed to be equal to one data transmission TTI or 80 ms.

The time it takes to complete the transmission of a radio block on a specific logical channel is defined in section 6.2.4.2 “Mapping of logical channels onto physical channels” in  [2]. For a coverage class 6 device the EC-RACH channel is mapped over 2 multi frames implying an average transmission time of 51+16 or 67 TDMA frames. Similarly, for a coverage class 6 device the EC-AGCH is mapped over 4 multi frames implying a total of 940 ms for the 32 repetitions of the EC-AGCH block.

Using the above assumptions the total latency for transmitting data at 20 dB extended coverage can be evaluated – the latency is calculated in Table 3

Table 3 Latency calculation for GPRS +20 dB extended coverage

|  |  | Number of occurrences | Time (ms) | Total time (ms) |
| --- | --- | --- | --- | --- |
| 1 | Network synchronization | 1 | 690 | 690 |
| 2 | Wait channel Request (44 TDMA frames) | 1 | 203 | 203 |
| 3 | Channel request (67 TDMA frames) | 1 | 309 | 309 |
| 4 | tBSS-Imm. Assign. | 1 | 484 | 484 |
| 5 | Immediate Assignment | 1 | 940 | 940 |
| 6 | tMS | 1 | 80 | 80 |
| 7 | t4 HARQ | 1 | 2400 | 2400 |
|  | Total time |  |  | 5106 |

# Conclusions

Based on the results provided in this paper it can be seen that the EC-GSM concept can meet the 10 second latency target of delivering an exception report with quite some margin. The calculated latency at different coverage conditions is summarized in Table 4.

Table 4 Exception report latency versus coverage condition

| Coverage condition | Exception report latency (sec) |
| --- | --- |
| GPRS reference MCL | 0.45 |
| GPRS reference MCL + 10 dB | 1.2 |
| GPRS reference MCL + 20 dB | 5.1 |

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TR 45.820 V1.01  (2015-03), Source Vodafone, GERAN Ad Hoc#1 on FS_IoT_LC

GP-150138, “EC-GSM, Network Synchronization”, Source Ericsson LM, GERAN#65

GPC150206, EC-GSM, HARQ modeling for coverage, throughput and delay analysis, source Ericsson LM, Cellular IoT adhoc#2