Source: 	Semtech Neuchatel SA

Title:    	Combined narrowband and CSS performance

Agenda:	1.4.2.6

Document for:	Discussion

# Introduction

At GERAN#62, a new SI [1] was approved to study cellular support for ultra-low complexity and low throughput IoT.  During GERAN#65 a combined chirp spread spectrum and narrow band modulation was presented. This document updates this presentation with performance simulation results over the channel model used by the group. The table below recalls the proposed channel bandwidths for narrowband GMSK modulations on the uplink. On the downlink, only 3200,6400, and 12800 Hz bandwidths are used.

| nb channels | Channel BW (Hz) |  | Symbol rate |
| --- | --- | --- | --- |
| 32 | 400 |  | 244.1 |
| 16 | 800 |  | 488.3 |
| 8 | 1600 |  | 976.6 |
| 4 | 3200 |  | 1953.1 |
| 4 | 6400 |  | 3906.3 |
| 8 | 12800 |  | 7812.5 |
| Total uplink spectrum (Hz) | 179200 |  |  |

# Downlink narrowband channels performance

The downlink performance of GMSK is evaluated in 1Tx/1Rx scenario with CSS beacon interference. For uplink, 2 receive chains with MRC are assumed. Channel model is TU 1Hz, pilot tones are inserted to track the channel phase and amplitude variations. The simulation for the lowest uplink data rate has been modified to fit the 140bps target, using a coding rate of 2/3 .

Figure 7: DL narrowband performance

Figure 8: UL narrowband performance

# Link budget analysis

| Transmitter | GMSK Uplink 160b/sec @ 20dBm | GMSK downlink 976b/sec | Beacon downlink SF9 |
| --- | --- | --- | --- |
| (0) Total Tx power (dBm) | 20.0 | 45.0 | 45.0 |
| (1) Tx power per channel (dBm) | 20.0 | 34.0 | 37.0 |
| Receiver |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174.0 | -174.0 | -174.0 |
| (3) Receiver noise figure (dB) | 5.0 | 7.0 | 7.0 |
| (4) Interference margin (dB) | 0.0 | 0.0 | 0.0 |
| (5) Occupied channel bandwidth (Hz) | 200.0 | 1600.0 | 125000.0 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log((5)) (dBm) | -146.0 | -135.0 | -116.0 |
| (7) Required SINR (dB) | 3.0 | 7.0 | -10 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -143.0 | -128.0 | -126 |
| Maximum coupling loss (MCL) = (1) – (8) (dB) | 163.0 | 162.0 | 163.0 |

The Chirp Spread Spectrum performance is shown below. 100bytes frames are simulated, over TU 1Hz. Coding scheme is convolutional K=7 with rate ½. This mode is intended for localization, mainly outdoor.

| Transmitter | CSS uplink 1Tx/2Rx 180bit/sec | CSS uplink 1Tx/2Rx 3.5kbit | CSS downlink 1Tx/1Rx 180bit/sec | CSS downlink 1Tx/1Rx 3.5kbit |
| --- | --- | --- | --- | --- |
| (0) Total Tx power (dBm) | 20.0 | 20.0 | 28.0 | 28.0 |
| (1) Tx power per channel (dBm) | 20.0 | 20.0 | 22.0 | 22.0 |
| Receiver |  |  |  |  |
| (2) Thermal noise density (dBm/Hz) | -174.0 | -174.0 | -174.0 | -174.0 |
| (3) Receiver noise figure (dB) | 5.0 | 5.0 | 7.0 | 7.0 |
| (4) Interference margin (dB) | 0.0 | 0.0 | 0.0 | 0.0 |
| (5) Occupied channel bandwidth (Hz) | 125000.0 | 125000.0 | 125000.0 | 125000.0 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log((5)) (dBm) | -118.0 | -118.0 | -116.0 | -116.0 |
| (7) Required SINR (dB) | -21.0 | -6.5 | -18.0 | -3.0 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -139.0 | -124.5 | -134.0 | -119.0 |
| Maximum coupling loss (MCL) = (1) – (8) (dB) | 159.0 | 144.5 | 156.0 | 141.0 |

# Summary

In this paper, we refine the combined CSS and narrow band proposal. We show that the proposed system allows a 20dB MCL improvement over GSM.

# References

- GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62