3GPP TSG GERAN#65 AdHoc#2 on FC_IoT_LC	Tdoc GPC150230

Sophia Antipolis, France	Agenda item 1.4.2.2, 2.4.2.2

20th – 23rd April, 2015

Source: Nokia Networks

N-GSM: Design and performance for N-SCH

# Introduction

This contribution depicts the N-PDTCH design and its performance for the N-GSM concept proposal earlier introduced in [1], which is proposed to be included into TR 45.820 in [2].

Section 2 describes various design aspects such as burst format, mapping on the 51-multiframe and encoding. Performance is investigated in section 3 in terms of BLER performance and network synchronisation time and further proceeding is outlined in the conclusion in section 4. This document is update of the same discussion paper [6] presented at GERAN#65. The changes are highlighted in blue color.

# DESIGN FOR N-SCH

This section describes the N-SCH design including burst format and mapping on to the 51-multiframe structure. N-SCH is designed to allow fast synchronization at MCL=164 dB with chase combining and possibility to decode synchronization information nearly as quickly as with the existing SCH.

## N-SCH burst format

Coverage for synchronization channel is expected to be improved by 20 dB with narrow band symbol sequences versus legacy SCH. With the new N-CCCH structure as depicted in Figure 1 in section 2.2 it is possible to carry 10 synchronization bursts with 20 bits payload each for synchronization over N-SCH in one 51 multiframe period. The burst structure of the legacy synchronization burst and the proposed hybrid modulation burst for synchronization are given in Table 1.

| Burst Content | Synchronization burst for legacy SCH | Synchronization burst for N-SCH |
| --- | --- | --- |
| Front tail symbols | 3 | 3 |
| Payload symbols (first part) | 39 | 40 (5 NB sequences of 8 symbols) |
| Training Sequence | 64 | 62 |
| Payload symbols (second part) | 39 | 40 (5 NB sequences of 8 symbols) |
| Tail symbols | 3 | 3 |
| Guard period | 8.25 | 8.25 |

Table 1:  Burst format for N-SCH versus legacy SCH.

The TSC for the N-SCH burst is derived from the 64 bit legacy SCH TSC cutting the outer two symbols and bit order reversed.

## Mapping of N-SCH bursts onto 51-multiframe structure

CCCH coverage including synchronization channel can be improved with an IoT dedicated N-CCCH time slot, applying the precoding technique. That new slot appears in TN 1 right after current basic CCCH timeslot in TN 0. The new 51 multi frame structure is depicted in Figure 1.

In regard to FCCH, the legacy FCCH in TN 0 is expected to be sufficient, as also stated in [2].

Figure 1: Mapping of N-SCH bursts onto 51-multiframe structure in TN 1 of BCCH carrier.

## Multiplexing of N-SCH bursts onto 51-multiframe structure

The N-SCH information block is transmitted in four N-SCH bursts which are multiplexed into two 51-multiframes with 5 interleaved repetitions. The N-SCH bursts are located in the same TDMA frame in TN 1 following the FCCH and SCH in TN 0 which is expected to make synchronization faster with less power consumption. Signalled frame number information refers to the first frame within pair of 51 multi frames. It enables both chase combining and compression of timing information from 25 to 21 bits, whilst the CIoT capable device shall perform decompression based on the 51-multiframe structure. For receiving a complete N-SCH block, bursts 1 to 4 need to be correctly received. The mapping of the N-SCH bursts onto two successive 51-multiframes is shown in Figure 2.

Figure 2: Multiplexing 5 N-SCH blocks onto two successive 51-multiframes.

The alternated interleaving scheme used to identify odd /even frame is depicted in Table 2.

| N-SCH Burst number | Frames in even MF51 | Frames in odd MF51 |
| --- | --- | --- |
| 0 | 0, 20, 40 | 10, 30 |
| 1 | 1, 21, 41 | 11, 31 |
| 2 | 10, 30 | 0, 20, 40 |
| 3 | 11, 31 | 1, 21, 41 |

Table 2: Multiframe 51 alternated interleaving for N-SCH.

## Derivation of Timing information

Timing information is derived from signalled T1 and T2’ referring to the first TDMA frame of 51-multiframe of pair and the Cellular IoT capable devices then calculates frame numbers for other frames accordingly. Cellular IoT device will be able to determine FN as:

FN  = (51 x 26 x T1) + (2 x 51 x T2’ + 51 x T2’’) + T3’’

where

T2’’ is derived from SCH’ multiplexing and defined as

T2’’= 0, for even 51-multiframes,

T2’’= 1 for odd 51-multiframes.

T3’’ is derived from 51-multiframe borders during the FCCH acquisition on TS0.

## Information bits in N-SCH

The N-SCH payload is reduced from the legacy SCH payload. The legacy SCH payload carries 78 encrypted bits generated from a payload of 25 bits (6 bits for BSIC, 11 bits for T1, 5 bits for T2 and 3 bits for T3’). For N-SCH, the payload is reduced by omission of T3’ parameter (3 bits) that can be derived from 51-multiframe structure. The signalled reduced frame number information then refers to the start of the 51-multiframe pair, see Figure 2. The reduced payload of 21 bits for the N-SCH is depicted in Table 3.

| Content | Definition | Range | Number of bits |
| --- | --- | --- | --- |
| BSIC | Base transceiver station identity code | 0 to 63 | 6 |
| T1 | FN div (26 x 51) | 0 to 2047 | 11 |
| T2’ | (FN –T1*26*51) div 102 | 0 to 12 | 4 |

#### Table 3: Payload for N-SCH.

## Multiplexing and channel coding

Channel coding parameters for legacy SCH and N-SCH are compared in Table 4 below.

| Coding parameter | SCH | N-SCH |
| --- | --- | --- |
| Encoded bits | 78 | 80 |
| Payload bits | 25 | 21 |
| Parity bits | 10 | 10 |
| Tail bits | 4 | 6 |
| Spare bits | - | 3 |
| Code Rate | 1/2 | 1/2 |
| Bursts per Block | 1 | 4 |
| Constraint Length | 5 | 7 |

Table 4: Channel Coding for SCH and N-SCH.

## Cell Synchronization

If Cellular IoT capable device after reception of FCCH on TN 0 is not able to decode followed SCH at the same slot TN 0 due to low signal level, it tries to decode the more robust N-SCH sent in TN 1. The trigger to start decoding the N-SCH in TS1 and the feasibility of concurrent decoding of SCH and N-SCH bursts for faster synchronisation in better coverage conditions are FFS.

Timing information is derived from signalled T1 and T2’ referring to the first TDMA frame of the 51-multiframe pair and the Cellular IoT capable device then calculates frame numbers for other TDMA frames accordingly.

Acquisition of the N-SCH information block for CIoT device ,after synchronization  to 51-multiframe structure by synchronization to FCCH, is based on decoding attempts of N-SCH as follows.

- CIoT device first attempts to decode SCH from first 4 N-SCH bursts received from the 51 multiframe boundary. Decoding is tried with two interleaving patterns possible within the N-SCH bursts. Based on the interleaving pattern for which the decoding is successful, if the frame is odd/even is also identified. If the decoding is not successful, MS will chase combine next 4 SCH bursts and continue the decoding attempt as mentioned above.

- The device in MCL=164 dB coverage condition will attempt to chase combine 5 N-SCH blocks from 2*51 multiframes. If the decoding attempt fails ,new decoding is attempted with 10 N-SCH bursts from next 51 multiframe and oldest 10 N-SCH bursts are discarded.

- When the device attempts to chase combine the SCH blocks across 2*51 multiframes which are not part of the same pair, then the decoding will fail as the contents of SCH blocks is different across these 51 multiframes. In this case the decoding will be successful, when MS attempts decoding using next 10 SCH bursts from next 51 multiframe. In such case, the decoding will be successful after 3*51 multiframes.

As per the above procedure for cell synchronisation, it is possible to decode N-SCH within 11 TDMA frames from the detection of 51 multiframe boundary (best case). For eg if the MS starts network synchronisation at 40th TDMA frame of earlier 51 multi-frame, then N-SCH decoding will be successful by 11th Frame of next 51 multi-frame. In this case synchronisation time is 101.5 msec (22 TDMA Frames).

# PERFORMANCE Evaluation

## Simulation Assumptions

Simulation assumptions for evaluating N-SCH performance are depicted in Table 5 below.

| Parameter | Value |
| --- | --- |
| Logical Channel | N-SCH |
| Channel profile | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Number of repetitions | 5 |
| Starting Frequency offset at SCH Detection | 40 Hz |
| Number of SCH Blocks | 40000 |

Table 5: Simulation Assumptions for N-SCH performa8nce evaluation.

Performance for N-SCH was assessed for the sensitivity scenario and is reported in the following.

## Sensitivity

Figure 3 depicts the sensitivity performance for N-SCH. N-SCH is observed to achieve 10% BLER at 164 dB MCL for TU1.2nFH, corresponding to Es/No=-6.3 dB. Hence it is expected that N-SCH allows for fast enough synchronization for coverage constrained CIoT devices.

Figure 3: N-SCH Coverage Performance for TU1.2nFH, TU50nFH, TUiFH, RA250 and HT100 (BLER vs Es/No).

It is also observed that N-SCH coverage performance is significantly better in other legacy GSM radio propagation conditions (TU50, RA250 and HT100).

## Synchronisation Time

As per Study Item objective network synchronization is defined as the equivalent of acquisition of FCCH+SCH for GSM. Network synchronization is evaluated with one (equivalent of) BCCH carrier for initial cell search and cell re-confirmation. The timing of the broadcast carrier is assumed to be unknown, and uniformly distributed for initial cell search.

The network synchronisation time for N-SCH is evaluated for various coupling loss scenarios in this section. Frequency and Time accuracy after network synchronisation will be updated in the next version of this contribution.

SCH decoding is considered unsuccessful, if SCH cannot be decoded within MFmax multiframes. The number MFmax for the N-GSM system is FFS.

### Simulation Assumptions

Simulation environment as mentioned in table 1 is assumed.SCH decoding time with frequency offset of 40 Hz at the time of SCH decoding is evaluated.

The time taken for MS which starts the FCCH reception with initial frequency offset of +/- 18 kHz to correct its frequency to the accuracy of 40 Hz depends on the Frequency correction algorithm used by MS. This value is not evaluated in the simulation.

This additional delay for frequency correction to achieve 40 Hz frequency offset needs to be added to the synchronisation time estimated in the below simulation to obtain the total network synchronisation time including FCCH acquisition.

### Simulation Method

Following are the steps followed in estimating the network synchronisation time using this simulation.

- The TDMA frame number at which MS starts to detect frame boundary is randomly chosen between [0…51]. Depending on the starting TDMA frame number, time taken to detect the frame boundary for SCH Block decoding is noted as T1.

- If the N-SCH decoding is successful based on single SCH block, the synchronisation time needed for this SCH block is calculated as T1 +11 TDMA frames.

- If N-SCH decoding required more than one N-SCH block, the synchronisation time needed is calculated as T1+11 + (M-1)*20 TDMA frames where M is number of SCH  blocks required for successful decoding.

- If the N-SCH decoding requires chase combining of more than one 51 multiframe contents, the synchronisation time is increased by one additional 51 multiframe if the starting 51 multiframe is odd frame. The starting 51 multiframe in simulation is randomly selected between odd/even for each block.

CDF of Synchronisation time calculated for all the N-SCH bursts is plotted to identify the distribution of synchronisation time.

### Simulation Results

Below Figures indicate the CDF function for synchronisation time in terms of number of TDMA frames for MCL=164 dB and MCL=154 dB.

Figure 4: N-SCH Network Synchronisation Time (in TDMA frames) CDF for MCL=164 dB.

Figure 5: N-SCH Network Synchronisation Time CDF (in TDMA frames) for MCL =154 dB.

It is observed that at target MCL conditions the network synchronisation time corresponding to 95% percentile is 807 ms (Figure 4). For MCL=154 dB condition network synchronisation time reduces to 350 ms (Figure 5).

Figure 6 provides the average synchronisation time for various MCL values in the range from MCL=154 dB and MCL=164 dB.

Figure 6: N-SCH Average Network Synchronisation time (in TDMA frames) for  MCL range [154…164 dB].

In the above simulation for MCL=164 dB coverage condition: in 96% of the cases N-SCH detection is possible within 2*51 multiframes. For the remaining 4% blocks the successful decoding will be possible within one or more additional 51 multiframes. The probability of missing N-SCH decoding after 4*51 multiframes is expected to be negligible.

# Conclusion

In this contribution the N-SCH design is presented and the performance is assessed for sensitivity and network synchronisation time. It is observed that the target MCL of 164 dB can be met at BLER of 10%.

Average network synchronisation time for MCL=164 dB condition is evaluated as 313 ms without considering the additional delay required to correct the frequency error based on FCCH bursts. Network synchronisation time corresponding to 95 percentile for MCL=164 dB is evaluated as 807 ms.

Average network synchronisation time for MCL=154 dB condition is evaluated as 226 ms without considering the additional delay required to correct the frequency error based on FCCH bursts. Network synchronisation time corresponding to 95 percentile for MCL=154 dB is evaluated as 350 ms.

The missing ratio for N-SCH detection is expected to be negligible. Final value of the missing ratio will be updated in next version of this contribution.

Robustness of N-SCH channel is also verified for other radio channel conditions and it is observed that performance was not compromised.

We propose to include the concept of network synchronisation and cell detection mentioned here into text proposal as part of [2].

# REFERENCEs

- GPC150092 – “Narrow Band Hybrid Modulation for Cellular IoT”, Nokia Networks, 3GPP TSG GERAN1 Ad Hoc#1 on FS_IoT_LC

- GPC150235 – “pCR to TR 45.820 - Text proposal for N-GSM”, Nokia Networks, 3GPP TSG GERAN Ad Hoc#2 on FS_IoT_LC

- GP-150197 – “Narrowband Hybrid Modulation – NB-PDTCH design and performance”, Nokia Networks, 3GPP TSG GERAN #65

- GPC150234 – “N-GSM: Adaptive chase combining with N-MAC”, Nokia Networks, 3GPP TSG GERAN Ad Hoc#2 on FS_IoT_LC

- GP-140885 – “Frequency offset model for Cellular IoT”, Ericsson, 3GPP TSG GERAN#64

- GP-150196 – “Narrowband Hybrid Modulation SCH Design and Performance”, Nokia Networks, 3GPP TSG GERAN #65