3GPP TSG GERAN AdHoc#2 on FC_IoT_LC	GPC150232

Sophia Antipolis, France	Agenda items 1.4.2.2, 2.4.2.2

20th – 23rd April, 2015

Source: Nokia Networks

N-GSM: Design and Performance for N-RACH

# Introduction

This contribution depicts the N-RACH design for the N-GSM concept proposal earlier introduced in [1], which is proposed to be included into TR 45.820 in [2]. Section 2 describes various design aspects such as burst format and design for the path loss adapted chase combining scheme. Section 3 provides the link simulation results for N-RACH performance without collision and initial capacity estimation which will be the base for further evaluations on N-RACH performance. Section 4 summarises the key points for N-RACH and further proceeding.

# DESIGN FOR N-RACH

This section describes the N-RACH design including the new burst format with length of two slots using Narrow Band Hybrid Modulation sequences as specified in [1]. This section also describes the MS behaviour in deciding the burst type, the number of required repetitions and also the selection of repetition intervals.

## RACH procedure

### Current RACH control parameters

Currently RACH transmission is controlled by some parameters listed below. These are applied when the CIoT device is under normal coverage.

- RXLEV-ACCESS-MIN defines the minimum received signal level that must be detected by MS for accessing the network.

- MS-TXPWR-MAX-CCH [0…31] defines the maximum transmitter power level in dBm used for RACH in CCH.

- MAX-RETRANS parameter [1, 2, 4 or 7] defines the maximum number of RACH re-transmissions.

- TX-INTEGER parameter [3 - 50] in TDMA frame periods defines the maximum delay for every new randomly timed RACH attempt and is linked to S-parameter that defines spacing for next attempts.

- CELL-BAR-ACCESS and Access Class Barring can be used to inhibit the access to a cell or for certain access classes and Extended Access Barring (EAB) authorization mask introduced in Rel-10.

- INIT_PWR_RED parameter controls Rel-10 MS to reduce TX power for first random access based on DL RX level estimate.

### Doubled RACH opportunities for CIoT device under normal coverage

The CIoT device under normal coverage condition, i.e. when received signal level exceeds RXLevAccessMin, uses the existing RACH burst format in the CCH timeslot as specified in 45.002. CIoT devices may send normal RACH also in both time slots 0 and 1 on BCCH carrier, providing doubled amount of random access opportunities compared to existing RACH. Selection of which timeslot to use is random, but it is kept same during consecutive attempts.

### CIoT device beyond normal coverage

When the coverage condition is below RxlevAccessMin, new N-RACH burst format with pre-coding of data bits for Narrowband Hybrid Modulation sequences will be used. N-RACH uses different TSC code, so it can be distinguished from normal RACH types.  For allowing chase combining for up to 16 N-RACH bursts, the timing of N-RACH re-transmissions can’t be random. In order to maximise random access opportunities, the number of N-RACH repetitions is determined from DL path loss.

#### Determining number of N-RACH repetitions for chase combining

Transmission of RACH is stopped when immediate assignment is received or when the amount of re-transmissions is reached path loss based count. This latter mechanism is meant to limit unnecessary repetitions at better conditions, by signalling NRXLevAccessMin and N-ACCESS-PATHLOSS-DIVIDER [3, 4, 5 or 6] dB in N-BCCH. Number of N-RACH repetitions is be defined as:

Table 1 provides an example with RXLevAccessMin =-111dBm, MaxNumRTX=16, NRXLevAccesMin =-131 dBm for possible N-ACCESS-PATHLOSS-DIVIDER values.

Table 1 – Example for number of N-RACH repetitions

| RX levels vs. N-ACCESS-PATHLOSS-DIVIDER |  |  |  | Number of N-RACH Transmissions |
| --- | --- | --- | --- | --- |
| 3 | 4 | 5 | 6 |  |
| >-131 dBm | >-131 dBm | >-131 dBm | >-131 dBm | 16 |
| >-128 dBm | >-127 dBm | >-126 dBm | >-125 dBm | 8 |
| >-125 dBm | >-123 dBm | >-121 dBm | >-119 dBm | 4 |
| >-122 dBm | >-119 dBm | >-116 dBm | >-113 dBm | 2 |
| >-119 dBm | >-115 dBm | >-111dBm*) | >-109 dBm*) | 1 |
| >-111 dBm | >-111 dBm | >-111 dBm | >-111 dBm | Normal RACH |

*) Normal RACH is selected

#### Spacing between N-RACH re-transmissions

The spacing NSPACE between retransmissions is derived from the number of determined retransmissions and NTIN, signalled in N-BCCH, which defines the exact delay between retransmissions, as depicted in the table below. The purpose of differentiated spacing is to avoid collisions between chase combining processes and also to enable estimating the used number of repetitions in BSS as an indication of path loss seen by CIoT device.  Parameterisation allows wider spreading of repetitions, e.g., at higher load to obtain better diversity but with cost of added delay.

Table 2 Spacing of N-RACH repetitions

| NTIN | N-RACH Spacing value NSPACE vs. Number of N-RACH transmissions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 4 | 8 | 16 |
| 1 | N.A. | 33 | 15 | 7 | 4 |
| 2 | N.A. | 68 | 31 | 13 | 7 |
| 3 | N.A. | 100 | 41 | 22 | 12 |
| 4 | N.A. | 157 | 59 | 27 | 14 |

Selection of N-RACH spacing values aims for minimising collisions between two CIoT devices.

Re-attempts e.g. due to collisions for up to N-MAX-RETRANS with set of new re-transmissions may occur at earliest NTIN*2 51-multframes after the last repetition and with T3 being selected again randomly.

Example of 4 random access attempts where one N-RACH attempt is repeated 4 times with NTIN=1 (red) and 3 RACH attempts (green) with one reattempt for each with TX-INTEGER=5 is shown in the Figure 1.

Figure 1 N-RACH (red) transmitted 4 times and 3 RACH (green) attempts with a reattempt.

## N-RACH burst format

The N-RACH burst using Narrowband Hybrid Modulation to carry the same amount of encoded data-bits will require two time slots. For this purpose, new burst format using two time slots is defined as given in Table 3. By default the N-RACH burst will be mapped to TS0 and TS1. It is possible to configure the time-slots for N-RACH via System information also as similar to PRACH configurations.

Table 3 Definition of periods for N-RACH burst

| Period | Length in symbols |
| --- | --- |
| Guard symbols | 8 |
| Training Sequence | 89 |
| Data | 144 (18 NB symbol sequences) |
| Guard Symbols | 68.5 |

### Channel coding

Channel coding is like for existing RACH burst.

## Chase Combining for N-RACH bursts

BSS may process normal RACH bursts received in slot 0 linked in RX processing of timeslot 0 and RX processing of N-RACH bursts may occur at next slot, when whole transmission of two slots N-RACH is received.

In each frame BSS may try to decode possible N-RACH with 1, 2, 4, 8 or 16 repetitions, i.e. 4 decoding attempts per frame, by combining information from earlier possible locations of bursts. So chase combining memory of BTS contains information from about 1 s period (14*16=224 frames) of bursts in total with NTIN=4.

# PERFORMANCE Evaluation

## Link Simulation Assumptions

Simulation assumptions for evaluating N-RACH performance are depicted in Table 4 below.

| Parameter | Value |
| --- | --- |
| Logical Channel | N-RACH |
| Channel profile | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Number of repetitions | 8 |
| Frequency error model | Typical |

Table 4 Simulation Assumptions for N-RACH performance evaluation.

Performance for N-RACH was assessed for the sensitivity scenario without collision first to decide the base BLER Performance.

## Sensitivity

The simulation results for BLER performance for N-RACH burst with repetition interval of 8 and maximum 8 repetitions without collisions are illustrated in Figure 2 – Sensitivity of N-RACH with 8 repetitions at TU1.2nFH. Larger spacing is expected to provide further gain.

Figure 2 – Sensitivity of N-RACH with 8 repetitions at TU1.2nFH

## Link Layer Impact of collision

### Capture effect and receiver algorithms

The link level performance is known to have impact on mitigating collisions, through so called capture effect where the strongest random access burst is detected. Capture effect may provide 50% gain with capture threshold of 6dB [5] over maximum 37% throughput of slotted Aloha system. The capacity gain is also shown to improve with lowering the capture threshold, thus link performance enhancements, e.g., Interference Rejection Combining for RACH, expected to further improve the RACH capacity.

### Different Training sequences and chase combining

Use of different training sequences for RACH and N-RACH enables lower or even negative capture threshold. The chase combining process may tolerate part of N-RACH bursts colliding with RACH.

# N-RACH CAPACITY CALCULATIONS

## Assumptions

The following assumptions are made in initial capacity evaluation:

- Path loss distribution is derived from the system model [3].

- Both MAR and Network Command based traffic models [3] are assumed to generate 1 RACH per transaction.

- Initial power reduction is not used for Normal RACH.

- Only coverage constrained case without interference is modelled with 6 dB margin.

Figure 3 RX level distribution based on system model [3]

## Analytical modelling

Simple N-RACH capacity analysis [4] is made in this chapter to estimate number of RACH attempts served based on number of random access opportunities per each N-RACH repetition scheme, if all devices are with the same path loss. Estimate is given for 10% and 37% collision probabilities.

The number of random access opportunities per second (N) is given by,

N=number of frames per second *  number of slots reserved for RACHNumber of slots per attempt

For the given Intensity of RACH per seconds (λ) is, the collision probability Pc can be calculated as:

Pc=1-e-λ/N

E.g. for collision rate of 10% that would mean 99% success with second attempt.

Intensity at collision rate of Pc is given by

λ=-N*ln1-Pc

In this analysis all devices are assumed capable of N-RACH, including possibility to transmit normal RACH also in time slot 1.

Table 5 Random access throughput per second

| Minimum RX Level | RACH type, repetitions | RACH oppor-tunities per second N | Succeeded Random Access attempts per second |  | Portion of cell users in system model (coverage, with 6 dB margin) |
| --- | --- | --- | --- | --- | --- |
|  |  |  | P(c) =10% | P(c) = 37% |  |
| -131 dBm | N-RACH, 16 | 14 | 1 | 5 | 1% |
| -128 dBm | N-RACH, 8 | 27 | 3 | 10 | 1% |
| -125 dBm | N-RACH, 4 | 54 | 6 | 20 | 2% |
| -122 dBm | N-RACH, 2 | 108 | 11 | 40 | 2% |
| -119 dBm | N-RACH, 1 | 217 | 23 | 80 | 9% |
| -111 dBm | RACH, 1 | 434 | 46 | 160 | 85% |

As reference the MAR and NC traffic models generate average of 13.6 attempts per second per cell or 0.93 RACH attempts per hour/device from devices distributed evenly in cell area.

If more N-RACH capacity is needed e.g. in larger cells, it is possible to multiply with extended CCCH with 2, 3, or 4 pairs of BCCH and N-BCCH time slots.

Figure 4 Probability of collision vs. RACH and N-RACH intensity

Figure 5 RACH and N-RACH Throughput vs. Attempt Intensity without impacts of capture effect and chase combining

With given system assumptions of 52547 CIoT devices producing intensity of 13.6 random access attempts per second per cell, it was estimated that collision probability for first attempt in coverage limited scenario with 6 dB margin is 2.6% with N-RACH.

# Conclusion

- Initial analytical system modelling based on path loss with 6dB margin shows that RACH load in given system and traffic models [3] can be served with random access collision probability of 2.6% for the first attempt.

- Link simulations including study for capture thresholds with the same and different training sequences and partial collisions with chase combining are for further study.

- Section 2 is proposed to be included in TR 45.820.

# REFERENCEs

- GPC150092 – “Narrow Band Hybrid Modulation for Cellular IoT”, Nokia Networks, 3GPP TSG GERAN1 Adhoc#1 on FS_IoT_LC

- GPC150235 – “pCR to TR 45.820 - Text proposal for N-GSM”, Nokia Networks, 3GPP TSG GERAN Ad Hoc#2 on FS_IoT_LC

- 3GPP TR 45.820 v1.0.1, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, Rapporteur (Vodafone Plc)

- 3GPP TR 37.868 Study on RAN Improvements for Machine- type Communications

- Lüders, C. , Haferbeck, R. , “The performance of the GSM random access procedure”, Vehicular Technology Conference, 1994