3GPP TSG GERAN Ad Hoc#2 on FS_IoT_LC	GPC150233

Sophia Antipolis, France	Agenda Items 1.4.2.2, 2.4.2.2

20th – 23rd April, 2015

Source: Nokia Networks

N-GSM: Design and Performance for N-PDTCH

# Introduction

This contribution depicts the N-PDTCH design and its performance for the N-GSM concept proposal earlier introduced in [1], which is proposed to be included into TR 45.820 in [2]. Section 2 describes design aspects such as burst format and block structures for N-PDTCH and other layer 1 parameter settings. Performance for N-PDTCH is investigated in section. The further proceeding is outlined in section 4. This contribution is an update to [3] with following changes:

- The burst structure and block structure characteristics for N-PDTCH supporting the embedded transmission of the N-MAC header info are described.

- Performance evaluation results are updated for data payload with modified physical layer parameters.

- Performance evaluation results are added for N-MAC header data.

# DESIGN FOR N-PDTCH

This section describes the N-PDTCH design in terms of burst format and layer 1 parameter settings.

## Normal burst format for N-PDTCH

The narrow band (NB) sequences each having a length of 8 normal symbol periods fit into the current burst structure with minor modifications. Figure 1 depicts the burst structure and Table 1 shows the length of different sections of a burst in normal symbol periods in a time slot carrying the N-PDTCH.

| Tail symbols | User payload symbols | N-MAC header symbol | TSC symbols | N-MAC header symbol | User payload symbols | Tail symbols | Guard Period |
| --- | --- | --- | --- | --- | --- | --- | --- |

Figure 1:  Normal burst format for N-PDTCH.

| Burst Content | PDTCH normal burst [bits] | N-PDTCH burst carrying NB sequences in payload [bits] |
| --- | --- | --- |
| Front tail symbols | 3 | 3 |
| User payload symbols (1st part) | 57 | 48 (6 NB sequences) |
| N-MAC header symbols | N.A. | 8 (1 NB sequence) |
| Stealing flag/CS indicator | 1 | 0 |
| Training Sequence | 26 | 30 |
| Stealing flag/MCS indicator | 1 | 0 |
| N-MAC header symbols | N.A. | 8 (1 NB sequence) |
| User payload symbols (2nd part) | 57 | 48 (6 NB sequences) |
| Tail symbols | 3 | 3 |
| Guard period | 8.25 | 8.25 |

Table 1:  Burst format for legacy PDTCH and for N-PDTCH.

The lengths are given in number of narrow band sequences for the new burst in parenthesis. The need for stealing bits has not yet been identified in order to signal the coding scheme, thus the TSC length is extended to 30 symbols. For the prolongeded TSC the legacy TSC is used as basis and extended by 2 symbols on either side. The two symbols on either side are choosen such that the auto-correlation properties of the TSC are optimized. The extended TSC has increased energy and thus allows for a relative improvement in sensitivity up to 0.6 dB against the legacy TSC. Within a single N-PDTCH burst a total of 14 NB sequences is available for carrying data bits. In order to allow chase combining of radio blocks the two NB sequences surrounding the TSC are reserved for carrying the N-MAC header.

## Precoding of Information into NB Sequences for N-PDTCH

Precoding selects a NB sequence based on input bits. These NB sequences in effect create 2-FSK+BPSK modulation waveforms over the duration of the NB sequence, allowing for mapping a pair of data bits to 1 NB sequence as shown in Table 2.

| Data bits | NB sequence | Resulting phase | Resulting FSK tone |
| --- | --- | --- | --- |
| 00 | 00000000 | +1 | +1/(4TS) = +68 kHz |
| 01 | 11111111 | -1 | +1/(4TS) = +68 kHz |
| 10 | 10101010 | +1 | -1/(4TS) = -68 kHz |
| 11 | 01010101 | -1 | -1/(4TS) = -68 kHz |

Table 2:  Precoding using 2-FSK+BPSK modulation.

With the above precoding 28 data bits can be carried per burst. 4 bits of them are dedicated to transport encoded N-MAC header info, whilst 24 bits transport of encoded user data bits.

## RLC Block Structure for N-PDTCH

The information block, delivered to the channel encoder, has a fixed size of 186 information bits {d(0),d(1),...,d(185)}, is depicted in Figure 2 and constitute of the following parts:

- N-RLC header: 8 bits (for DL and UL)

- Parity information for N-RLC/N-MAC headers: 18 bits (for DL and UL)

Note: derivation of the parity information is FFS (e.g. based on NBSN and NTI from N-MAC)

- User payload data (160 bit)

| N-RLC header | Parity information | User payload |
| --- | --- | --- |

Figure 2:  RLC block structure for N-PDTCH.

## Channel Encoding

Channel encoding for N-PDTCH is done using convolutional encoding with constraint length 7 as described below.

1. Addition of tail bits:

Six tail bits equal to 0 are added to the information block described above, the result being a block of 192 bits {u(0),u(1),...,u(191)}:

u(k) = d(k)	for k = 0,1,...,185

u(k) = 0	for k = 186,187,…,191 (tail bits)

2. Convolutional encoder

This block of 192 bits {u(0),u(1),...,u(191)} is encoded with the 1/2 rate convolution code defined by the polynomials:

G4 = 1 + D2 + D3 + D5 + D6

G7 = 1 + D + D2 + D3 + D6

This results in a block of 384 encoded bits: {C(0),C(1),...,C(383)} defined by:

C(2k)     = u(k) + u(k-2) + u(k-3) + u(k-5) + u(k-6)

C(2k+1) = u(k) + u(k-1) + u(k-2) + u(k-3) + u(k-6)

for k = 0,1,...,191; u(k) = 0 for k < 0

## Interleaving for N-PDTCH

For the block of 384 encoded bits rectangular interleaving over 16 bursts (i.e. TTI = 80 ms) is applied as shown in the description below with B indicating the burst number and j the position in this burst:

for (k=0 to 383)

{

B=mod(12*k+floor(k/2)+mod(k,2),16);

j=mod(23*mod(5*k,24)+ floor(7*k/16),24);

}

## Block Structure for N-MAC header

Two NB sequences in each burst are used to encode the N-MAC header as depicted in section 2.1. Thus a total of 64 bits is encoded over 32 NB sequences available in 16 bursts (TTI = 80 ms).

N-MAC header block structure and encoding is different for uplink and downlink and is detailed below.

### Downlink N-MAC Header

In downlink the N-MAC header payload size is 8 bits, see [4]. The header contains information about the block sequence number (NBSN) and about the transaction ID (NTI), each of them having a length of 4 bits.

#### Block Structure for DL N-MAC header

Table 3 below provides high level summary of the block structure and encoding used for downlink N-MAC.

| DL N-MAC header part | Value |
| --- | --- |
| Payload bits | 8 bits |
| Tail bits | 4 bits |
| Check sum | 1 bit |
| Convolutional coding | 1/5 |
| Puncturing | 1 bits |
| Final Encoded Bits | 64 bits |

Table 3:  Block Structure for DL N-MAC header.

#### Interleaving

N-MAC header is interleaved over 16 bursts (TTI = 80 ms). It is mapped to NB sequences surrounding the TSC. Details are FFS.

### Uplink N-MAC header

In uplink the N-MAC header has a payload size of 4 bits, since it contains merely information about the block sequence number (NBSN).

#### Block Structure UL N-MAC header

Table 4 below provides high level summary of the block structure and coding used in uplink N-MAC.

| UL N-MAC header part | Value |
| --- | --- |
| Payload bits | 4 bits |
| Tail bits | 4 bits |
| Convolutional coding | 1/4 |
| Repetition | Repetition of convolutional encoded bits (32) |
| Final Encoded Bits | 64 bits |

Table 4:  Block Structure for UL N-MAC header.

#### Interleaving

N-MAC header is interleaved over 16 bursts (TTI = 80 ms) as follows.

for k=0 to 63
{
  B=mod(k*7+floor(k/16),16);
  j=mod(3*mod(k,3)+ floor(k/3)+mod(3*B,7),4);
 }

It is mapped to NB sequences surrounding the TSC.

## Block Repetition mode

Different block repetition modes can be configured for N-PDTCH:

- Using single block transmission with TTI of 80 ms.

- Using concatenated block transmission within M_init*80 ms based on initial block transmission with TTI of 80 ms and successive (M_init-1) repetitions with the same TTI. This results in an extended TTI of M_init*80 ms.

It is assumed that the block repetition mode is selected by the network and sent to the CIoT device at packet traffic channel assignment. The repetition mode is applicable for both N-PDTCH user payload and N-MAC header data.

## Maximum Number of repetitions

Maximum number of repetitions to provide required coverage performance for target coverage is FFS. With the use of IP header compression at SNDCP layer it is possible to increase the number of repetitions to higher value without impacting throughput at layer above SNDCP.

# PERFORMANCE Evaluation

## Simulation Assumptions

Simulation assumptions for evaluating N-PDTCH performance are depicted in Table 5 below. The investigation here was limited to one precoding scheme.

| Parameter | Value |
| --- | --- |
| Logical Channel | N-PDTCH |
| Frequency band | 900 MHz |
| Interference profile | Sensitivity |
| Input | ES/N0 |
| Output | - BLER for N-PDTCH user data - BLER for N-MAC header data |
| Propagation conditions | TU |
| Mobile speeds | 1.2 km/hr (corresponding to 1 Hz Doppler) |
| Frequency hopping | non hopping |
| BTS RF impairments | Typical |
| MS RF impairments | Typical |
| Frequency error model | typical DL: 40 Hz UL: model is described in section 2.1 of [5] |
| Block repetition mode | - Single block transmission (TTI=80 ms) - Concatenated block transmission (TTI=80ms, M_init = 3 resulting in extended TTI of 240 ms) |
| Chase combining | Maximum repetitions (M_max): - Single block transmission: 11 or 14 repetitions - Concatenated block transmission: 3 or 4 repetitions Spacing between blocks used for chase combining: - 5 RLC blocks in between (single block transmission) - no RLC block in between (concatenated block transmission) |
| Antenna configuration | BTS: 1Tx, 2Rx MS: 1Tx, 1Rx |
| Precoding | 2-FSK+BPSK modulation |

Table 5: Simulation Assumptions for N-PDTCH performance evaluation.

For these simulations, a user data payload of 120 B was assumed, resulting in the transmission of 6 consecutive single N-RLC blocks or 6 N-RLC blocks in concatenated mode (with extended TTI of 240 ms), before a N-RLC block is retransmitted. For single block repetition mode, the spacing between chase combined radio blocks due to retransmissions using HARQ will thus be 6*TTI = 6*80 ms = 480 ms. For concatenated repetition mode in 240 ms, the chase combining spacing is 80 ms and 160 ms and for retransmissions due to HARQ enlarged to 6*240 ms =1440 ms, see [4].

The simulation results are presented for 11 and 14 repetitions with single block repetition mode and also for 3 and 4 repetitions for concatenated repetition mode.

Calculation of maximum possible transmission for achieving 160 bps throughput for transfer of MAR with IP header compression:

For transmission of MAR of 42B, MS generates IP packet of size 107 Bytes (IP header 65 bytes). At SNDCP/LLC layer with IP Header compression and addition of overhead for these layers, the LLC PDU generated for transmission will be of size 42 B+29 B (Reduced Header) +10 B = 81 B.

For 81 B RLC level payload, with RLC PDU size of 20 B, the number of transmissions calculated corresponds to 160 bps throughput with 10% BLER is 15 transmissions as given below:

Number of Maximum repetitions = [[107*8/160]*0.9]*1000ms / [[81*8/160]*80ms] = 14.86.

## Simulation Results

N-PDTCH performance was evaluated here only for the non-hopping channel as the performance limiting radio channel using the block encoding and burst structure as explained in section 2.

### Downlink BLER for user data

Results for downlink N-PDTCH user data are shown in Figure 3 for single block transmission and in Figure 4 for concatenated block transmission.

11 repetitions

14 repetitions

Figure 3: DL BLER vs Es/N0 for N-PDTCH, single block transmission.

It is observed that for single block repetition mode the MCL of 164 dB, corresponding to Es/No = -6.3 dB, is met with a negligible BLER in both cases. 10% BLER is met with a margin of about 3.5 dB for 11 repetitions and 4.5 dB for 14 repetitions, which are spaced by 480 ms.

4 retrans.

3 retrans.

N-PDTCH: BLER for 240ms extended TTI (DL Sensitivity, TU1.2nFH)

3 Repetitions

4 Repetitions

Figure 4: DL BLER vs Es/N0 for N-PDTCH, concatenated block transmission.

It is observed that for concatenated block repetition mode the MCL of 164 dB, corresponding to Es/No = -6.3 dB, is again met with a negligible BLER. 10% BLER is met with a margin of about 4.7 dB for 4 retransmissions and 3.7 dB for 3 retransmissions both spaced by 1440 ms.

### Downlink BLER for N-MAC

Results for downlink N-MAC header data are shown in Figure 5 for single block transmission and in Figure 6 for concatenated block transmission.

Figure 5: DL BLER vs Es/N0 for N-MAC, single block transmission.

It is observed that for single block repetition mode (without repetition of N-MAC) the MCL of 164 dB, corresponding to Es/No = -6.3 dB, 10% BLER is met with a negative margin of 6 dB, i.e. MCL = 158 dB.

Figure 6: DL BLER vs Es/N0 for N-MAC, concatenated block transmission.

It is observed that for concatenated block repetition mode (with 2 repetitions for N-MAC) and MCL of 164 dB, corresponding to Es/No = -6.3 dB, 10% BLER is met with a negative margin of only 0.8 dB. This is considered as sufficiently robust to provide the expected performance for the user payload data.

### Uplink BLER for user data

Results for uplink N-PDTCH user data are shown in Figure 7 for single block transmission and in Figure 8 for concatenated block transmission.

11 repetitions

14 repetitions

Figure 7: UL BLER vs Es/N0 for N-PDTCH, single block transmission.

It is observed that for single block repetition mode the MCL of 164 dB, corresponding to Es/No = -14.3 dB, is not met in both cases. 10% BLER is met with a negative margin of about 1.4 dB for 11 repetitions and only 0.5 dB for 14 repetitions, which are spaced by 480 ms.

4 retrans.

3 retrans.

N-PDTCH: BLER for 240ms extended TTI (DL Sensitivity, TU1.2nFH)

Figure 8: UL BLER vs Es/N0 for N-PDTCH, concatenated block transmission.

It is observed that for concatenated block repetition mode the MCL of 164 dB, corresponding to Es/No = -14.3 dB,  10% BLER is met with a negative margin of 1.4 dB for 3 retransmissions and only 0.5 dB for 4 retransmissions spaced by 1440 ms.

### Uplink BLER for N-MAC

Results for uplink N-MAC header data are shown in Figure 9 for single block transmission and in Figure 10 for concatenated block transmission.

Figure 9: UL BLER vs Es/N0 for N-MAC, single block transmission.

It is observed that for single block repetition mode (without repetition of N-MAC) and MCLof 164 dB, corresponding to Es/No = -14.3 dB, 10% BLER is met with a negative margin of 2.3 dB, i.e. MCL = 161.7 dB.

Figure 10: UL BLER vs Es/N0 for N-MAC, concatenated block transmission.

It is observed that for concatenated block repetition mode (with 2 repetitions for N-MAC) and MCL of 164 dB, corresponding to Es/No = -14.3 dB, 10% BLER is met with a negative margin of only 0.3 dB. This is considered as sufficiently robust to provide the expected performance for the user payload data.

## Discussion

Performance evaluation for DL and UL indicates that the concatenated block repetition mode with extended TTI is required for the extreme MCL range up to 164 dB, whilst single block repetition is efficient for the MCL range from 144 dB up to around 158 dB.

# Conclusion

In this contribution the N-PDTCH design for the N-GSM concept is presented and the sensitivity performance is assessed for the promising precoding scheme 2-FSK+BPSK. It is observed that the MCL target of 164 dB is met for N-PDTCH payload data in DL for single block repetition mode with 11 repetitions or 14 repetitions and also for concatenated block repetition mode with 3 or 4 retransmissions. The limiting factor has been identified to be N-MAC header, which requires the operation of the concatenated repetition mode for the extreme MCL range up to 164 dB. Performance for N-PDTCH payload data in UL is close to target MCL of 164 in case of concatenated repetition mode and inferior for single block transmission mode.

This suggests that the concatenated block repetition mode with extended TTI is required for the extreme MCL range up to 164 dB, whilst single block repetition is efficient for the MCL range from 144 dB up to around 158 dB.

Further consideration will be put on evaluating the concurrent reception of N-MAC header data and N-PDTCH payload data to optimise the performance in DL and UL.

The N-PDTCH design is included in the pCR to TR 45.820 [2].

# REFERENCEs

- GPC150092 – “Narrow Band Hybrid Modulation for Cellular IoT”, Nokia Networks, 3GPP TSG GERAN1 Ad Hoc#1 on FS_IoT_LC

- GPC150235 – “pCR to TR 45.820 - Text proposal for N-GSM”, Nokia Networks, 3GPP TSG GERAN Ad Hoc#2 on FS_IoT_LC

- GP-150197 – “Narrowband Hybrid Modulation – NB-PDTCH design and performance”, Nokia Networks, 3GPP TSG GERAN #65

- GPC150234 – “N-GSM: Adaptive chase combining with N-MAC”, Nokia Networks, 3GPP TSG GERAN #65

- GP-140885 – “Frequency offset model for Cellular IoT”, Ericsson, 3GPP TSG GERAN#64