# EC-GSM – EC-BCCH link-Level performance

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved [1].

One of the main objectives in FS_IoT_LC is to increase the coverage compared to existing GPRS services. In case of evolving GSM, a straightforward way is to make use of blind repetitions. This is what has been proposed for the Extended Coverage GSM (EC-GSM) concept (earlier known as GSM Evolution), see [2].

In this contribution, the link level performance of EC-BCCH with blind repetitions is provided, and it shows current repetition scheme can achieve the -6.3dB SNR requirement with a low complexity implementation.

# EC-BCCH burst mapping with blind repetition

Figure 1 EC-BCCH with normal and extended mode

As Figure 1 shows, one EC-BCCH burst is repeated 32 times in 16 51-multiframe [3].

# Simulations

## Simulation assumptions

Simulations have been run for EC-BCCH with extended mode.

Simulation parameters are listed in Table 1.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Logical Channel | EC-BCCH |
| Channel | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Impairments | Typical RX and TX. Frequency offset=3,30,90 |
| AFC | no |
| Number of repetitions | 32 (extended mode) |
| BLER requirement | 10% |

In the simulation, combining is implemented only in the soft-bit level but not in IQ level. Therefore, a lower complexity implementation is possible.

## Simulation Results

Figure 2. Performance of EC-BCCH with extended coverage (32 repetitions)

It can be seen from Figure 2 that EC-BCCH with the extended coverage, i.e. up to 32 blind repetitions in 15 51-multiframes, can achieve 10% BLER at -7.2dB SNR. It is also observed that it is not sensitive to the frequency offset (FO). The performance with 90Hz FO is almost same as without FO.

# Conclusions

The simulation result confirms that current design of EC-BCCH can satisfy the -6.3dB SNR requirement with a low complexity implementation.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

GP-150317, “3GPP TR 45.820 V1.0.0 (2015-03)”