# EC-GSM – EC-PDTCH Link Level Performance

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

One of the main objectives in FS_IoT_LC is to increase the coverage compared to existing GPRS services. In case of evolving GSM, a straightforward way is to make use of blind repetitions. This is what has been proposed for the Extended Coverage GSM (EC-GSM) concept (earlier known as GSM Evolution), see [2].

In this contribution, the link level performance of EC-PDTCH with blind repetitions is provided, and it shows current repetition scheme can achieve the -6.3dB SNR requirement with a low complexity implementation even with only 1 HARQ transmission (no retransmission).

# EC-PDTCH burst mapping with blind repetition

Figure 1 EC-PDTCH blind repetitions with CC=6

As Figure 1 shows, one PDTCH burst is repeated 4 times in one TDMA frame and the 4-frame PDTCH block is repeated 4 times in 16 TDMA frames for coverage class (CC) 6[3]. Since frequency hopping can be used between different TDMA frames, usually IQ accumulation is used only within a single TDMA frame.

# Simulations

## Simulation assumptions

Simulations were performed for EC-PDTCH with CC 6.

Simulation parameters are listed in Table 1.

| Parameter | Value |
| --- | --- |
| Logical Channel | EC-PDTCH(MCS-1) |
| Channel | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Impairments | Typical RX and TX. Frequency offset=3,30,90 |
| Frequency Hopping | Ideal Frequency Hopping (iFH) |
| AFC | no |
| Number of repetitions | 16 |
| BLER requirement | 10% |

In the simulation, IQ accumulation is used inside one TDMA frames. Since there is no need to compensate the frequency offset after receiving the data, the IQ accumulation is very

straightforward and enables low cost implementation, which means it can be done in real time, and no need to buffer all repeated bursts in one TDMA frames. Besides IQ accumulation, a simple soft-bit combining is used.

## Results

Figure 2. Performance of EC-PDTCH with Coverage Class 6 and up to 4 HARQs

As can be seen, with 4 burst IQ accumulation, for Coverage Class 6, EC-PDTCH can achieve 10% BLER at -9.3dB SNR with up to 4 HARQs. When the frequency offset increases to 30Hz, very limited performance degradation is observed. It can also be seen that only 1dB degradation is observed when the frequency offset is increased to 90Hz.

Figure 3. Performance of EC-PDTCH with Coverage Class 6 with only 1 HARQ

As shown in Figure 3, only 1 HARQ retransmission is sufficient to meet the MCL target if the FO is lower than 30 Hz. MCL target is achieved even with 90Hz FO with a small margin.

# Conclusions

The simulation results confirm that current EC-PDTCH blind repetition scheme can achieve the -6.3dB SNR requirement with a low complexity implementation even with only 1 HARQ transmission (no retransmission).

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GP-140882, “GSM Evolution for cellular IoT – On using blind repetitions”, source Ericsson LM. GERAN#64.

GP-150317, “3GPP TR 45.820 V1.0.0 (2015-03)”