# Core network assistance for paging devices in extended coverage

# (Update of GPC150227)

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (WI code: FS_IoT_LC)  was approved, see [1].

At the first Cellular Adhoc meeting there were several contributions addressing the need for core network assistance for transmission of paging as well as how to convey the last know coverage class to the core network nodes [2][3][4].  There was in addition incoming LS from SA2 triggered by RAN1 on the topic [5].  An LS response was prepared but in the end postponed in order to be able to provide SA2 with more details concerning a desired solution.

This paper proposes a set of common working assumptions that may be used as input to the Reply LS on paging for MTC to SA2.

Updates are highlighted in red and have been concentrated to updates of the Working assumptions with the purpose to Liaise to SA2 a more complete description of the functionality needed for core network assistance for transmission of paging to device in extended coverage and eDRX mode in GERAN.  New working assumptions have been added implying different numbering compared to the previous version.

# At Routing Area Update

All proposals mention the need for device specific DL coverage class information to be conveyed to the SGSN at either Routing Area Update or at Attach.  In particular, whenever a MS changes Routing Area the new SGSN needs to be updated with the latest known coverage class in order to ensure that any subsequent paging is sent according to the latest coverage class. It appears that the most straightforward way for the MS to include the latest coverage class to the SGSN is to include it in the NAS message but in order to have a single solution for both the MS and SGSN for all scenarios (see section 3 and 4) it is proposed to use the same method as in packet transfer mode (see section 4) since the MS will always enter this mode when performing a RAU.

# Idle Mode update of coverage class

When analyzing all proposals it is clear that a common approach is that there is a need for a device in (packet) idle mode to notify the SGSN whenever it determines that the coverage class has changed (especially for the case where the radio environment has become worse). It should be noted that this only applies for devices that support eDRX based reachability where the notification can potentially be sent to the SGSN at any point in time other than when a device is in a power saving state. This is to ensure that any subsequent paging message is sent according to the latest known device coverage class. In [4] it is proposed that the Routing area update procedure is used for conveying the last known coverage class while in [2] and [3] it is proposed to use either the Cell update or similar procedure that minimizes the volume of signaling information a devices needs to send to the SGSN.  Given that the two latter approaches are more battery efficient and that no additional RAI has to be configured in the BSS it is suggested to not further pursue the Routing Area update based approach (other when the RAU procedure is performed due to a change in routing area). For EC-GSM it is furthermore suggested to use the Cell Update procedure for conveying change of coverage class in idle mode since this doesn’t require an introduction of an additional code point in the RACH message to indicate coverage class update.  As the BSS will be aware of the new coverage class through code points in the RACH message the most straight forward way to convey the new coverage class is to introduce a new IE in UL-UNITDATA PDU. Note that this is a BSSGP PDU used to transfer an LLC-PDU and its associated radio interface information across the Gb-interface.

WA 1: The MS shall determine if currently estimated coverage class is different from last reported Coverage Class.

WA 2: The MS should report changes of its coverage class to the SGSN. The trigger of the reporting, to avoid frequent signaling, is FFS.

WA 3:  At least the estimated coverage class for downlink will be indicated to the BSC when attempting system access.

# Packet transfer mode

As identified both in the proposals by Ericsson [2] and Huawei et al [3] it is sometimes necessary to update the SGSN with the latest known coverage class while a device is in packet transfer mode. As the BSS assigns/modifies the resources used for all TBFs it will always be aware of the DL coverage class it has last commanded a device to use while there is an on-going packet transfer. As such, whenever it receives a LLC PDU from a device it can forward it along with a new IE (defined at the BSSGP layer) that contains the latest DL coverage class.

WA4: The changes of estimated coverage class for downlink may be indicated to the base station during data transmission.

WA 5: The BSS will include the coverage class information in the UL-UNITDATA PDU sent to the SGSN.

WA 6: Upon reception of the device specific coverage class information the SGSN stores it for use in subsequent paging for delivery of downlink data to that device.

# Paging in the BSS

Whenever the BSS needs to send a page to a device in extended coverage it needs to know the current coverage class of the device in order to determine the appropriate number of repetitions to use on the radio interface. If there is no MS context in the BSS the SGSN has to provide the BSS with this information. It is therefore proposed to add a new IE to the PAGING-PS PDU to convey the latest known coverage class from the SGSN to the BSS.

WA 7: In order for BSS to send a page with the appropriate coverage enhancements the SGSN needs to convey the latest known coverage class information in the PAGING-PS PDU.

# Conclusion

This paper proposes a set of common working assumptions related to core network assistance for paging devices in extended coverage. It is proposed to adopt the working assumptions and use them as input for the LS reply to SA2 on paging for MTC.

WA 1: The MS shall determine if currently estimated coverage class is different from last reported Coverage Class.

WA 2: The MS should report changes of its coverage class to the SGSN. The trigger of the reporting, to avoid frequent signaling, is FFS.

WA 3:  At least the estimated coverage class for downlink will be indicated to the BSC when attempting system access.

WA4: The changes of estimated coverage class for downlink may be indicated to the base station during data transmission.

WA 5: The BSS will include the coverage class information in the UL-UNITDATA PDU sent to the SGSN.

WA 6: Upon reception of the device specific coverage class information the SGSN stores it for use in subsequent paging for delivery of downlink data to that device.

WA 7: In order for BSS to send a page with the appropriate coverage enhancements the SGSN needs to convey the latest known coverage class information in the PAGING-PS PDU.

# References

GP-140421,  “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GPC150077, “EC-GSM, Dynamic Coverage Class Update”, Source Ericsson LM, GERAN Ad Hoc#1 on FS_IoT_LC.

GPC150050, “NB M2M - Paging for Cellular IoT”, Source Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., GERAN Ad Hoc#1 on FS_IoT_LC.

GPC150087, “Radio interface Paging & Routeing Area concept”, Source Vodafone GROUP Plc, GERAN Ad Hoc#1 on FS_IoT_LC

GPC150098, “Reply LS on paging for MTC”, Source SA2, GERAN Ad Hoc#1 on FS_IoT_LC

GPC160xxx, “Cellular IoT, Mobility Management”, Source Ericsson LM, GERAN Ad Hoc#2 on FS_IoT_LC