C-UNB technology for Cellular IoT

-

Downlink Physical Layer Design

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. Various radio access technology proposals are candidates for this WI. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB), for which key features are:

ultra-narrow band transmission,

random access,

cooperative reception.

Overall description of the C-UNB radio access technology can be found in [2]. The present document details the design of the C-UNB physical layer in downlink (DL). It is a revision of [3], where changes are in red.

# DL Physical design

## Bit rate

The downlink bit rate is set to 600bps. This value is low enough to have UNB transmissions. It is higher that the UL transmission rate to get a reduced transmission time of DL radio packets, hence reducing the reception time in devices.

## Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

## Transmission power

The sensitivity of receivers in devices is slightly lower than receivers in base stations, because the reception bandwidth can't be as narrow as what we have in base stations. The bit rate is higher. Given the maximum value of +164dB for the DL MCL, it is proposed to set the maximum transmission power on each ad'hoc micro-channel to +34dBm.

### Micro-channel power management

In practice, actual transmission power in downlink is adjusted according to the strength of the received signal upstream. This very simple power management can be implemented because every DL transmission is triggered by a UL packet.

If we consider the regular distribution of devices in a cell, the mean distance to the base station is 2/3 of the max distance. This mean value improves the link budget by about 5 dB.

Figure 1: Relationship between frequencies of corresponding UL and DL micro-channels

### Maximum PSD in base station

The power spectral density (PSD) of a GSM/GPRS base station is an important limiting factor for C-UNB, because the total available power is split among all the DL micro-channels, which are transmitted simultaneously. The worst case is where all micro-channel transmissions are aimed at devices close to the cell border: each micro channel is set to its maximum transmit power, i.e. +34dBm. Given the PSD of +43dBm in GSM/GPRS base stations, the total number of micro-channels, which can be transmitted simultaneously, is 8.

If we consider the mean distance of devices, the mean transmit power is +29dBm. Hence, the mean number of DL micro-channels is about 25.

The values (i.e. 8 max and 25 mean) are reasonable for carrying the traffic in downstream.

## Center frequency in DL

The center frequency of each DL micro-channel is chosen according to the actual frequencies used by the device during uplink transmission. The precise center frequency of a given micro-channel is not known a priori. It is calculated for each downlink message (see figure 1).

This feature helps reducing the complexity and cost of devices, because no automatic frequency control is needed during reception in devices: the AFC is performed by the base station itself.

## Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 1200 Hz. Pulse shaping filter is given in figure 2, whereas figure 3 gives the corresponding spectrum mask.

Figure 2: Pulse shaping filter in DL

Figure 3: Spectrum mask of DL in C-UNB

## Error correction code

Each MAC-PDU is protected by an Error Correction Code (ECC) of 32 bits. Given the max length of a MAC-PDU of 384 bits, this ECC adds a 1dB gain to the MCL.

# Downlink MCL

Table 1 gives the maximum coupling loss (MCL) of the downlink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 of TR 36.888 [4]. ]. Figures in table 1 are common, except the SINR, which is explained here after.

Subclause 5.1 of TR 36.888 [4] states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message : when a MS moves at less than 30 km/h, the Doppler shift is about 25Hz. This small shift is not seen by the MS, which band filter is larger than the signal (i.e. 1kHz vs 600Hz).

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are relatively small when the device speed is lower than 30km/h: they don't move the received signal out of the 1kHz band of the MS receiver

fast fading, which is critical in GSM, is not an issue for C-UNB, because the symbol duration is 1,66 ms in C-UNB DL transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception or multiple transmissions. Considering the duration of DL radio packets (i.e. about 0,2-0,6 s), we propose to have four blind retransmissions of each DL radio packet.

When putting all this together, we propose to set the SINR value to 9dB. Table 1 gives the Maximum Coupling Loss of the C-UNB downlink transmission.

Table 1: Maximum Coupling Loss in C-UNB downlink

| Logical channel name | C-UNB downlink |
| --- | --- |
| Data rate (bps) | 600 |
| Transmitter |  |
| (1) Tx power (dBm) | +34 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 6 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 1000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -138 |
| (7) Required SINR (dB) | 9 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

# Conclusion

This contribution details the DL physical layer design of the C-UNB technology and shows how this radio access technology can meet requirements from Cellular IoT with very limited complexity in the device modems.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150364, "Cooperative Ultra Narrow Band for Cellular IoT - General Description", SIGFOX, GERAN Ad'hoc meeting#3, 29 June - 02 July 2015, Kista, Sweden

[3]	GPC150143, "C-UNB technology for Cellular IoT - Downlink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting on CIot #2, 20-23 April 2015, Sophia Antipolis, France

[4]	TR 36.888, "Study on provision of low cost Machine-Type Communication (MTC) User Equipment (UEs) based on LTE (v12.0.0)"