C-UNB for Cellular IoT

-

Battery Life evaluation

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. The Cooperative Ultra Narrow Band (C-UNB) is a clean slate solution, whose general concepts are described in [2], and radio frames and link layer are defined in [3], [4] and [5].

This document evaluates the battery life of C-UNB MSs. It is a revised extract of document [6].

# Evaluation methodology

The battery life evaluation for C-UNB MS is based on methodology detailed in [7], but with two simplifying assumptions due to the C-UNB radio access technology:

MAR are supposed to be transmitted with two repetitions at physical layer,

no acknowledgement is transmitted in DL.

The second assumption is valid as long as the collision rate is less than 10%, which is the case in all performance evaluations because of network design.

The battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 1. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 1: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU |
| --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 |
| 200 | 7 | 29 | 30 | 46,5 | 14 |

Table 2 gives key parameters for energy consumption analysis, whereas figure 1 gives energy consumption model for C-UNB devices. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission.

Table 2: Key input parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power when Idle but not in PSS (mW) | (5) Battery power in Power Save State (PSS) (mW) | (6) Idle time from PSS to Tx mode (ms) | (7) Tx time (ms) | (8) Idle time between two Tx (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 | 50 | MAC-PDU size/bit rate | 25 |

Figure 1: Energy consumption model for C-UNB devices

# Results

Table 3 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage levels.

Table 3: Battery life (in years)

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia.

[2]	GPC150364, "Cooperative Ultra Narrow Band - General Description", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[3] 	GPC150309, "C-UNB technology for Cellular IoT - Downlink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[4] 	GPC150310, "C-UNB technology for Cellular IoT - Uplink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[5] 	GPC150311, "C-UNB Link Layer Aspects", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[6]	GP-150059, "C-UNB technology for Cellular IoT - Performance evaluation", SIGFOX, GERAN#65 meeting, Shanghai, 09-12 March 2015, PRC

[7]	TR 45.820, "3GPP/GERAN; Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things", v1.3.1, June 2015