C-UNB for Cellular IoT - Latency evaluation

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. The Cooperative Ultra Narrow Band (C-UNB) is a clean slate solution, whose general concept is described in [2]. C-UNB radio frames and link layer are defined in [3], [4] and [5].

This document evaluates the latency of C-UNB networks during transmission of MAR exception reports.

# Evaluation methodology

## Header

Annex E.2.1 of document [6] defines a MAR exception report payload of 20 bytes; the protocol overhead is 29 bytes (compressed) or 65 bytes (full overhead).

In C-UNB, the MAC-PDUs have relatively small payload size, resulting in segmentation when long application data have to be transmitted. In order to reduce the among of data carried over the air interface, the preferred implementation of the C-UNB protocol is to skip the transmission of the protocol header and let it manage by the MS-avatar hosted in the C-UNB server (see [7] for system architecture).

Nevertheless, this contribution evaluates latency with and without protocol header for the sake of comparison.

## Blind repetition

The C-UNB radio access technology assumes a number of blind repetitions for each MAC-PDU. Blind repetitions helps increasing the probability of good reception but reduces the overall network capacity. MAR exception reports require a high quality of service, therefore the present contribution considers that each MAC-PDU is transmitted with three PHY-PDUs (i.e. three repetition of the same radio packet) and then acknowledged by the base station.

# Results

Table 1 details latency calculation for a MAR exception report. Figure 1 illustrates the UL and DL transmission without and with protocol overhead.

Table 1: latency evaluation (size in bytes and time in ms)

| Step | Item | transmission with 29 protocol overhead |  |  | transmission with protocol overhead managed by MS-Avatar |  |
| --- | --- | --- | --- | --- | --- | --- |
| MAR exception report transmission | application data | 20 |  |  | 20 |  |
|  | protocol overhead | 29 |  |  | 0 |  |
|  | segmentation | 2x 25 |  |  | 1x 20 |  |
|  | LLC-PDU # and size | 2x 26 |  |  | 1x 20 |  |
|  | MAC-PDU # and size | 2x 42,5 |  |  | 1x 36,5 |  |
|  | PHY-PDU # and duration | 6x 1 360 |  |  | 3x 1 168 |  |
|  | radio wake up time | 50 |  |  | 50 |  |
|  | gap between 2 radio packet | 5 |  |  | 5 |  |
| Loop-back | network loop-back delay | 2 000 |  |  | 2 000 |  |
| Acknowledgment transmission | application data | 0 |  |  | 0 |  |
|  | protocol overhead | 29 |  |  | 0 |  |
|  | segmention | 1x 29 |  |  | 1x 0 |  |
|  | LLC-PDU # and size | 1x 29 |  |  | 1x 0 |  |
|  | MAC-PDU # and size | 1x 43 |  |  | 1x 14 |  |
|  | PHY-PDU # and duration | best case 1x 573 |  | worst case (*) 4x 573 | best case 1x 187 | worst case (*) 4x 187 |
|  | Latency | 10 793 | 12 527 |  | 5 751 | 6 327 |

(*) worst case includes 5ms gap between radio frames in DL

Figure 1: UL and DL transmission of exception reports

without protocol overhead (a) and with it (b)

# Conclusion

This contribution deals with latency evaluation of MAR exception reports. The C-UNB radio access technology delivers the MAR exception reports in less than 10s when the transmission over the air interface is optimized, that is to say, when protocol overhead is not transmitted over the air, but managed by the MS avatars located in the C-UNB server.

When compressed protocol header is transmitted over the air along with the application data, the latency for MAR exception reports is 10.9 to 12.5 seconds.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia.

[2]	GPC150364, "Cooperative Ultra Narrow Band - General Description", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[3] 	GPC150309, "C-UNB technology for Cellular IoT - Downlink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[4] 	GPC150310, "C-UNB technology for Cellular IoT - Uplink Physical Layer Design", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[5] 	GPC150311, "C-UNB Link Layer Aspects", SIGFOX, GERAN ad'hoc meeting #3, 29June-02July 2015, Kista Sweden

[6]	TR 45.820, "Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things", revision 1.3.1, June 2015

[7]	GPC150368, "C-UNB architecture", 3GPP GERAN meeting#66, 25-29 May 2015, Vilnius, Lithuania