pCR for text inclusion in TR 45.820 :

C-UNB Physical Layer Design

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.3.2 and 7.3.3 of the Technical Report 45.820 revision 1.3.1.

The proposed text details the implementation of downlink and uplink transmission of C-UNB radio access technology.

Proposed text for the TR

Start of Changes

### Downlink physical design

#### Bit rate

The downlink bit rate is set to 600bps. This value is low enough to have UNB transmissions. It is higher that the UL transmission rate to get a reduced transmission time of DL radio packets, hence reducing the reception time in devices.

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power

The sensitivity of receivers in devices is slightly lower than receivers in base stations, because the reception bandwidth can't be as narrow as what we have in base stations. The bit rate is higher. Given the maximum value of +164dB for the DL MCL, it is proposed to set the maximum transmission power on each ad'hoc micro-channel to +34dBm.

#### Micro-channel power management

In practice, actual transmission power in downlink is adjusted according to the strength of the received signal upstream. This very simple power management can be implemented because every DL transmission is triggered by a UL packet.

If we consider the regular distribution of devices in a cell, the mean distance to the base station is 2/3 of the max distance. This mean value improves the link budget by about 5 dB.

Figure 7.3.2-1: Relationship between frequencies of corresponding UL and DL micro-channels

#### Maximum PSD in base station

The power spectral density (PSD) of a GSM/GPRS base station is an important limiting factor for C-UNB, because the total available power is split among all the DL micro-channels, which are transmitted simultaneously. The worst case is where all micro-channel transmissions are aimed at devices close to the cell borders: each micro channel is set to its maximum transmit power, i.e. +34dBm. Given the PSD of +43dBm in GSM/GPRS base stations, the total number of micro-channels, which can be transmitted simultaneously, is 8.

If we consider the mean distance of devices, the mean transmit power is +29dBm. Hence, the mean number of DL micro-channels is about 25.

The values (i.e. 8 max and 25 mean) are reasonable for carrying the traffic in downstream.

#### Center frequency in DL

The center frequency of each DL micro-channel is chosen according to the actual frequencies used by the device during uplink transmission. The precise center frequency of a given micro-channel is not known a priori. It is calculated for each downlink message (see figure 7.3.2-1).

This feature helps reducing the complexity and cost of devices, because no automatic frequency control is needed during reception in devices: the AFC is performed by the base station itself.

#### Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 1200 Hz. Pulse shaping filter is given in figure 7.3.2-2, whereas figure 7.3.2-3 gives the corresponding spectrum mask.

Figure 7.3.2-2: Pulse shaping filter in DL

Figure 7.3.2-3: Spectrum mask of DL in C-UNB

#### Error correction code

Each MAC-PDU is protected by an Error Correction Code (ECC) of 32 bits. Given the max length of a MAC-PDU of 384 bits, this ECC adds a 1dB gain to the MCL.

### Uplink physical design

#### Bit rate

The uplink bit rate is set to 250bps. This value is compliant with the ultra narrow band feature of C-UNB; it allows meeting the 160bps requirement at SNDCP interface.

#### Modulation

The proposed modulation for C-UNB downlink transmission is D-BPSK. It is chosen because of its robustness against noise and interferers. Differential modulation is implemented with a  phase shift for each bit 0 transmitted. Bit 1 keeps the phase constant.

#### Transmission power

The transmission power is a key parameter for IoT devices because it sets most part or the power consumption of devices. Considering the very good selectivity and dynamic of C-UNB receivers in base stations, the Tx power in uplink is constant and set at +24dBm.

#### Uplink frequency

In Cellular IoT, the objective is to re-use the legacy 200kHz blocks for IoT traffic. In usual cellular system, the center frequency of a channel is defined precisely and thus it requires costly TCXO or complex frequency correction features.

In the case of C-UNB technology, the legacy frequency block is used to carry uplink and downlink "ad'hoc micro-channels" that are ultra narrow band. The center frequency of an ad'hoc micro-channel is not precisely defined, provided it is within the 200kHz block allocated to CIoT. On the contrary, a device sets pseudo-randomly the center frequency for each UL PHY-PDU. The precise frequency detection and correction is done by the Software Defined Radio base station in uplink.

#### Pulse shaping and spectrum mask

Amplitude of the transmitted signal is reduced during phase transition in order to keep the signal spectrum within tight boundaries, i.e. 500Hz. Pulse shaping filter is given in figure 7.3.3-1.

The obtained spectrum and spectrum mask are illustrated in figure 7.3.3-2.

Figure 7.3.3-1: Pulse shaping filter in UL

Figure 7.3.3-2: Actual spectrum and spectrum mask for UL

#### Error correction code (ECC)

A 16-bit Reed-Solomon ECC protects each MAC-PDU. Given the max length of a MAC-PDU of 388 bits, this ECC adds a 1dB gain to the MCL with a limited complexity in the device. The most complex part of the Reed-Solomon code (i.e. the decoding part) is performed in the base station.

End of Changes