pCR for text inclusion in TR 45.820 :

C-UNB Link layer aspects

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.3.4 of the Technical Report 45.820 revision 1.3.1.

The proposed text details the link layer aspects of C-UNB radio access technology.

Proposed text for the TR

Start of Changes

### Link layer aspects

#### Downstream link layer aspects

##### Downlink MAC-PDU format

In C-UNB, downlink MAC-PDUs are optimized considering the fact that they are transmitted in reponse to up link transmission. Figure 7.3.4-1 illustrates the MAC-PDU format in C-UNB downlink. The various fields are detailed in the following subclauses.

Figure 7.3.4-1: Format of a MAC-PDU frame in C-UNB downlink transmission

##### Header

Downlink header is 56 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a payload length,

acknowledgement bits.

##### Payload

The payload of a MAC-PDU is 0-34 byte length. It carries application data and, when necessary, LLC header for segmentation/re-assembly.

##### Authentication

The authentication field is a hash, computed with an adaptation of the common C-MAC algorithm. It uses the 128-bit secrete key of the device plus:

the device identifier received in corresponding uplink packet,

the sequence number in the corresponding uplink packet,

the payload field.

This field is used to sign a downlink packet and also to identify the destination device implicitly: the probability to have many devices listening on a given frequency at a given time is very limited; hence a 16-bit hash is enough to authenticate the destination.

##### FCS

The frame check sequence in C-UNB downlink consists in a CRC-8. It checks the MAC-PDU after error correction, before further decoding.

##### ECC

This field is a 32-bit Error-Correction Code. It uses BCH code applied to the four first fields: header (except preamble), Payload, Authentication and FCS. Given the length of DL MAC-PDU, the ECC is expected to add 1 dB of processing gain in the DL MCL.

##### Segmentation & reassembly

The present TR defines the maximum application packet to 2 000 bytes, with addition of a protocol header of 65 bytes (extended) or 29 bytes (compressed). Given the small size of C-UNB radio packets, it is proposed to have a segmentation/re-assembly mechanism with 32-bytes of application data and 2 bytes for segment numbering (see figure 7.3.4-2).

Figure 7.4.3-2: segmentation in C-UNB downlink

This implementation gives a maximum size of the application data of 4 096 bytes, which is enough for the data packets expected in CIoT.

##### Downlink mean Throughput

At the SNDCP interface, the transmission of long application blocks requires the maximum throughput. The average throughput of C-UNB radio access technology at SNDCP interface is given by the formula (1):

Given the data rate of 600bps and the sizes of MAC and LLC headers, the DL throughput is about 400bps, which is above the 160bps required by Cellular IoT.

#### Upstream link layer aspects

##### Uplink MAC-PDU format

Figure 7.4.3-3 illustrates the MAC-PDU format in C-UNB uplink. The various fields are detailed in the following subclauses.

Figure 7.4.3-3: Format of a MAC-PDU frame in C-UNB uplink transmission

##### Header

Uplink header is 32 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a frame length,

acknowledgement flags,

a repetition counter (value 1, 2 or 3) tagging the actual repetition number of MAC-PDUs.

Note: the number of repetition of a given MAC-PDU depends on the expected success rate. Two repetitions are for normal MAR, whereas three repetitions are for exception reports.

##### Sequence counter

The uplink sequence counter is 12 bits. It is incremented for each new LLC-PDU sent by a device. The sequence counter is used for authentication purpose (see subclause on authentication).

##### Identifier

This field is 40 bits long. It contains the device unique identifier set during personalization of the device. Each identifier is unique in a C-UNB system.

##### Payload

The payload is 0-32 bytes. Payload length is defined by a 5-bit length field in the MAC-PDU header. Empty payload corresponds to ACK packet; they have a specific frame type.

##### Authentication

The authentication field is a 16 bits hash, used to authenticate the MAC-PDUs received by the networks. The 16 bits of the authentication field are generated using a proprietary implementation of the well-know C-MAC algorithm. It uses a 128 secrete key. Authentication check is performed in the core network and not in the base stations to avoid secrete key transmission on the back-haul network.

##### FCS

The frame check sequence in C-UNB uplink consists in a CRC-8. In reception, it is used to check MAC-PDU consistency before transmission to the core network.

##### ECC

The last field of an uplink MAC-PDU is an Error-Correcting Code of 16 bits. It used Reed-Solomon algorithm and is evaluated over the whole MAC-PDU except the preambule.

##### Segmentation & reassembly

The CIoT requirements define Mobile Autonomous Reports with 20-200 byte length. Given the fact that only small radio packets are affordable for the C-UNB radio access technology, it is proposed to have a segmentation/re-assembly mechanism based on 31 blocks of data with a LLC header of 8 bits: 4 bits for segment numbering and 4 bits for total number of segments (see figure 7.3.4-4). These values set the maximum block size of application data to 496 bytes.

Figure 7.3.4-4: LLC header in C-UNB uplink

##### Uplink mean throughput

Given the data rate of 250bps in UL and the sizes of the MAC and LLC headers, the formula (1) gives a mean throughput of 160bps in uplink.

End of Changes