pCR for text inclusion in TR 45.820 :

C-UNB Concept Evaluation

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.3.6 of the Technical Report 45.820 revision 1.3.1.

The proposed text for section 7.3.6 evaluates the C-UNB technology with respect to the TR 45.829 requirements.

Proposed text for the TR

Start of Changes

### Concept evaluation

#### Downlink Maximum Coupling loss

Table 7.3.6-1 gives the maximum coupling loss (MCL) of the downlink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 here after. Figures in table 7.3.6-1 are common, except the SINR, which is explained here after.

Subclause 5.1 states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message : when a MS moves at less than 30 km/h, the Doppler shift is about 25Hz. This small shift is not seen by the MS, which band filter is larger than the signal (i.e. 1kHz vs 600Hz).

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are relatively small when the device speed is lower than 30km/h: they don't move the received signal out of the 1kHz band of the MS receiver

fast fading, which is critical in GSM, is not an issue for C-UNB, because the symbol duration is 1,66 ms in C-UNB DL transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception or multiple transmissions. Considering the duration of DL radio packets (i.e. about 0,2-0,6 s), we propose to have four blind retransmissions of each DL radio packet.

When putting all this together, we propose to set the SINR value to 9dB. Table 7.3.6_1 gives the Maximum Coupling Loss of the C-UNB downlink transmission.

Table 7.3.6-1: Downlink Maximum Coupling Loss in C-UNB

| Logical channel name | C-UNB downlink |
| --- | --- |
| Data rate (bps) | 600 |
| Transmitter |  |
| (1) Tx power (dBm) | +34 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 6 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 1000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -138 |
| (7) Required SINR (dB) | 9 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

#### Uplink Maximum Coupling Loss

Table 7.3.6-2 gives the maximum coupling loss (MCL) of the uplink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1. Figures in table 7.3.6-2 are quite understandable, except for the SINR one that is explained here after.

Subclause 5.1 here above states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message (about 25Hz at 30km/h): this frequency shift is totally managed by the base station because there is no channelization in C-UNB radio interface. Center frequencies are randomly selected by MSs, therefore a Doppler shift is not seen, in itself, by the base stations

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are detected and corrected by the signal processing in C-UNB base stations

fast fading, which is critical in GSM, is not an issue for C-UNB, because of the 4ms symbol duration in C-UNB transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception. As C-UNB implements cooperative reception and large overlap of base station coverage, at least three different base stations receive each radio packet. Slow fading is, therefore, less critical than in GSM/GPRS system.

When putting all this together, we propose to set the SINR value to 8dB.

Table 7.3.6-2: Uplink Maximum Coupling Loss in C-UNB

| Logical channel name | C-UNB uplink |
| --- | --- |
| Data rate (bps) | 250 |
| Transmitter |  |
| (1) Tx power (dBm) | +24 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 3 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 250 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -147 |
| (7) Required SINR (dB) | 8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -139 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

#### Battery Life evaluation

The battery life evaluation for C-UNB MS is based on methodology detailed in the present TR, but with two simplifying assumptions due to the C-UNB radio access technology:

MAR are supposed to be transmitted with two repetitions at physical layer,

no acknowledgement is transmitted in DL.

The second assumption is valid as long as the collision rate is less than 10%, which is the case in all performance evaluations because of network design.

The battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 7.3.6-3. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 7.3.6-3: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU |
| --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 |
| 200 | 7 | 29 | 30 | 46,5 | 14 |

Table 7.3.6-4 gives key parameters for energy consumption analysis, whereas figure 7.3.6-1 gives energy consumption model for C-UNB devices. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission.

Table 7.3.6-4: Key input parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power when Idle but not in PSS (mW) | (5) Battery power in Power Save State (PSS) (mW) | (6) Idle time from PSS to Tx mode (ms) | (7) Tx time (ms) | (8) Idle time between two Tx (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 | 50 | MAC-PDU size/bit rate | 25 |

Figure 7.3.6-1: Energy consumption model for C-UNB devices

Table 7.3.6-5 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage levels.

Table 7.3.6-5: Battery life (in years)

#### Capacity evaluation

For network capacity evaluation, the present TR sets assumptions on traffic model and MS density. In the case of C-UNB, capacity evaluation is based on analytical models of the C-UNB radio access technology. In order to simplify the analytical model, some work has been done on the assumptions stated in the TR. It is detailed in the two following sub-sections.

##### Traffic model assumption

In annex E, the traffic model per MS is defined with a first parameter: the number of MAR per day. Four types of MSs are defined with for values for the inter-arrival time of MAR.

As there is a large number of MS in a given cell, we propose to evaluate the network capacity with one "mean type" of MS, implementing a mean value of inter-arrival time (i.e. an average number of MAR per day). Table 7.3.6-6 gives the value used in the C-UNB analytical model.

Table 7.3.6-6: Average # of MAR per day used in C-UNB capacity analysis

| inter-arrival time | ratio | number of MAR per day | average number of MAR per day |
| --- | --- | --- | --- |
| 1 day | 40% | 1 | 11,2 |
| 2 hours | 40% | 2 |  |
| 1 hour | 15% | 24 |  |
| 30mn | 5% | 48 |  |

In annex E, the second parameter for defining the traffic model is the size of application data carried by MARs. This size is defined as a Pareto distribution with min value of 20 bytes, max value of 200 bytes and a coefficient of 2.5. This formula is quite complex to evaluate in an analytical model, such as the one we use in C-UNB. Therefore we propose to split the work in four parts and made a weighted average at the end:

use four different values of MAR sizes,

evaluate the network capacity with these four MAR sizes

make a weighted average of rough capacity results with the Pareto ratios.

As there is a full decade between min and max values of the application data size, we propose to define the MAR size with a log scale. The four MAR sizes and corresponding Pareto's ratios are given in table 7.3.6-7.

Table 7.3.6-7: MAR sizes and corresponding Pareto ratios
used in network capacity evaluation

| index | MAR size (in bytes) | Pareto ratio |
| --- | --- | --- |
| 1 | 20 | 0,833 |
| 2 | 40 | 0,147 |
| 5 | 100 | 0,015 |
| 10 | 200 | 0,005 |

##### Coverage assumption

Annex E defines hexagonal tiles for the cellular coverage and the cell site distance of tri-sectorized BTS (see figure 7.3.6-2). In common cellular system, a MS is attached to a given sector; hence the network capacity is evaluated per sector that is hexagon shape.

In the case of C-UNB, the cooperative reception feature requires an MS to be seen by several base stations. In our capacity evaluation, we have enlarged the coverage of a given sector up to the next BTS. Each sector has a diamond shape (see figure 7.3.6-3) giving three overlaps per sector.

Figure 7.3.6-2: Cell site sector area definition

Figure 7.3.6-3: C-UNB sector definition (a: theoretical; b: used in analytical model)

##### Other assumptions

The present TR defines other assumptions for the capacity evaluation, such as the ratio of MS transmitting MAR and receiving network command, the protocol overheads and the system level assumptions (annex D).

The specific features of C-UNB radio access technology allow us to simplify some of the assumptions without going against the objectives of the study. The extra assumptions made in the present contributions are:

network capacity is mainly limited by uplink and not by downlink in C-UNB

protocol overheads (65 bytes or 29 bytes (compressed) can be omitted over the C-UNB RAT because they can be generated by MS avatars hosted in C-UNB server. Nevertheless, capacity evaluation are given with and without headers for the sake of comparison,

building penetration model can be omitted because MSs transmit always at full power and base station can cope with highly dynamic signals.

As MAC-PDUs have limited payload size in C-UNB, table 7.3.6-8 details the number of segments and MAC-PDU size for three assumptions on the header size: 65, 29 and 0 bytes.

Table 7.3.6-8: segmentation with various headers (all size in bytes)

| Assumption | Data size | Application packet size | Segmentation | Number and size of MAC-PDUs |
| --- | --- | --- | --- | --- |
| full header of 65 bytes | 20 | 85 | 3x29 | 3x 46,5 |
|  | 40 | 105 | 4x27 | 4x 44,5 |
|  | 100 | 165 | 6x28 | 6x 45,5 |
|  | 200 | 265 | 9x30 | 9x 47,5 |
| compressed header of 29 bytes | 20 | 49 | 2x25 | 2x 42,5 |
|  | 40 | 69 | 3x23 | 3x 40,5 |
|  | 100 | 129 | 5x26 | 5x 43,5 |
|  | 200 | 229 | 8x29 | 8x 46,5 |
| header not transmitted over the air | 20 | 20 | 1x20 | 1x 37,5 |
|  | 40 | 40 | 2x20 | 2x 37,5 |
|  | 100 | 100 | 4x25 | 4x 42,5 |
|  | 200 | 200 | 7x29 | 8x 46,5 |

##### Capacity results

Table 7.3.6-9 gives the maximum capacity of a C-UNB network defined as a number of MSs per sector. These figures are computed with a repetition rate (i.e. blind repetition) of 2 per MAC-PDU and a retransmission ratio of 10%.

Table 7.3.6-9: C-UNB capacity

| Header | Max number of MS per sector |
| --- | --- |
| full size, 65 bytes | 21 460 |
| compressed, 29 bytes | 34 640 |
| not over the air, 0 byte | 75 900 |

#### Latency evaluation

##### Assumption on header for latency evaluation

Annex E.2.1 defines a MAR exception report payload of 20 bytes; the protocol overhead is 29 bytes (compressed) or 65 bytes (full overhead).

In C-UNB, the MAC-PDUs have relatively small payload size, resulting in segmentation when long application data have to be transmitted. In order to reduce the among of data carried over the air interface, the preferred implementation of the C-UNB protocol is to skip the transmission of the protocol header and let it manage by the MS-avatar hosted in the C-UNB server (see C-UNB general description). Nevertheless, this contribution evaluates latency with and without protocol header for the sake of comparison.

##### Blind repetitions

The C-UNB radio access technology assumes a number of blind repetitions for each MAC-PDU. Blind repetitions helps increasing the probability of good reception but reduces the overall network capacity. MAR exception reports require a high quality of service, therefore the present contribution considers that each MAC-PDU is transmitted with three PHY-PDUs (i.e. three repetition of the same radio packet) and then acknowledged by the base station.

##### Latency results

Table 7.3.6-10 details latency calculation for a MAR exception report. Figure 7.3.6-4 illustrates the UL and DL transmission without and with protocol overhead.

Table 7.3.6-10: latency evaluation (size in bytes and time in ms)

| Step | Item | transmission with 29 protocol overhead |  |  | transmission with protocol overhead managed by MS-Avatar |  |
| --- | --- | --- | --- | --- | --- | --- |
| MAR exception report transmission | application data | 20 |  |  | 20 |  |
|  | protocol overhead | 29 |  |  | 0 |  |
|  | segmentation | 2x 25 |  |  | 1x 20 |  |
|  | LLC-PDU # and size | 2x 26 |  |  | 1x 20 |  |
|  | MAC-PDU # and size | 2x 42,5 |  |  | 1x 36,5 |  |
|  | PHY-PDU # and duration | 6x 1 360 |  |  | 3x 1 168 |  |
|  | radio wake up time | 50 |  |  | 50 |  |
|  | gap between 2 radio packet | 5 |  |  | 5 |  |
| Loop-back | network loop-back delay | 2 000 |  |  | 2 000 |  |
| Acknowledgment transmission | application data | 0 |  |  | 0 |  |
|  | protocol overhead | 29 |  |  | 0 |  |
|  | segmention | 1x 29 |  |  | 1x 0 |  |
|  | LLC-PDU # and size | 1x 29 |  |  | 1x 0 |  |
|  | MAC-PDU # and size | 1x 43 |  |  | 1x 14 |  |
|  | PHY-PDU # and duration | best case 1x 573 |  | worst case (*) 4x 573 | best case 1x 187 | worst case (*) 4x 187 |
|  | Latency | 10 793 | 12 527 |  | 5 751 | 6 327 |

(*) worst case includes also 5ms gap between radio frames in DL

Figure 7.3.6-4: UL and DL transmission of exception reports

without protocol overhead (a) and with it (b)

End of Changes