DL System Level Simulation Results of GERAN CIoT Solutions: EC-GSM

1.  Introduction

A study item named “Cellular IoT” was approved in GERAN#62 for evaluating how to support low throughput and low complexity machine type communications [1].  Several proposals have been discussed under the categories of an evolved low-complexity EGPRS (EC-GSM) and clean slate solutions (NB M2M and NB OFDMA).  The evaluation methodology including the system assumptions, parameters and traffic models were captured in the draft TR 45.820 [2].

In this document, we present some downlink system level simulation results for EC-GSM solution for GERAN CIoT systems. The full-buffer traffic model is assumed to verify whether the requirement of 160 bps throughput at the equivalent of the SNDCP layer in [2] can be satisfied.  Furthermore, the CIoT traffic model specified in [2] was employed in the simulations and system capacity and latency results were obtained.

2.  Simulation Setup and Parameters

The major system level simulation assumptions and parameters are listed in Table 1.

Table 1: Simulation parameters and assumptions.

| Parameter | Assumption |
| --- | --- |
| Cellular Layout | Hexagonal grid, 19 cell sites, 3 sectors per site |
| Frequency band | 900 MHz |
| Inter site distance | 1732 m |
| Number of carriers per cell (1 sector) | 1 |
| System bandwidth | 600 kHz |
| Frequency Hopping | Off |
| Modulation coding schemes | MCS 1-4 |
| Doppler frequency | 1 Hz |
| User distribution | Devices dropped uniformly in each sector |
| BS transmit power per 200 KHz (at the antenna connector) | 43 dBm |
| Pathloss model | L=I + 37.6log10(R), R in kilometers I=120.9 dB for the 900 MHz band |
| Shadowing standard deviation | 8 dB |
| Correlation distance of Shadowing | 110 m |
| Shadowing correlation | 0.5 Between cell sites/ 1 Between sectors of the same cell site |
| Antenna pattern (horizontal) | See table 5-7, 3GPP TR 45.914, 65° H-plane |
| BS Antenna gain | 18 dBi |
| MS Antenna gain | -4 dBi |
| BS cable loss | 3 dB |
| Building Penetration Loss | Based on TR 45.820 Scenario 1 with correlation coefficient = 0.5 |
| Frequency reuse | 1/3 |
| Link to system mapping | Proposed in [3] |
| Noise figure | 5 dB |

In addition to the CIoT traffic models specified in [2], full-buffer traffic and proportional fair (PF) scheduling are considered in the simulations. The choice of full-buffer traffic model and uniform distribution of larger number (20, 30, 40) of devices per sector, although not a part of the agreed evaluation methodology, is primarily motivated by the aim of facilitating an easier analysis and better understanding of the performance of the EC-GSM design at the system level considering the minimum MAC throughput requirement for a challenging scenario w.r.t. system loading (when compared to practical traffic loading from CIoT traffic).  This assumes 160 bps at the equivalent of the SNDCP layer according to the requirements of [2] and approximately an additional 40 bps overhead for the physical layer/MAC overhead. The link-to-system mapping procedure proposed in [3] has been employed for the physical layer abstraction. Perfect time and frequency synchronization and channel estimation are assumed in this study. Note that, in this work, we considered a frequency reuse factor of 1/3 which can be considered as the worst case scenario when compared to the nominal GPRS scenario where we have a reuse of 4/12, however in practical deployments, it can differ based on the operator’s strategy. When comparing the two reuse cases, in the 1/3 case, we have 18 interferers given a 57-cell layout, whereas in the 4/12 case we might have much less interferers for the same layout, i.e., can be as low as 6 interferers only.

3. Simulation Setup

In this proposal, we considered an EC-GSM system with blind repetition and scheduling penalty. The number of blind repetitions considered were 2, 4, 8, and 16. Each of these repetitions were decoded assuming coherent energy combining.

The PF scheduler considers the channel quality of the devices and determines the number of repetitions based on predefined channel quality thresholds.

If the blind repetitions are scheduled with higher or equal priority, the scheduling opportunities of the other devices in normal coverage can be expected to be adversely affected, which potentially leads to  a reduction in the overall system capacity as well. To address this concern, we applied a scheduling penalty proportional to the number of repetitions used to transmit to the devices in deep coverage holes. Specifically, the PF metric of the devices with blind repetitions is penalized by a factor of log2(1+N), where N is the number of repetitions.

4. System Level Simulations results

## 4.1	Building penetration loss

Scenario 1 of the building penetration loss specified in [2] with 0.5 correlation has been considered.  The penetration loss values were generated for the 5000 uniformly distributed devices in the cell area to obtain sufficient statistics for penetration loss distribution. However, for better illustration, in Figure 1 we present the CDFs of the two BPL scenarios of [2] with different correlation coefficients. From this figure, it can be seen that the penetration loss varies between 5 and 35 dB with a median value of 19.8 dB for Scenario 1 with 0.5 correlation.

Figure 1.  CDF of the building penetration loss for the scenarios given in [2].

## 4.2	Results with full buffer traffic model

### 4.2.1	Geometry

The geometry CDFs of the downlink 19 cell cellular system with the parameter assumptions in Section 2 are depicted in Figure 2.  The geometry is defined as the long term SINR including pathloss, shadowing, antenna pattern and penetration loss. Frequency reuse cases 1 and 3 are considered here. For the reuse 3 case the system bandwidth (600 kHz) is used in each 3-sector site. I.e. each cell (1 sector) is assigned a dedicated 200 kHz band (no FH used).  For example, in the 19-cell (57 sectors) deployment layout, sectors 1, 4, 7, … use the same frequency. For the reuse 1 case, there is only one 200 kHz band available in the whole system and all the sectors are assigned to the same band.

Figure 2.  The Geometry CDFs for frequency reuse 1 and 3.

Note that EC-GSM systems are not expected to be deployed at a reuse factor 1. Hence, for example, if we consider an EC-GSM system operating at reuse factor of 3, from the reuse 3 CDF, if the normal coverage is assumed as 10 dB geometry, 60% of the devices can be seen to be in normal coverage.

### 4.2.2	Throughput

#### 4.2.2.1		With PF scheduling

The downlink MAC throughput CDFs for the full buffer traffic, are presented in Figure 3.  The scheduling mechanism with penalty for blind repetition discussed in Section 3 was simulated.  We assume 200 bps as the target MAC throughput for the GERAN CIoT devices. The results indicate that around 92% of the devices may be able to achieve the 200 bps throughput when the system concurrently supports 20, 30 and 40 UEs. The percentage of devices using blind repetition is about 2 % for all cases. The average sector throughput for different scenarios is presented in Table 2. As expected, the average sector throughput slightly decreases with the increasing number of devices. Finally, it is worth mentioning here that the CIoT applications in practice are expected to have very infrequent and sparse transmissions. Thus, the full-buffer traffic model assumption in the simulations helps in understanding the worst case performance of EC-GSM deployment.

Figure 3.  The device throughput CDFs of the EC-GSM system with scheduler penalty.

Table 2.  Average DL MAC throughput for the EC-GSM systems with full buffer traffic.

|  | 20 UEs | 30 UEs | 40 UEs |
| --- | --- | --- | --- |
| System Throughput | 116.2 Kbps | 114.1 Kbps | 112 Kbps |
| Percentage of UEs using blind repetitions | 1.7 | 2.1 | 2.1 |

#### 4.2.2.2		Without Penalization in PF scheduling

To further evaluate the performance, the downlink MAC throughput CDFs for the full buffer traffic, are presented in Figure 4, but without the penalty discussed in Section 3. We assume 200 bps as the target MAC throughput for the GERAN CIoT devices. The results indicate that around 98% of the devices may be able to achieve the 200 bps throughput at the equivalent of the SNDCP layer when the system concurrently supports 20, 30 and 40 UEs. However, unlike the previous case wherein penalization is applied, the percentage of devices using blind repetition is between 13.3% and 14.5%. The average sector throughput for different scenarios is presented in Table 3. As expected, the average sector throughput significantly decreased when compared to the scenario wherein penalization is applied.

Figure 4.  The device throughput CDFs of the EC-GSM system without penalization.

Table 3.  Average DL MAC throughput for the EC-GSM systems with full buffer traffic, but without penalization.

|  | 20 UEs | 30 UEs | 40 UEs |
| --- | --- | --- | --- |
| System Throughput | 82.9Kbps | 81.7 Kbps | 77.4 Kbps |
| Percentage of UEs using blind repetitions | 13.5 | 13.3 | 14.4 |

## 4.2	Results with traffic models according to evaluation framework

Now we evaluate the downlink performance of the EC-GSM with respect to the traffic model described in [2] for system capacity evaluation. In this traffic model it is assumed that 20% of the reports will be generated from the Network Command traffic model whereas the remaining 80% will be generated from the MAR periodic reporting traffic model [2].  Hence, in this traffic model, the size of 80% of the transmitted reports follow a Pareto distribution with minimum and maximum payload sizes of 20 and 200 bytes, respectively, whereas the size of the remaining 20% is fixed to 20 bytes. We consider a maximum of 50 simultaneous UEs in the system. The downlink offered traffic versus the carried traffic and the packet delay CDFs of this traffic model are presented in Figure 3 and 4, respectively. The scheduling mechanism discussed in Section 3 was employed here as well, but without the penalization of Section 3. We see a linear relationship between offered traffic and carried traffic until 900K reports/200KHz/Hour. It indicates that all the offered traffic can be transmitted without increasing the buffer size. Furthermore, up to 900K reports/200KHz/Hour, more than 99% of the reports are delivered in less than 4 seconds (thus meeting the maximum 10 seconds delay requirement).

Figure 5.  The offered traffic versus carried traffic in the traffic model simulation of [2].

Figure 6.  Report delay CDFs in traffic model simulations.

5. Summary

In this contribution, we have presented our downlink system simulation results for the EC-GSM systems with full-buffer traffic and realistic CIoT traffic model. Our full buffer simulation results indicate that around 92% of the devices would be able to receive at least 200 bps MAC throughput for the case wherein penalization is applied whereas 98% of the devices were able to achieve a throughput of at least 200 bps without penalization, i.e., the 160 bps throughput at the equivalent of the SNDCP layer.  We supported blind repetitions and scheduling enhancements in the study however, the percentage of devices employed blind repetitions in the downlink is around 2% with penalization and around 14% without penalization. In the realistic traffic model simulations, i.e., the 20% Network Command and 80% MAR periodic reporting traffic model, up to 900K reports/200KHz/Hour, more than 99% of the reports were delivered within 10 sec packet delay. Thus, EC-GSM systems satisfy the requirements put forward in the TR 45.820 with a huge margin in the downlink.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TR 45.820 v1.2.1, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

GP-15xxx, “GERAN CIoT Link-to-System Mapping Methodology,” Intel Corporation, GERAN #66.