3GPP TSG GERAN Ad Hoc#3 on FC_IoT_LC	Tdoc GPC150508

Kista, Sweden	Agenda items 1.4.2.2, 2.4.1.2

29th June - 2nd July, 2015

Source: Nokia Networks

N-GSM: Design and Performance for N-AGCH

# Introduction

This contribution depicts the N-AGCH design and its performance for the N-GSM concept proposal included into TR 45.820 [1].

Section 2 describes the message content of immediate assignment commands for serving CIoT terminals. Based on the message size, the channel structure is defined for the N-AGCH blocks. Section 3 details the mapping of the N-AGCH blocks to the N-CCCH multiframe structure including repetitions of N-AGCH messages.

This document contains a revision of [2] with updates highlighted in blue color.

# DESIGN FOR N-AGCH

## Message content

N-AGCH will carry following messages

- N-Fixed Uplink Assignment

- N-Downlink Assignment

- N-Immediate Assignment Reject.

The largest message among these will be N-Fixed Uplink Assignment message. Below the content of this message is analysed to derive the maximum message size for N-AGCH.

Table 1 below lists the parameters of N-Fixed Uplink Assignment message carried in N-AGCH.

| Parameter | Size | Comment |
| --- | --- | --- |
| Message Type | 6 bit |  |
| CIOT-REQ-REF | 11 bit | corresponds to received content on N-RACH. |
| Timing Advance | 6 bit |  |
| TSC | 3 bit |  |
| SI_FP_INDEX | 3 bit |  |
| TTI_MODE | 2 bit | 1 bit for UL, 1 bit for DL. Each bit indicates 80ms or 240ms TTI. |
| TSN | 3 bit | TS number |
| UPLINK_TFI | 5 bit |  |
| NUM_RLC_BLOCKS | 4 bit |  |
| NUM_FRMS_GAP | 2 bit | gap length between assigned RLC blocks 0: 4 TDMA frames 1: 8 TDMA frames 2: 12 TDMA frames 3: 16 TDMA frames |
| UL_ALLOCATION_BITMAP | 16 bit | Each bit indicates the allocation for a specific RLC block. - if 1: the allocation starts immediately from the end of previous RLC block. - if 0: the allocation starts after the defined gap of fixed frames. |
| NUM_ADDITIONAL_ALLOCATIONS | 2 bit | Number of times the allocation bitmap repeats. This field is applicable for normal TTI only to allow more than one transmissions from single allocation. |
| TRANSMISSION_GAP | 2 bit | Gap between allocations to allow reception of PUAN |
| Total Message Size | 65 bit |  |

Table 1: Information elements of the N-Fixed Uplink Assignment message using N-AGCH.

The fixed uplink allocation message provides resource allocation for transmission of uplink RLC blocks. The allocation bitmap included in this message indicates the resource allocation for each of the RLC blocks and its position with respect to the previous RLC blocks. The bitmap value of 1 in position N represents the allocation for of continuous uplink TDMA frames for the Nth RLC Block. For 80 ms TTI this value is 16 TDMA frames and for 240 ms TTI, this value is 48 TDMA frames.

In case of 240 ms TTI, fixed uplink allocation provides resource for concatenated transmission applied for each RLC block. After transmission of all RLC blocks using the allocated uplink TDMA frames, the MS starts to wait for DL N-PACCH to receive the PUAN message along with the additional resource allocation bitmap for retransmitting RLC blocks which have not yet been successfully received.

In case of 80 ms TTI, fixed uplink allocation provides additional resource for up to 4 more transmissions through ADDITIONAL_ALLOCATIONS. This additional allocation follows the same allocation bitmap.

The detailed procedure for uplink data transfer using above fixed uplink allocation will be dealt with as part of Link layer aspects. It is noted that further optimisation of the information conveyed over N-AGCH is being investigated.

## N-AGCH Channel Design

The block structure, CRC construction and convolutional coding scheme for the above payload size are mentioned in Table 2.

| N-AGCH coding parameters | Size [bits] |
| --- | --- |

| Payload bits | 65 |
| --- | --- |
| CRC | 18 |
| Tail bits | 6 |
| Input bits for channel coding | 89 |
| Convolutional coding | 1/3 |
| Constraint Length | 7 |
| Puncturing | 43 |
| Interleaving | FFS |

Table 2:  Channel structure and coding details for N-AGCH.

Channel encoded data bits of 224 bits are mapped to 8 N-GSM bursts carrying 14 GMSK symbol sequences and hence 28 bits each. Thus the N-AGCH has a block size of 8 bursts (TTI = 40 ms).

# mapping of n-agch in n-ccch multiframe

N-AGCH blocks can be mapped next to N-BCCH blocks in the N-CCCH frame structure in TN 1. In case more N-AGCH blocks are required it can be extended to next 8 TDMA frames but will reduce the number of N-PCH blocks within the N-CCCH multiframe. N-AGCH is repeated in the same locations up to 3*51-multiframes.

The mapping is depicted in Figure 1 below.

Figure 1:  Mapping of N-AGCH blocks (different or repetitions)                  in subsequent  N-CCCH multiframes (51-multiframes)                   following N-BCCH blocks.

## Repetition of N-AGCH

To ensure correct reception of N-AGCH messages more than one transmission of the N-AGCH immediate assignment command may be required. The number of blind repetitions required for N-AGCH is decided by the network based on the downlink Radio Quality indicator included in the N-RACH access request message. Based on this indicator the network decides about the required number of blind repetitions for N-AGCH messages and also the TTI size for downlink N-PDTCH. The network may also use detection of the N-RACH burst positions used by the MS to derive the required number of blind repetitions as well as the received signal strength / quality for uplink transmissions of that MS. Mapping of the Radio Quality indicator to the number of N-AGCH repetitions is given in Table 3 below.

| Downlink Radio Quality indicator | Required number of Transmissions for N-AGCH message |
| --- | --- |
| 00 | 1 transmission |
| 01 | 1 initial transmission + 1 repetition |
| 10 | 1 initial transmission + 2 repetitions |
| 11 | 1 initial transmission + 3 repetitions |

Table 3:  Mapping of DL Radio Quality Indicator to required number of N-AGCH transmissions.

# coverage PERFORMANCE Evaluation

## Simulation Assumptions

Simulation assumptions for evaluating N-AGCH coverage performance are depicted in Table 4 below.

| Parameter | Value |
| --- | --- |
| Logical Channel | N-AGCH |
| Channel profile | TU |
| Mobile speed | 1.2 km/h |
| Band | GSM 900 |
| Number of repetitions | 3,4 (i.e. 4, 5 transmissions) |
| Repetition Period (Chase combining spacing) | 51-multiframe |
| Frequency error model | Frequency start offset: 40 Hz |

Table 4: Simulation Assumptions for N-AGCH performance evaluation.

Performance for N-AGCH was assessed for the sensitivity scenario based on above assumptions and is illustrated below.

## Simulation Results

BLER performance of N-AGCH channel for different number of transmissions is shown in Figure 2 below.

Figure 2: N-AGCH coverage performance for TU1.2nFH.

N-AGCH is observed to achieve 10% BLER at 164 dB MCL for TU1.2nFH, corresponding to Es/No=-6.3 dB, in case of 5 transmissions, thus it requires 5*51 multiframes (1177 ms) to receive the channel assignment message in response to the N-RACH attempt. The device at MCL=162 dB coverage condition can achieve the required coverage performance with 4 transmissions itself corresponding to a message delay of 942 ms.

# Conclusion

Payload bits required for N-AGCH is estimated by analysing the largest message sent over this channel (N-Fixed Uplink Assignment message). The channel structure is derived based on this analysis and mapping of N-AGCH blocks in N-CCCH control frame is illustrated. The number of blind repetitions required for N-AGCH is derived based on the downlink Radio Quality indicator sent in the preceding N-RACH channel access message.

The coverage performance for N-AGCH indicates that for a device at target MCL=164 dB 5 transmissions are required, whilst 4 transmissions can be achieved for an MCL up to 162 dB, corresponding to message delay of 1177 ms and 942 ms, respectively.

# REFERENCEs

- TR 45.820, v.1.3.1 “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, Rapporteur (Vodafone Group Plc)

- GP-150512 – “N-GSM: Design and Performance for N-AGCH”, Nokia Networks, 3GPP TSG GERAN#66