C-UNB technology for Cellular IoT

-

Link Layer aspects

revision of document GPC150311

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. Various radio access technology proposals are candidate for this WI. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB), for which key features are:

ultra-narrow band transmission,

random access,

cooperative reception.

Overall description of the C-UNB radio access technology can be found in [2]. The present document details the link layers aspects of the C-UNB radio access technology.

Revisions from GPC150145 are in red.

Revisions from GPC150311 are in green.

# Downlink link layer aspects

## Format of downlink MAC-PDUs

In C-UNB, downlink MAC-PDUs are optimized considering the fact that they are transmitted in reponse to up link transmission. Figure 1 illustrates the MAC-PDU format in C-UNB downlink. The various fields are detailed in the following subclauses.

Figure 1: Format of a MAC-PDU frame in C-UNB downlink transmission

### Header

Downlink header is 56 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a payload length,

acknowledgement bits.

### Payload

The payload of a MAC-PDU is 0-34 byte length. It carries application data and, when necessary, the link layer header (LL-header) for segmentation/re-assembly.

### Authentication

The authentication field is a hash, computed with an adaptation of the common C-MAC algorithm. It uses the 128-bit secrete key of the device plus:

the device identifier received in corresponding uplink packet,

the sequence number in the corresponding uplink packet,

the payload field.

This field is used to sign a downlink packet and also to identify the destination device implicitly (see figure 4): the probability to have many devices listening on a given frequency at a given time is very limited; hence a 16-bit hash is enough to authenticate the destination.

When a DL packet is trigged by a segmented UL packet, its authentication field uses the last sequence number transmitted in UL.

When a DL application packet is segmented at link layer, all its segments use the same sequence number, i.e. the sequence number corresponding to the UL packet that triggered the downlink transmission.

### FCS

The frame check sequence in C-UNB downlink consists in a CRC-8. It checks the MAC-PDU after error correction, before further decoding.

### ECC

This field is a 32-bit Error-Correction Code. It uses BCH code applied to the four first fields: header (except preamble), Payload, Authentication and FCS. Given the length of DL MAC-PDU, the ECC is expected to add 1 dB of processing gain in the DL MCL.

## Segmentation & reassembly in DL

Document [3] defines the maximum application packet to 2 000 bytes, with addition of a protocol header of 65 bytes (extended) or 29 bytes (compressed). Given the small size of C-UNB radio packets, it is proposed to have a segmentation/re-assembly mechanism with 32-bytes of application data and 2 bytes for segment numbering (see figure 2 and 3).

Figure 2: segmentation in C-UNB downlink

Figure 3: LL header format in downlink

This implementation gives a maximum size of the application data of 4 096 bytes, which is enough for the data packets expected in CIoT.

## DL mean throughput

Document [3] defines a target throughput of at least 160 bps at SNDCP interface. As C-UNB does not implement the legacy SNDCP and LLC layers, the mean throughput is evaluated with formula (1). In this formula, "time over the air" includes radio packet duration, gap between repetitions and four repetitions of each radio packet. Data rates at LL layer (i.e. equivalent to legacy SNDCP) are given in table 1.

Table 1: throughput at SNDCP-equivalent layer in DL

| Type of DL packet | Equivalent throughput |
| --- | --- |
| single data packet | 104 bps |
| segmented data packet | 99 bps |

# Uplink link layer aspects

## Uplink MAC-PDU

Figure 3 illustrates the MAC-PDU format in C-UNB uplink. The various fields are detailed in the following subclauses.

Figure 3: Format of a MAC-PDU frame in C-UNB uplink transmission

### Header

Uplink header is 32 bits. It consists in:

a preamble for frame detection and bit rate synchronization,

a frame type,

a frame length,

acknowledgement flags,

a repetition counter (value 1, 2 or 3) tagging the actual repetition number of MAC-PDUs.

Note: the number of repetition of a given MAC-PDU depends on the expected success rate. Two repetitions are for normal MAR, whereas three repetitions are for exception reports.

### Sequence counter

The uplink sequence counter is 12 bits. It is incremented for each new link layer packet (i.e. LL-PDU) sent by a device. The sequence counter is used for authentication purpose (see subclause 3.1.5).

### Identifier

This field is 40 bits long. It contains the device unique identifier set during personalization of the device. Each identifier is unique in a C-UNB system.

### Payload

The payload is 0-32 bytes. Payload length is defined by a 5-bit length field in the MAC-PDU header. Empty payload corresponds to Ack packet; they have a specific frame type.

### Authentication

The authentication field is a 16 bits hash, used to authenticate the MAC-PDUs received by the network. The 16 bits of the authentication field are generated using a proprietary implementation of the well-know C-MAC algorithm. It uses a 128 secrete key. Authentication check is performed in the core network and not in the base stations to avoid secrete key transmission on the back-haul network. Figure 4 illustrates the authentication process in uplink and the corresponding process in downstream.

Figure 4: Authentication of UL and DL packets

### FCS

The frame check sequence in C-UNB uplink consists in a CRC-8. In reception, it is used to check MAC-PDU consistency before transmission to the core network.

### ECC

The last field of an uplink MAC-PDU is an Error-Correcting Code of 16 bits. It used Reed-Solomon algorithm and is evaluated over the whole MAC-PDU except the preambule.

## Segmentation & reassembly in UL

The CIoT requirements define Mobile Autonomous Reports with 20-200 byte length. Given the fact that only small radio packets are affordable for the C-UNB radio access technology, it is proposed to have a segmentation/re-assembly mechanism based on 31 blocks of data with a LL header of 8 bits: 4 bits for segment numbering and 4 bits for total number of segments (see figure 5). These values set the maximum block size of application data to 496 bytes.

Figure 5: LL-header in C-UNB uplink

## UL mean throughput

The mean throughput at SNDCP-equivalent layer in uplink is given in table 2. Evaluation is made for two types of data packet (unsegmented and segmented) and for various repetitions rates in uplink.

Table 2: Throughput at SNDCP-equivalent layer in UL (in bps)

| Type of data packet | 1 PHY-PDU (no repetition) | 2 PHY-PDUs (1 repetition) | 3 PHY-PDUs (2 repetitions) |
| --- | --- | --- | --- |
| unsegmented data packet | 163 | 81 | 54 |
| segmented data packet | 160 | 80 | 53 |

# Conclusion

This contribution gives details on the link layer and MAC layer frame formats in C-UNB radio access technology and shows how they fulfill the requirements set by WI FS_IoT_LC [1].

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia

[2]	GPC150364, "Cooperative Ultra Narrow Band for Cellular IoT - General Description", SIGFOX, GERAN Ad'hoc meeting #3, 29June-02July 2015, Kista, Sweden

[3]	TR 45.820 v0.3.1 (2015-02), "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support Ultra Low Complexity and Low Throughput Internet of Things; (Release 13)"

[4]	GPC150145, " C-UNB technology for Cellular IoT - Link Layer aspects", SIGFOX, GERAN ad'hoc meeting#2, 20-23 April 2015, Sophia Antipolis, France