pCR for text inclusion in TR 45.820 :

C-UNB Concept Evaluation

revision of GPC150433

Introduction

This document provides text on the C-UNB candidate solution for inclusion in section 7.4.6 of the Technical Report 45.820 revision 1.3.1.

The proposed text for section 7.4.6 evaluates the C-UNB technology with respect to the TR 45.820 requirements.

This is a revision of GPC150433, where changes are in red.

Proposed text for the TR

Start of Changes

### Concept evaluation

#### Downlink Maximum Coupling loss

Table 7.4.6-1 gives the maximum coupling loss (MCL) of the downlink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 here after. Figures in table 7.4.6-1 are common, except the SINR, which is explained here after.

Subclause 5.1 states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message : when a MS moves at less than 30 km/h, the Doppler shift is about 25Hz. This small shift is not seen by the MS, which band filter is larger than the signal (i.e. 1kHz vs 600Hz).

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are relatively small when the device speed is lower than 30km/h: they don't move the received signal out of the 1kHz band of the MS receiver

fast fading, which is critical in GSM, is not an issue for C-UNB, because the symbol duration is 1,66 ms in C-UNB DL transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception or multiple transmissions. Considering the duration of DL radio packets (i.e. about 0,2-0,6 s), we propose to have four blind retransmissions of each DL radio packet.

When putting all this together, we propose to set the SINR value to 9dB. Table 7.3.6_1 gives the Maximum Coupling Loss of the C-UNB downlink transmission.

Table 7.4.6-1: Downlink Maximum Coupling Loss in C-UNB

| Logical channel name | C-UNB downlink |
| --- | --- |
| Data rate (bps) | 600 |
| Transmitter |  |
| (1) Tx power (dBm) | +34 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 6 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 1000 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -138 |
| (7) Required SINR (dB) | 9 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -129 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

#### Uplink Maximum Coupling Loss

Table 7.4.6-2 gives the maximum coupling loss (MCL) of the uplink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1. Figures in table 7.4.6-2 are quite understandable, except for the SINR one that is explained here after.

Subclause 5.1 here above states that SINR is evaluated with a TU1 path loss model. TUx path loss models have been proposed for modeling the influence of multipath propagation onto GSM radio signals that have a bandwidth of 200kHz and a modulation rate of 270kbps. In the case of ultra narrow band signals, the usual four effects of TU propagation models are mitigated the following ways:

the Doppler effect that shifts the center frequency of a message (about 25Hz at 30km/h): this frequency shift is totally managed by the base station because there is no channelization in C-UNB radio interface. Center frequencies are randomly selected by MSs, therefore a Doppler shift is not seen, in itself, by the base stations

the Doppler effect that creates a frequency drift within a received message : this is the case when the relative speed of a device is not constant. Such drifts are detected and corrected by the signal processing in C-UNB base stations

fast fading, which is critical in GSM, is not an issue for C-UNB, because of the 4ms symbol duration in C-UNB transmission

slow fading, which is due to interference between waves with a long term coherency, is usually overcome with multi-antenna reception. As C-UNB implements cooperative reception and large overlap of base station coverage, at least three different base stations receive each radio packet. Slow fading is, therefore, less critical than in GSM/GPRS system.

When putting all this together, we propose to set the SINR value to 8dB.

Table 7.4.6-2: Uplink Maximum Coupling Loss in C-UNB

| Logical channel name | C-UNB uplink |
| --- | --- |
| Data rate (bps) | 250 |
| Transmitter |  |
| (1) Tx power (dBm) | +24 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 3 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 250 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -147 |
| (7) Required SINR (dB) | 8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -139 |
| (9) Rx processing gain | 1 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

#### Downlink rate at SNDCP layer

The present document defines a target throughput of at least 160 bps at SNDCP interface. As C-UNB does not implement the legacy SNDCP and LLC layers, the mean throughput is evaluated with formula (1). In this formula, "time over the air" includes radio packet duration, gap between repetitions and four repetitions of each radio packet in DL. Data rates at LL layer (i.e. equivalent to legacy SNDCP) are given in table 7.4.6-3.

Table 7.4.6-3: throughput at SNDCP-equivalent layer in DL

| Type of DL packet | Equivalent throughput |
| --- | --- |
| single data packet | 104 bps |
| segmented data packet | 99 bps |

#### Uplink rate at SNDCP layer

The mean throughput at SNDCP-equivalent layer in uplink is given in table 7.4.6-4. Evaluation is made with formula (1) and for two types of data packet (unsegmented and segmented) and for various repetitions rates in uplink.

Table 7.4.6-4: Throughput at SNDCP-equivalent layer in UL (in bps)

| Type of data packet | 1 PHY-PDU (no repetition) | 2 PHY-PDUs (1 repetition) | 3 PHY-PDUs (2 repetitions) |
| --- | --- | --- | --- |
| unsegmented data packet | 163 | 81 | 54 |
| segmented data packet | 160 | 80 | 53 |

#### Battery Life evaluation

The battery life evaluation for C-UNB MS is based on methodology detailed in the present TR, but with two simplifying assumptions due to the C-UNB radio access technology:

MAR are supposed to be transmitted with two repetitions at physical layer,

no acknowledgement is transmitted in DL.

The second assumption is valid as long as the collision rate is less than 10%, which is the case in all performance evaluations because of network design.

The battery life evaluation assumes two lengths for the MAR: 50 and 200 bytes at SNDCP level. In C-UNB, such data packets are segmented and encapsulated as stated in table 7.4.6-3. MAC-PDU size and total number of transmission give the inputs for power consumption.

Table 7.4.6-3: MAR mapping in C-UNB PHY-PDU  (all sizes in bytes)

| MAR size | Number of segments | Segment size | LLC-PDU size | MAC-PDU size | Total # of PHY-PDU |
| --- | --- | --- | --- | --- | --- |
| 50 | 2 | 25 | 26 | 42,5 | 4 |
| 200 | 7 | 29 | 30 | 46,5 | 14 |

Table 7.4.6-4 gives key parameters for energy consumption analysis, whereas figure 7.4.6-1 gives energy consumption model for C-UNB devices. Figures for Tx and Rx power are those from off-the-shelves sub-gigaHertz chipsets or what is expected in the next generation of chipsets where architecture and power consumption will be optimized for ultra narrow band transmission.

Table 7.4.6-4: Key input parameters for energy consumption analysis

| (1) Battery capacity (Wh) | (2) Battery power during Tx (mW) | (3) Battery power for Rx (mW) | (4) Battery power when Idle but not in PSS (mW) | (5) Battery power in Power Save State (PSS) (mW) | (6) Idle time from PSS to Tx mode (ms) | (7) Tx time (ms) | (8) Idle time between two Tx (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 450 @ +24dBm 100 @ +14dBm 30 @ +4dBm | 40 | 10 | 0,015 | 50 | MAC-PDU size/bit rate | 25 |

Figure 7.4.6-1: Energy consumption model for C-UNB devices

Table 7.4.6-5 gives the battery life of C-UNB devices with a combination of two packet sizes, two reporting intervals and three coverage levels.

Table 7.4.6-5: Battery life (in years)

#### Capacity evaluation

FFS

#### Latency evaluation

##### Assumption on header for latency evaluation

Annex E.2.1 defines a MAR exception report payload of 20 bytes; the protocol overhead is 29 bytes (compressed) or 65 bytes (full overhead).

In C-UNB, the MAC-PDUs have relatively small payload size, resulting in segmentation when long application data have to be transmitted. In order to reduce the among of data carried over the air interface, the preferred implementation of the C-UNB protocol is to skip the transmission of the protocol header and let it manage by the MS-avatar hosted in the C-UNB server (see C-UNB general description). Nevertheless, this contribution evaluates latency with and without protocol header for the sake of comparison.

##### Blind repetitions

The C-UNB radio access technology assumes a number of blind repetitions for each MAC-PDU. Blind repetitions helps increasing the probability of good reception but reduces the overall network capacity. MAR exception reports require a high quality of service, therefore the present contribution considers that each MAC-PDU is transmitted with three PHY-PDUs (i.e. three repetition of the same radio packet) and then acknowledged by the base station.

##### Initial BLER evaluation

C-UNB radio access technology has three sources of BLER: propagation, noise and collision. Appropriate link budget allows keeping the related BLER under a given threshold, whereas multiple transmissions and space diversity helps to mitigate collision related BLER.

Given the length of the PHY-PDUs in table 7.4.6-6, the propagation-related BLER is always lower than 3.7%, hence lower than 5.10-5 with three repetitions.

Given the three repetitions of each MAC-PDU and the multiple receptions by several base stations, the collision related BLER is less than 3.10-4.

Therefore the latency is evaluated with the assumption that the initial message BLER is ≤ 1% (according to section 5.6 of the present document).

##### Results

Table 7.4.6-6 details latency calculation for MAR exception reports.

Table 7.4.6-6: latency evaluation (size in bytes and time in ms)

| Item | transmission with 65 byte protocol overhead | transmission with 29 byte protocol overhead | transmission with no protocol overhead (managed by MS-Avatar) |
| --- | --- | --- | --- |
| application data | 20 | 20 | 20 |
| protocol overhead | 65 | 29 | 0 |
| segmentation | 3x 29 | 2x 25 | 1x 20 |
| LL-PDU # and size | 3x 30 | 2x 26 | 1x 20 |
| MAC-PDU # and size | 3x 46,5 | 2x 42,5 | 1x 36,5 |
| PHY-PDU # and duration | 9x 1 488 | 6x 1 360 | 3x 1 168 |
| radio wake up time | 50 | 50 | 50 |
| gap between 2 radio packet | 5 | 5 | 5 |
| Latency | 13 482 | 8 235 | 3 564 |

The C-UNB radio access technology delivers the MAR exception reports in less than 10s when the transmission over the air interface is optimized, that is to say, when protocol overhead is either not transmitted over the air, or transmitted with compressed IP address.

When the full protocol header is transmitted over the air along with the application data, the latency for MAR exception reports is about 13.5 seconds.

End of Changes