UL System Level Simulation Results of GERAN CIoT Solutions: EC-GSM (update of GPC-150474)

1.  Introduction

A study item named “Cellular IoT” was approved in GERAN#62 for evaluating how to support low throughput and low complexity machine type communications [1].  Several proposals have been discussed under the categories of an evolved low-complex EGPRS (EC-GSM) and clean slate solutions (NB M2M and NB OFDMA).  The evaluation methodology, including the system assumptions, parameters and traffic models are captured in the draft TR 45.820 [2].

In this document, we present some uplink system level simulation results for EC-GSM solution for GERAN CIoT systems. In this work, we consider 2 traffic models and accordingly evaluate the system throughput and the delays experienced by the transmitted reports.

2.  Simulation Setup and Parameters

The major system level simulation assumptions and parameters are listed in Table 1.

TABLE 1: Simulation Parameters and Assumptions

| Parameter | Assumption |
| --- | --- |
| Cellular Layout | Hexagonal grid, 19 cell sites, 3 sectors per site |
| Frequency band | 900 MHz |
| Inter site distance | 1732 m |
| Number of carriers per cell site (3 sectors) | 3 |
| System bandwidth | 600 kHz |
| Frequency Hopping | off |
| Modulation coding schemes | MCS 1-4 |
| User distribution | Devices dropped uniformly in each sector |
| UE transmit power per 200 KHz | 33 dBm |
| Channel model | TU, 1 Hz Doppler |
| Antenna gains BS/UE | 18 dBi / -4 dBi |
| Traffic Model | Full-buffer, 3GPP LTE MTC Traffic models (TR 36.888, TR 45.820) |
| Number of Rx antennas | 2 |
| Noise figure | UL : 3 dB |
| Antennas | BS: GERAN directional (Am=20 dB, 65 degrees) / Omni |
| Pathloss | 120.9+ 37.6log10(R), R in kilometers |
| Frequency reuse | 1/3 |
| Shadowing standard deviation | 8 dB |
| Shadowing correlation distance | 110m ( 1 between sectors and 0.5 between sites ) |
| Shadowing site correlation factor | 50% |
| Penetration loss | TR 45.820, scenario 1 |
| Penetration loss correlation factor | 50% |
| Power Control | P0+ α *PL P0=-73.64dBm α = 0.8 |

Proportional Fair (PF) scheduling is considered in the simulations. In addition, we also consider a full-buffer traffic model with 10, 20, 30 and 40 simultaneous devices that are uniformly distributed in each sector. The objective is to evaluate the performance of the proposed EC-GSM system based on devices’ throughput and overall system throughput  for challenging scenarios in which all UEs are assumed to have a full-buffer traffic. The link-to-system mapping procedure proposed in [3] is employed for the physical layer abstraction. Note that, in this framework, perfect time and frequency synchronization and channel estimation are assumed and the MCS are selected based on a pessimistic approach; i.e., the MCS level is selected based on the 10% BLER performance curve corresponding to the equal interferers’ powers at the measured interference to noise ratio. This is in contrast with the conventional selection methodology wherein only the AWGN curves are considered. To illustrate, we note that in this work we utilized the link to system mapping technique proposed in [3]. In this technique, the BLER curve is selected, not only based on MCS level, but also based on the measured Interference. This allows for a more reliable capturing of the link level performance at the system level simulator. In addition, we note that the ratio between the dominant interferer and the total interference power has an impact on the BLER curve with the case in which all the interferers having equal power being the worst.  Furthermore, we also consider data traffic model with a deterministic number of reports per 200 KHz per Hour. This model is used to evaluate the reports’ delay and to check whether the system meets the 10 seconds delay requirement of [2] for the MAR exceptional reporting traffic model.

3. Simulation scenarios

In this proposal, we consider an EC-GSM system with blind repetition and scheduling penalty. The number of blind repetitions considered are 2, 4, 8, and 16. Each of these repetitions is decoded assuming coherent energy combining. The PF scheduler considers the channel quality of the devices and determines the number of repetitions based on predefined thresholds. In particular, based on the measured link quality, i.e., the SINR, the PF scheduler selects the approximate MCS level and number of repetitions necessary for reliable communications at 0.1 BLER.   In addition, to enhance the system capacity, we also consider the overlaid CDMA scenario described in [2]. This scenario allows up to 4 UEs to be concurrently scheduled in the same time slot given that these UEs are experiencing comparable link performance, i.e., they utilize the same MCS level and the same number of repetitions to achieve the target 0.1 BLER.

4. System level simulation results

4.1 Simulation of received signal strength

Scenario 1 of the building penetration loss specified in [2] is considered in our simulations.  The penetration loss values were generated for 1000 uniformly distributed devices within the cell area to obtain sufficient statistics for penetration loss parameters. In Figure 1, we present the CDF of the UEs received power assuming the channel parameters specified in table 1. It can be seen that the 50th percentile of the power received at the BS from different UE 
is -91.5dBm.

Figure 1.  CDF of the received signal strength.

4.1 System performance under a full-buffer traffic model

The uplink MAC throughput CDFs are presented in Figure 2. The scheduling mechanism discussed in Section 3 is simulated.  We assume 200 bps as the target MAC throughput for the GERAN CIoT devices. This assumes 160 bps at the equivalent of the SNDCP layer according to the requirements of [2] and approximately an additional 40 bps overhead for the physical layer/MAC overhead. The results indicate that, even under these heavy loading conditions, only less than 2% of the UEs are not fulfilling the throughput requirement of the study irrespective of the number of simultaneous UEs in the system. It is worthwhile mentioning that the presence of 20 simultaneous UEs in every scheduling interval corresponds to a system with approximately 642862 UEs according to the traffic model described in [4], given the assumption that UEs session were successfully transmitted within one scheduling interval.

Figure 2.  The UL device throughput CDFs of the EC-GSM system.

Finally, it is worth mentioning here that the CIoT applications in practice are expected to have very infrequent and sparse transmissions. Thus, the full-buffer traffic model assumption in the simulations helps in understanding the worst case performance for EC-GSM deployment.

4.2 System performance for MAR periodic reporting and Network Command traffic models

In this section, we evaluate the uplink performance of the EC-GSM with respect to the traffic model described in [2] for system capacity evaluation. In this traffic model it is assumed that 20% of the packets will be generated from the Network Command (NC) traffic model whereas the remaining 80% will be generated from the MAR periodic reporting traffic model [2].  In this traffic model, the payload size follows a Pareto distribution with minimum and maximum payload sizes of 20 and 200 bytes, respectively. Note that, for each payload there is a 65 bytes header overhead at the SNDCP layer as well as 15 bytes SNDCP to MAC overhead, i.e., each payload has an overhead of approximately 80 bytes at the MAC layer. The delivered reports/hour/200kHz versus the offered traffic and the reports’ delay CDF for the traffic model [2], are presented in Figures 5 and 6, respectively. Note that for MAR periodic reporting, approximately 6.81 reports per second per sector on average correspond to 52547 UEs per sector [6]. In this work, the scheduling mechanism discussed in Section 3 was employed. In addition, when evaluating the packet delays, we included both the random access delay, i.e., the delay associated with requesting the UL resources from the base station, as well as the synchronization delays, i.e., the delay associated with detecting and decoding the PSCH. However, we note that the random access delay is applied only to the MAR periodic reporting traffic. This is because, for the NC traffic, it is assumed that the device is already in the ready state due to the fact that it received the DL network command report and thus it does not need to do random access. In addition, when evaluating the random access delays, it is assumed that each device can perform up to a maximum of 6 random access attempts and that a collision between two or more devices would result in the base station not being able to decode the access requests and subsequently a back off delay of 1.5 seconds for the colliding devices [7].  From Figure 4, it can be clearly seen that there exists a linear relationship between the offered traffic and carried traffic, which indicates that all the offered traffic was transmitted without causing a significant increase in the buffer size. In other words, it indicates that the system is capable of handling the traffic without having a significant impact on the packet delays as shown in Figure 5.

Figure 4.  The Delivered reports/hour/200kHz versus # of generated reports/200KHz/Hour per sector of the traffic model in [2].

Figure 5.  Reports’ delay CDFs of the traffic model in [2].

5. Summary

In this contribution, we have presented our uplink system simulation results for the EC-GSM systems with full-buffer traffic as well as the MAR traffic model of [2].  Our results indicate that under the worst case load, i.e., the full-buffer traffic, approximately 98% of the devices would be able to receive at least 200 bps MAC throughput with blind repetitions for the simulation framework assumed in the study when the number of simultaneous UEs in the system is less than 40. Hence, we conclude that EC-GSM can efficiently operate in the very challenging scenario of full-buffer traffic. In addition, under the MAR/NC traffic models, the system is able to operate efficiently and satisfy the 10 second delay requirement for approximately 225K reports/200KHz/Hour. It was shown by simulations that up to this load, the offered traffic and the carried traffic have a linear relationship which indicates that the system is capable of handling such load.

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

3GPP TR 45.820 v1.2.1, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

GP-15xxx, “GERAN CIoT Link-to-System Mapping Methodology,” Intel Corporation, GERAN # 66.

GP-150090, “NB M2M-System Level Simulation (Update of GPC-150118)”, source Huawei Technologies Co., Ltd, HiSilicon Technologies Co., Ltd. GERAN#65.

GPC-150280, “EC-GSM, EC-PDTCH UL system capacity evaluation”, source Ericsson LM, GERAN#66.

GP-150477m “NB M2M – System Level Simulation for Capacity and Latency Evaluation (update of GPC 150261)”, source Huawei Technologies Co., Ltd, HiSilicon Technologies Co., Ltd. GERAN#66.

GP-150433 “EC-GSM, EC-RACH System Capacity Evaluation (update of GPC150189)”, source Ericsson LM, GERAN #66.