3GPP TSG-GERAN Ad Hoc on EC-GSM and eDRX 	GPE150007

# Stockholm, Sweden, 12-13 October, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.820 | CR | 0002 | rev | - | Current version: | 13.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EC-GSM System Level Simulations Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-08-28 |
| Category: | D |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | Editorial error in Sections 6.2.6.15.3 and 6.2.6.15.4 in TR 45.820 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Figures 6.2.6.15.3.1-1 and 6.2.6.15.4.1-1 need to be updated and their corresponding text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In the TR45.820 there is an editor’s note below the figures saying that it needs to be updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TR 45.820 CR ... TS 24.008 CR ... TS 44.004 CR ... TS 44.018 CR ... TS 44.060 CR ... TS 45.001 CR ... TS 45.002 CR ... TS 45.003 CR ... TS 45.004 CR ... TS 45.005 CR ... TS 48.018 CR … |  |  |  |
| affected: |  |  |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

##### 6.2.6.15.3	Downlink System Level Performance

6.2.6.15.3.1 	System performance for MAR periodic reporting and Network Command traffic models

In this clause, the DL performance of the EC-GSM with respect to the traffic model of Annex E is evaluated. In the presented system simulations, frequency reuse factor of 4/12 and proportional fair scheduling are considered. In addition, the effect of the BPL according to scenario 1 with 0.5 correlation is applied. The DL offered traffic versus the carried traffic and the packet delay CDFs of this traffic model are presented in Figures 6.2.6.15.3.2-1 and 6.2.6.15.3.2-2, respectively. When evaluating the packet delays, we included both the random access delay, i.e. the delay associated with requesting the UL resources from the base station, as well as the synchronization delays, i.e. the delay associated with detecting and decoding the PSCH. However, we note that the random access delay is applied only to the NC traffic since the MAR periodic reporting traffic is only an acknowledgment which implies that the devices are already in the ready state. In addition, when evaluating the random access delays, it is assumed that each device can perform up to a maximum of 6 random access attempts and that a collision between two or more devices would result in the base station not being able to decode the access requests and subsequently a back off delay for the colliding devices according to their class [6.2-20]. In particular, classes 1-6 are considered to apply back-off times of [0.5 0.5 0.5 0.5 1.5 2] seconds, respectively. Note that, the probability of having a collision increases with the device class due to the access burst repetitions. Furthermore, in this work, the synchronization delay is lower and upper bounded by the time required to acquire the synchronization by accumulating from 1 up to 28 SCH burst repetitions, respectively. In addition, all sectors are assumed to have the same average device load. For example, when the system performance is evaluated for a 64K device load at the center cell, it is assumed that each of the remaining 56 sectors has 64K devices. A linear relationship between the offered traffic and the carried traffic up to 250K users per sector can be seen. This relationship indicates that all the offered traffic can be transmitted without increasing the buffer size. Furthermore, up to 240K users per sector, around 98% of the reports are delivered in less than 2 seconds (thus meeting the maximum 10 seconds delay requirement). Note that, to obtain the reports arrival rate per second, the number of devices per sector can be multiplied by 5.448/52547 as illustrated in the previous section.

Figure 6.2.6.15.3.1-1: The number of UEs per sector versus carried traffic in the traffic model simulation with frequency reuse 4/12 and 43 dBm TX power

Figure 6.2.6.15.3.1-2:  Report delay CDFs in traffic model simulations with frequency reuse 4/12  and 43 dBm TX power

##### 6.2.6.15.4	Uplink System Level Performance

6.2.6.15.4.1	System performance for MAR periodic reporting and Network Command traffic models

In this section, the uplink performance of the EC-GSM with respect to the traffic model discussed in the previous section is evaluated. The delivered reports/hour/200kHz versus the offered traffic and the reports' delay CDF for the traffic model of Annex E, are presented in Figures 6.2.6.15.4.1-1 and 6.2.6.15.4.1-2, respectively. When evaluating the packet delays, both the random access delay, i.e. the delay associated with requesting the UL resources from the base station, as well as the synchronization delays, i.e. the delay associated with detecting and decoding the PSCH are  included. However, note that the random access delay is applied only to the MAR periodic reporting traffic. This is because, for the NC traffic, it is assumed that the device is already in the ready state due to the fact that it already received the DL network command report and thus it does not need to do random access. In addition, when evaluating the random access delays, it is assumed that each device can perform up to a maximum of 6 random access attempts and that a collision between two or more devices would result in the base station not being able to decode the access requests and subsequently a back off delay for the colliding devices according to their class [6.2-20]. In particular, classes 1 to 6 are considered to implement back-off times of [0.5 0.5 0.5 0.5 1.5 2] seconds, respectively. Note that, the probability of having a collision increases with the device class due to the access burst repetitions. Furthermore, in this work, the synchronization delay is lower and upper bounded by the time required to acquire the synchronization by accumulating from 1 up to 28 SCH burst repetitions, respectively.

In the presented simulation results below, 4/12 frequency reuse factor and proportional fair scheduling are considered. In addition, all the sectors are assumed to have the same average device load. For example, when the system performance is evaluated for a 64K device load at the center cell, it is assumed that each of the remaining sectors has 64K devices. In addition, we considered the effect of the BPL according to scenario 1 with 0.5 correlation. A power control protocol was implemented to reduce the interference incurred by the devices.  According to this model, the power transmitted by each device is equal to min (Pmax, P+ α *PL), where Pmax= 33dBm, P= -73.64 dBm, α = 0.8 and PL is the pathloss between the BS and the device. Note that, the value of P was selected based on the received signal strength CDF such that, on average, only 20% of the devices are transmitting at full power.

From Figure 6.2.6.15.4.1-1, it can be clearly seen that there exists a linear relationship between the offered traffic and carried traffic, which indicates that all the offered traffic was transmitted without causing a significant increase in the buffer size. Note that the slight decrease in the slope of the carried traffic is directly related to the collisions occurring at the random access channel. This is because in the uplink, the devices are assumed to utilize high numbers of blind repetitions and thus they are expected to experience some collisions even at low loading scenarios. However, this approximately linear relationship indicates that the system is capable of handling the traffic without having a significant impact on the packet delays as shown in Figure 6.2.6.15.4.1-1. Note that, to obtain the reports arrival rate per second, the number of devices per sector can be multiplied by 6.81/52547 as illustrated in the previous section. From the packets' delay perspective, in Figure 6.2.6.15.4.1-2, it can be seen that the EC-GSM system is capable of transmitting all the packets with significantly low delays. In particular, even for a load of 194K users per sector, almost 99% of the packets can be delivered in less than 4 seconds. Note that in this figure, the cross over between the 193K and the 63K curves is basically due to the fact that the UEs performing large numbers of repetitions have a higher probability of failure at the RACH channel as the system load increases. Subsequently, this reduces the number of users experiencing large delays since they will not be admitted to the system.

Figure 6.2.6.15.4.1-1: The number of UEs per sector versus carried traffic with frequency reuse 4/12 and 33 dBm TX power.

Figure 6.2.6.15.4.1-2: Reports' delay CDFs of the traffic model with frequency reuse 4/12 and 33 dBm TX power

End of modifications