3GPP TSG-GERAN Ad Hoc on EC-GSM and eDRX 	GPE150030

# Stockholm, Sweden, 12-13 October, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0184 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-09-29 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of logical channels capable of operating in extended coverage, EC-channels. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.2, 1.2a(new), 3.2.1, 3.2.4, 3.3.2.2, 3.3.2.3, 3.3.3, 3.3.4.1, 3.3.4.2, 3.3.4.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR xxxx,xxxx TS 44.060 CR xxxx,xxxx,xxxx,xxxx TS 45.001 CR 0079,0080 TS 45.002 CR 0183, 0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  | The CR contains changes that are relevant to both Work item code eDRX_GSM and work item code CIoT_EC_GSM. Background text, or conflicting text from the corresponding eDRX_GSM CR is included without highlighting, and text specific to CIoT_EC_GSM is included with grey highlighted text. CR conflicts need to be resolved before agreeing the CRs |  |  |  |  |  |  |  |  |

1st modification

## 1.2	Abbreviations

Abbreviations used in the present document are listed in 3GPP TR 21.905. In addition to abbreviations in 3GPP TR 21.905, the following abbreviations are applied:

BTTI	Basic Transmission Time Interval

eDRX	Extended Discontinuous Reception

ETTI	Extended Transmission Time Interval

FANR	Fast Ack/Nack Reporting

PSM	Power Saving Mode

RTTI	Reduced Transmission Time Interval

TTI	Transmission Time Interval

2nd modification

## 1.2a	Definitions

Coverage Class: see definition in 3GPP TS 43.064.

EC-EGPRS: see definition in 3GPP TS 43.064.

3rd modification

### 3.2.1	General

Traffic channels (TCH's) are intended to carry either encoded speech or user data in circuit switched mode. Five general forms of traffic channel are defined:

i)	Full rate traffic channel (TCH/F). This channel carries information at a gross rate of 22,8 kbit/s.

ii)	Half rate traffic channel (TCH/H). This channel carries information at a gross rate of 11,4 kbit/s.

iii)	Enhanced circuit switched full rate traffic channel (E-TCH/F). This channel carries information at a gross rate of 69,6 kbit/s including the stealing symbols.

iv)	8-PSK full rate traffic channel (O-TCH/F). This channel carries information at a gross rate of 68,4 kbit/s.

v)	8-PSK half rate traffic channel (O-TCH/H). This channel carries information at a gross rate of 34,2 kbit/s.

Packet data traffic channels (PDTCH's) are intended to carry user data in packet switched mode. For the purpose of this Technical Specification, any reference to traffic channel does not apply to PDTCH unless explicitly stated.

All traffic channels are bi-directional unless otherwise stated. Unidirectional downlink full rate channels, TCH/FD, are defined as the downlink part of the corresponding TCH/F.

Multiple full rate channels can be assigned to the same MS. This is referred to as multislot configurations, which is defined in subclause 6.4.2.1.

Multiple packet data traffic channels can be assigned to the same MS or, in the case of point-to-multipoint transmission, a group of MSs. This is referred to as multislot packet configurations, as defined in subclause 6.4.2.2 and subclause 6.4.2.3a.

A combination of a half rate traffic channel and a half rate packet data traffic channel on the same basic physical channel can be assigned to the same MS as defined in subclause 6.4.2.3.

A combination of a traffic channel and one or more full rate packet data traffic channels can be assigned to the same MS.

A pair of speech traffic channels along with their associated control channels sharing the same timeslot number (see subclause 4.3), ARFCN (see subclause 6.2.3) and TDMA frame number (see subclause 4.3) is referred to as a VAMOS pair. The speech traffic channels along with their associated control channels in a VAMOS pair are said to be in VAMOS mode and are referred to as VAMOS subchannels.

In case of speech traffic channels in VAMOS mode, up to 4 speech traffic channels can be mapped on the same basic physical channel both in downlink and uplink (see subclause 6.4.1).

The specific traffic channels available in the categories of speech and user data are defined in the subclauses following.

4th modification

### 3.2.4	Packet data traffic channels (PDTCH and EC-PDTCH)

A PDTCH/F corresponds to the resource assigned to a single MS or, in the case of point-to-multipoint transmission, to multiple MSs for user data transmission. An EC-PDTCH/F always corresponds to the resource assigned to a single MS.

In BTTI configuration, a (EC-)PDTCH/F is mapped onto one physical channel (see subclause 6.3.2.1). Due to the dynamic multiplexing onto the same physical channel of different logical channels (see subclause 6.3.2), a (EC-)PDTCH/F in BTTI configuration carries information at an instantaneous bit rate ranging from 0 to a maximum value dependent on the modulation and on the symbol rate, as given in the following table.

| Modulation | Maximum instantaneous bit rate (kbit/s) when using the normal symbol rate (see 3GPP TS 45.004) | Maximum instantaneous bit rate (kbit/s) when using the higher symbol rate (see 3GPP TS 45.004) |
| --- | --- | --- |
| GMSK | 22,8 | - |
| QPSK | - | 55,2 |
| 8-PSK | 69,6 | - |
| 16QAM | 92,8 | 110,4 |
| 32QAM | 116,0 | 138,0 |

In RTTI configuration, a PDTCH/F is mapped onto two physical channels, i.e. a PDCH-pair (see subclause 6.3.2.1). A PDTCH/F in RTTI configuration carries information at an instantaneous bit rate ranging from 0 to a maximum value which is double the corresponding value for that modulation and the symbol rate.

A PDTCH/H corresponds to the resource assigned to a single MS on half a physical channel for user data transmission. The maximum instantaneous bit rate for a PDTCH/H is half that for a PDTCH/F. A PDTCH/H is only possible in BTTI configuration if FANR is not activated (see 3GPP TS 44.060).

All packet data traffic channels are uni-directional, either uplink (PDTCH/U), for a mobile originated packet transfer or downlink (PDTCH/D) for a mobile terminated packet transfer.

In the case of point-to-multipoint transmission, a PDTCH/D can be used for communication with multiple MSs.

5th modification

#### 3.3.2.2	Synchronization channels

The synchronization channel carries information for frame synchronization of the mobile station and identification of a base transceiver station. It is required only for the operation of the radio sub-system. Different channels are used for SCH, EC-SCH and COMPACT CSCH.

##### 3.3.2.2.1	Synchronization channel (SCH)

Specifically the synchronization channel (SCH) shall contain two encoded parameters:

a)	Base transceiver station identity code (BSIC): 6 bits (before channel coding) consists of 3 bits of PLMN colour code with range 0 to 7 and 3 bits of BS colour code with range 0 to 7 as defined in 3GPP TS 23.003.

b)	Reduced TDMA frame number (RFN): 19 bits (before channel coding) =

T1		(11 bits)	range 0 to 2047	= FN div ( 26 x 51)

T2		(5 bits)	range 0 to 25		= FN mod 26

T3	'	(3 bits)	range 0 to 4		= (T3 - 1) div 10

where

T3		(6 bits)	range 0 to 50		= FN mod 51

and

FN = TDMA frame number as defined in subclause 4.3.3.

3GPP TS 44.006 and 3GPP TS 44.018 specify the precise bit ordering, 3GPP TS 45.003 the channel coding of the above parameters and 3GPP TS 45.010 defines how the TDMA frame number can be calculated from T1, T2, and T3'.

##### 3.3.2.2.2	COMPACT synchronization channel (CSCH)

The COMPACT packet synchronization channel CSCH shall contain two encoded parameters:

a)	Base transceiver station identity code (BSIC): 6 bits (before channel coding) consists of 3 bits of PLMN colour code with range 0 to 7 and 3 bits BS colour code with range 0 to 7 as defined in 3GPP TS 23.003.

b)	Reduced TDMA frame number (RFN): 19 bits (before channel coding) =

R1		(10 bits)	range 0 to 1023	= FN div (51 x 52)

R2		(6 bits)	range 0 to 50		= (FN div 52) mod 51

TG		(2 bits)	range 0 to 3

Reserved	(1 bit)

where

FN = TDMA frame number as defined in subclause 4.3.3

and

TG = time group as defined in subclause 4.3.4.

3GPP TS 44.006 and 3GPP TS 44.018 specify the precise bit ordering, 3GPP TS 45.003 the channel coding of the above parameters and 3GPP TS 45.010 defines how the TDMA frame number can be calculated from R1 and R2.

##### 3.3.2.2.3	Extended Coverage synchronization channel (EC-SCH)

The extended coverage synchronization channel (EC-SCH) shall contain two encoded parameters:

a)	Base transceiver station identity code (BSIC): 6 bits (before channel coding) consists of 3 bits of PLMN colour code with range 0 to 7 and 3 bits of BS colour code with range 0 to 7 as defined in 3GPP TS 23.003.

b)	Reduced TDMA frame number per quarter hyperframe (RFNQH): 14 bits (before channel coding) =

T1'		(8 bits)	range 0 to 255		= 8 least significant bits of T1'	 = T1 mod 256 (for definition of T1, see subclause 3.3.2.2.1)

NOTE: T1' identifies one of 256 pairs of superframes within a quarter hyperframe

T2'		(4 bits)	range 0 to 12			= (FN div 51*4) mod 13

NOTE: T2' identifies a specific set of four contiguous 51-multiframes in a pair of superframes)

T2'' 	(2 bits) 	range 0 to 3 			= (FN div 51) mod 4

NOTE: T2'' identifies the specific 51-multiframe in the set of four contiguous 51-multiframes). T2''  is signalled through the cyclic shift pattern used on the EC-SCH, see 3GPP TS 45.003.

T3'' has a value in the range {0, 1, 2, …50} and will be determined by the device through the identification of the mapping of the FCCH onto the specific TDMA frames within the 51-multiframe, see Table 3.

FN = TDMA frame number as defined in subclause 4.3.3.

After the decoding of EC-SCH, the MS will have knowledge about the frame structure within a quarter hyperframe. The QUARTER_HYPERFRAME_INDICATOR is acquired when receiving an immediate assignment, see 3GPP TS 44.018.

QUARTER_HYPERFRAME_INDICATOR (2 bits) 	range 0…3

3GPP TS 44.006 and 3GPP TS 44.018 specify the precise bit ordering, 3GPP TS 45.003 the channel coding of the above parameters and 3GPP TS 45.010 defines how the TDMA frame number can be calculated from T1, T2, and T3'.

6th modification

#### 3.3.2.3	Broadcast control channel (BCCH)

The broadcast control channel broadcasts general information on a base transceiver station per base transceiver station basis. Of the many parameters contained in the BCCH, the use of the following parameters, as defined in 3GPP TS 44.018 are referred to in subclause 6.5:

a)	CCCH_CONF which indicates the organization of the common control channels:

From this parameter, the number of common control channels (BS_CC_CHANS) and whether or not CCCH or SDCCH are combined (BS_CCCH_SDCCH_COMB = true or false) are derived as follows:

| CCCH_CONF | BS_CC_CHANS | BS_CCCH_SDCCH_COMB |
| --- | --- | --- |
| 000 | 1 | false |
| 001 | 1 | true |
| 010 | 2 | false |
| 100 | 3 | false |
| 110 | 4 | false |

b)	BS_AG_BLKS_RES which indicates the number of blocks on each common control channel reserved for access grant messages:

3 bits (before channel coding) range 0 to 7.

In a routing area where eDRX is supported, the number of blocks per 51-multiframe reserved for AGCH access grant messages is subject to the requirement that all cells in the routing area shall have the same number of paging blocks per 51-multiframe:

c)	BS_PA_MFRMS which indicates the number of 51-multiframes between transmission of paging messages to mobiles of the same paging group:

3 bits (before channel coding) range 2 to 9.

An exception is the case of a cell where eDRX is supported (see 3GPP TS 44.018 [10]) in which case the number of 51-multiframes between transmissions of paging messages is based on the eDRX value negotiated between the MS and the network - see Table 6.5.6a.1

d)	support of GPRS

The BCCH shall indicate whether or not packet switched traffic is supported. If packet switched traffic is supported and if the PBCCH exists, then the BCCH shall broadcast the position of the packet data channel (PDCH), as defined in subclause 6.3.2.1, carrying the PBCCH. (See sub-clause 1.3).

7th modification

### 3.3.3	Common control type channels

#### 3.3.3.1	Common control type channels, known when combined as a common control channel (CCCH)

i)	Paging channel (PCH): Downlink only, used to page mobiles.

ii)	Random access channel (RACH): Uplink only, used to request assignment of a SDCCH.

iii)	Access grant channel (AGCH): Downlink only, used to assign a SDCCH or directly a TCH.

iv)	Notification channel (NCH): Downlink only, used to notify mobile stations of voice group and voice broadcast calls.

#### 3.3.3.2	Packet Common control channels

##### 3.3.3.2.1	Packet Common Control Channels (PCCCH)

i)	Packet Paging channel (PPCH): Downlink only, used to page MS.

ii)	Packet Random access channel (PRACH): Uplink only, used to request assignment of one or several PDTCHs (for uplink or downlink direction).

iii)	Packet Access grant channel (PAGCH): Downlink only, used to assign one or several PDTCH.

If a PCCCH is not allocated, the information for packet switched operation is transmitted on the CCCH. If a PCCCH is allocated, it may transmit information for circuit switched operation. (See sub-clause 1.3).

The PCCCH channel of a cell shall be allocated on the same frequency band (see 3GPP TS 45.005) as the BCCH channel of that cell.

##### 3.3.3.2.2	COMPACT Common Control Channels (CPCCCH)

i)	Packet Paging channel (CPPCH): Downlink only, used to page MS.

ii)	Packet Random access channel (CPRACH): Uplink only, used to request assignment of one or several PDTCHs (for uplink or downlink direction).

iii)	Packet Access grant channel (CPAGCH): Downlink only, used to assign one or several PDTCH.

##### 3.3.3.2.3	MBMS Common Control Channels

i)	MBMS Packet Random access channel (MPRACH): Uplink only, used during the initial counting procedure for MBMS (see 3GPP TS 44.060).

##### 3.3.3.2.4	Extended Coverage Common Control Channels

i)	Extended Coverage Paging channel (EC-PCH): Downlink only, used to page MS.

ii)	Extended Coverage Random access channel (EC-RACH): Uplink only, used to request assignment of one or several PDTCHs (for uplink or downlink direction).

iii)	Extended Coverage Access grant channel (EC-AGCH): Downlink only, used to assign one or several PDTCH.

8th modification

#### 3.3.4.1	Circuit switched dedicated control channels

i) 		Slow, TCH/F or E-TCH/F associated, control channel (SACCH/TF).

ii)		Fast, TCH/F associated, control channel (FACCH/F).

iii)		Slow, TCH/H or O-TCH/H associated, control channel (SACCH/TH).

iv)		Fast, TCH/H associated, control channel (FACCH/H).

v)		Stand alone dedicated control channel (SDCCH/8).

vi)		Slow, SDCCH/8 associated, control channel (SACCH/C8)

vii)		Stand alone dedicated control channel, combined with CCCH (SDCCH/4).

viii)	Slow, SDCCH/4 associated, control channel (SACCH/C4).

ix)		slow, TCH/F, O-TCH/F or E-TCH/F associated, control channel for multislot configurations (SACCH/M).

x)		slow, TCH/F associated, control channel for CTS (SACCH/CTS).

xi)		Fast, E-TCH/F associated, control channel (E-FACCH/F).

xii)		Inband, E-TCH/F associated, control channel (E-IACCH/F).

xiii)	Slow, TCH/F or O-TCH/F associated, control channel for enhanced power control (SACCH/TPF).

xiv)	Slow, TCH/F or O-TCH/F associated, control channel for enhanced power control in multislot configurations (SACCH/MP).

xv)		Slow, TCH/H or O-TCH/H associated, control channel for enhanced power control (SACCH/TPH).

xvi)	Enhanced power control, TCH/F or O-TCH/F associated channel (EPCCH/F).

xvii)	Enhanced power control, TCH/F or O-TCH/F associated channel in multislot configurations (EPCCH/M).

xviii)	Enhanced power control, TCH/H or O-TCH/H associated channel (EPCCH/H);

xix)	Fast, O-TCH/H associated, control channel (O-FACCH/H);

xx)		Fast, O-TCH/F associated, control channel (O-FACCH/F).

All associated control channels have the same direction (bi-directional or unidirectional) as the channels they are associated to. The unidirectional SACCH/MD, SACCH/MPD and EPCCH/MD are defined as the downlink part of SACCH/M, SACCH/MP and EPCCH/M respectively.

9th modification

#### 3.3.4.2	Packet dedicated control channels

i)	The Packet Associated Control channel (PACCH): The PACCH is bi-directional. For description purposes PACCH/U is used for the uplink and PACCH/D for the downlink. The PACCH shall be transmitted using the same configuration (BTTI or RTTI) of the PDTCH that it is associated with.

ii)	Packet Timing advance control channel uplink (PTCCH/U): Used to transmit random access bursts to allow estimation of the timing advance for one MS in packet transfer mode.

iii)	Packet Timing advance control channel downlink (PTCCH/D): Used to transmit timing advance updates for several MS. One PTCCH/D is paired with several PTCCH/U's.

#### 3.3.4.3	Extended Coverage Packet dedicated control channels

i)	The Extended Coverage Packet Associated Control channel (EC-PACCH): The EC-PACCH is bi-directional. For description purposes EC-PACCH/U is used for the uplink and EC-PACCH/D for the downlink. EC-PACCH shall be transmitted using the same configuration (BTTI or ETTI) of the EC-PDTCH, in the same direction, it is associated with. The EC-PACCH shall be transmitted using a configuration (BTTI or ETTI) according to its Coverage Class. The Coverage Class need not be the same on uplink and downlink and hence different configurations could be used by the EC-PACCH and the EC-PDTCH, if used in different directions.

End of modifications