3GPP TSG-GERAN Ad Hoc on EC-GSM and eDRX 	GPE150033

# Stockholm, Sweden, 12-13 October, 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.002 | CR | 0183 | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS, Multislot capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-10-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | EC-EGPRS is a feature based on EGPRS that introduces a streamlined protocol implementation, ensuring energy efficient operation and improved security while supporting extended coverage up to [20 dB] compared to GPRS/EGPRS operation. The feature is not yet introduced in the GERAN specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of multislot capabilities for EC-EGPRS mobile stations |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The EC-EGPRS feature will not be introduced in the GERAN specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2, B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR xxxx,xxxx TS 44.060 CR xxxx,xxxx,xxxx,xxxx TS 45.001 CR 0079,0080 TS 45.002 CR 0184,0185,0186 TS 45.003 CR 0135 TS 45.004 CR 0020 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 51010 ... CR ... TS 51021 ... CR ... |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st modification

#### 6.4.2.2	Multislot configurations for packet switched connections in A/Gb mode

In A/Gb mode, an MS may be assigned several (EC-)PDTCH/Us or (EC-)PDTCH/Ds for one mobile originated or one mobile terminated communication respectively, mapped onto a corresponding number of PDCHs in BTTI configuration or ETTI configuration ,or, a corresponding number of PDCH-pairs in RTTI configuration. The total number of assigned uplink PDCHs and downlink PDTCHs shall not exceed the total number of uplink and downlink timeslots that can be used by the MS per TDMA frame (i.e., the parameter ‘Sum’ specified in Annex B). An exception is the case of EC-EGPRS operation where uplink and downlink PDCHs are not assigned simultaneously and hence the number of assigned uplink and downlink PDCHs shall not exceed the maximum number of receiving or transmitting timeslots that can used by the MS per TDMA frame (i.e. the parameters ‘Rx’ and ‘Tx’ respectively in Annex B). In this context "assignment" refers to the list of PDCH given in the assignment message and that may dynamically carry the (EC-)PDTCHs for that specific MS.

Alternatively, for a multislot class type 1 MS supporting Flexible Timeslot Assignment (see 3GPP TS 24.008) the network may assign a total number of uplink and downlink PDCHs exceeding the parameter ‘Sum’ specified in Annex B, provided that the number of assigned downlink PDCHs shall not exceed the number of downlink timeslots that can be used by the MS per TDMA frame (i.e., the parameter ‘Rx’ specified in Annex B) and the number of assigned uplink PDCHs shall not exceed the number of uplink timeslots that can be used by the MS per TDMA frame (i.e., the parameter ‘Tx’ specified in Annex B). In this case, the network shall ensure that, in each radio block period, the total number of uplink and downlink PDCHs that have been allocated to the MS does not exceed the total number of uplink and downlink timeslots that can be used by the MS per TDMA frame (i.e., the parameter ‘Sum’ specified in Annex B).

Alternatively, when Enhanced Flexible Timeslot Assignment, EFTA, is used (see 3GPP TS 24.008) the network shall follow the same procedure as for Flexible Timeslot Assignment as described above with the exception that, during any given radio block period, the total number of uplink and downlink PDCHs that have been allocated to the MS may exceed the total number of uplink and downlink timeslots defined by the parameter ‘Sum’ specified in Annex B.

NOTE 1:	In the downlink, a PDCH is ‘allocated’ to an MS in a radio block period if the network transmits an RLC/MAC block for the MS on that PDCH during that radio block period.

In RTTI configuration, PDCHs shall be assigned in pairs.

If there are m timeslots assigned for reception and n timeslots assigned for transmission:

–	For a multislot class type 1 MS, there shall be Min(m,n,2) reception and transmission timeslots with the same TN;

–	For a multislot class type 2 MS, there shall be Min(m,n) reception and transmission timeslots with the same TN.

In the case of downlink dual carrier or DLMC configurations, if timeslots with the same timeslot number are assigned on more than one carrier, in calculating the value of m they shall be counted as one timeslot.

The mapping of (EC-)PACCH onto the assigned downlink PDCHs or the allocated uplink PDCHs is specified in 3GPP TS 44.060.

For multislot class type 1 MS, Table 6.4.2.2.1 lists the number of timeslots (in a dual carrier configuration or a DLMC configuration the numbe of timeslots apply on a per radio frequency basis) that are possible to assign (provided that it is supported by the MS according to its multislot class) for different medium access modes (see 3GPP TS 44.060). It also indicates if the network (or the MS in case of EFTA) shall apply Tra or Tta (see annex B), and if Shifted USF operation shall apply (see 3GPP TS 44.060). Additionally, it indicates which configurations can also be used for allocation (provided that they are compatible with the number of timeslots assigned to the MS). For a MS supporting EC-EGPRS Table 6.4.2.2.1 does not apply since all assignments according to its multislot class are possible. Tra, Tta,, Trb, Ttb and ‘Sum’ (see Annex B) does not apply to in EC-EGPRS operation.

NOTE 2:	In case of extended dynamic allocation, the MS needs to support USF monitoring on the downlink PDCHs corresponding to (i.e. with the same timeslot number as) all assigned uplink PDCHs as defined in 3GPP TS 44.060.

In a dual carrier or a DLMC configuration, all the downlink timeslots on all radio frequency channels shall be assigned within a window of  size ‘d’ and all the uplink timeslots on all radio frequency channels shall be assigned within a window of  size ‘u’ where ‘d’ and ‘u’ are defined in Table 6.4.2.2.1.

In a dual carrier configuration the maximum number of timeslots that may be assigned (uplink and downlink) depends on the multislot class of the MS (or the Equivalent multislot class if different from the Signalled multislot class as described in B.4) and the multislot capability reduction for downlink dual carrier.

In a DLMC configuration the maximum number of timeslots that may be assigned (uplink and downlink) depends on the multislot class of the MS and the supported Maximum Number of Downlink Timeslots, see 3GPP TS 24.008.

The maximum number of radio frequency channels on which downlink timeslots may be assigned in a DLMC configuration is dependent on the supported Maximum Number of Downlink Carriers, see 3GPP TS 24.008.

In a dual carrier or a DLMC configuration, Shifted USF operation shall be determined per carrier according to the number of downlink and uplink timeslots assigned on each carrier

Table 6.4.2.2.1: Multislot configurations for packet switched connections in A/Gb mode

| Medium access mode | No of Slots (Note 0) | Tra shall apply | Tta shall apply | Applicable Multislot classes (see Note 7) | Note |
| --- | --- | --- | --- | --- | --- |
| Downlink, any mode | d = 1-6 | Yes | - | 1-12, 19-45 |  |
|  | d = 7-8 | No | - | 24-29 | 1,2 |
| Uplink, Dynamic | u = 1-2 | Yes | - | 1-12, 19-45 | 10 |
|  | u = 2 | - | Yes | 12, 36-39 | 11 |
|  | u = 3 |  | Yes | 12, 37-39 | 9 |
|  | u = 2-3 | Yes | - | 31-34, 41-45 | 9 |
| Uplink, Ext. Dynamic | u = 1-3 | Yes | - | 1-12, 19-45 |  |
|  | u = 4 | - | Yes | 12, 22-23, 27-29 | 2 |
|  | u = 4 | Yes | - | 33-34, 38-39, 43-45 | 2 |
|  | u = 5 | Yes | - | 34, 39 | 2,3,5 |
|  | u = 5 | - | Yes | 44-45 | 2,4 |
|  | u = 4 | Yes | - | 30-39 | 12 |
|  | u = 4 | - | Yes | 40-45 | 12 |
|  | u = 5 | Yes | - | 30-39 | 5, 12 |
|  | u = 5 | - | Yes | 40-45 | 5, 12 |
|  | u = 6 | - | Yes | 45 | 2,4,5 |
| Down + up, Dynamic | d+u = 2-5, u < 3 | Yes | - | 1-12, 19-45 | 10 |
|  | d+u = 6, u<3 | Yes | - | 30-45 | 2,3 |
|  | d+u = 7, u<3 | - | Yes | 40-45 | 2,4 |
|  | d = 2, u = 3 | Yes | - | 32-34, 42-45 | 9 |
|  | d+u = 5, u = 2 – 3 | - | Yes | 12,36-39 | 9 |
|  | d+u = 6, u = 3-4 | Yes | - | 32-34,37-39,42-45 | 2,3,9 |
|  | d+u = 7, u = 3-4 | - | Yes | 42-45 | 2,4,9 |
|  | d = 4, u = 4 | Yes | - | 33-34,38-39,43-45 | 2,3,8,9 |
|  | d = 4, u = 5 | - | Yes | 44-45 | 2,4,8,9 |
|  | d+u = 8-10, u<3 | Yes | - | 30-45 | 12 |
| Down + up, Ext. Dynamic | d+u = 2-4 | Yes | - | 1-12, 19-45 |  |
|  | d+u = 5, d > 1 | Yes | - | 8-12, 19-45 |  |
|  | d+u = 6-7, u<4 | Yes | - | 10-12 | 8 |
|  | d = 1, u = 4 | - | Yes | 12, 22-23, 27-29 | 2 |
|  | d>1, u = 4 | - | Yes | 12 | 2,8 |
|  | d = 1, u = 4 | Yes | - | 33-34, 38-39, 43-45 | 2,6 |
|  | d+u = 6, d>1 | Yes | - | 30-45 | 2,3 |
|  | d = 1, u = 5 | Yes | - | 34,39 | 2,3,5 |
|  | d+u = 7-9, u<5 | Yes | - | 31-34, 36-39 | 2,3,8 |
|  | d=2-5, u = 5 | Yes | - | 34,39 | 2,3,5,8 |
|  | d = 1, u = 5 | - | Yes | 44-45 | 2,4 |
|  | d+u = 7, d>1 | - | Yes | 40-45 | 2,4 |
|  | d = 1, u = 6 | - | Yes | 45 | 2,4,5 |
|  | d+u = 8-11, u<6 | - | Yes | 41-45 | 2,4,8 |
|  | d=2-6, u = 6 | - | Yes | 45 | 2,4,5,8 |
|  | d=6-8, u=1-4 | Yes | - | 30-39 | 12 |
|  | d=6-8, u=5 | Yes | - | 34,39 | 5,12 |
|  | d=7-8, u=1-4 | - | Yes | 40-45 | 12 |
|  | d=7-8, u=5-6 | - | Yes | 44-45 | 5,12 |
| Note 0 If the downlink timeslots assigned (allocated) to the mobile station are not contiguous, d shall also include the number of downlink timeslots not assigned (allocated) to the mobile station that are located between assigned (allocated) downlink timeslots. Similarly, if the uplink timeslots assigned (allocated) to the mobile station are not contiguous, u shall also include the number of uplink timeslots not assigned (allocated) to the mobile station that are located between assigned (allocated) uplink timeslots. Note 1 Normal measurements are not possible (see 3GPP TS 45.008). Note 2 Normal BSIC decoding is not possible (see 3GPP TS 45.008) except e.g. in case of a downlink dual carrier capable MS operating in single carrier mode using its second receiver for BSIC decoding. Note 3 TA offset required for multislot classes 35-39. Note 4 TA offset required for multislot classes 40-45. Note 5 Shifted USF operation shall apply (see 3GPP TS 44.060). Note 6 The network may fallback to a lower multislot class and may not apply Tra. A multislot class 38 or 39 MS shall in this case use Tta for timing advance values below 31. Note 7 For dual carrier operation the Applicable Multislot class is the Signalled multislot class or the Equivalent multislot class (if different from the Signalled multislot class) as defined in Table B.2. For EFTA operation the Applicable Multislot class is the Signalled multislot class. Note 8 These configurations can only be used for assignment to an MS supporting Flexible Timeslot Assignment (see 3GPP TS 24.008). For allocation additional restrictions apply. Note 9 These configurations can be used only in RTTI configuration. Note 10 These configurations can be used in RTTI configurations only when the timeslots of the corresponding downlink PDCH-pair are contiguous. Note 11 These configurations can be used only in RTTI configurations when the timeslots of the corresponding downlink PDCH-pair are not contiguous. Note 12 These configurations can only be used for assignment to an MS for which Enhanced Flexible Timeslot Assignment with extended receive capability is used (see Annex B.5 and 3GPP TS 44.060). Whether normal measurements (see 3GPP TS 45.008) and/or normal BSIC decoding (see 3GPP TS 45.008) are possible will be dependent on the allocation or on the use of a second receiver for this purpose. |  |  |  |  |  |

For multislot class type 2 MS, all assignments according to its multislot class are possible independent of the MAC mode.

The occupied physical channels shall consist of a combination of configurations xi, xii and xiii as defined in subclause 6.4.1. For COMPACT, the occupied physical channels shall consist of a combination of configurations xiii), xxii), and xxiii), as defined in subclause 6.4.1.

The network shall leave a gap of at least one radio block period between the old and the new configuration, when the assignment is changed and PDCHs with the lowest numbered timeslot are not the same in the old and new configuration. For multislot class type 1 MS, the gap shall be left in both uplink and downlink when the lowest numbered timeslot for the combined uplink and downlink configuration is changed. For multislot class type 2 MS, the gap shall be left in the link (uplink and/or downlink) where the lowest numbered timeslot has been changed.

2nd modification

# B.1	MS classes for multislot capability

When an MS supports the use of multiple timeslots it shall belong to a multislot class as defined below:

Table B.1

| Multislot class | Maximum number of slots |  |  | Minimum number of slots |  |  |  | Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Rx | Tx | Sum | Tta | Ttb | Tra | Trb |  |  |  |
| 1 | 1 | 1 | 2 | 3 | 2 | 4 | 2 |  | 1 |  |
| 2 | 2 | 1 | 3 | 3 | 2 | 3 | 1 |  | 1 |  |
| 3 | 2 | 2 | 3 | 3 | 2 | 3 | 1 |  | 1 |  |
| 4 | 3 | 1 | 4 | 3 | 1 | 3 | 1 |  | 1 |  |
| 5 | 2 | 2 | 4 | 3 | 1 | 3 | 1 |  | 1 |  |
| 6 | 3 | 2 | 4 | 3 | 1 | 3 | 1 |  | 1 |  |
| 7 | 3 | 3 | 4 | 3 | 1 | 3 | 1 |  | 1 |  |
| 8 | 4 | 1 | 5 | 3 | 1 | 2 | 1 |  | 1 |  |
| 9 | 3 | 2 | 5 | 3 | 1 | 2 | 1 |  | 1 |  |
| 10 | 4 | 2 | 5 | 3 | 1 | 2 | 1 |  | 1 |  |
| 11 | 4 | 3 | 5 | 3 | 1 | 2 | 1 |  | 1 |  |
| 12 | 4 | 4 | 5 | 2 | 1 | 2 | 1 |  | 1 |  |
| 13 | 3 | 3 | NA | NA | a) | 3 | a) |  | 2 |  |
| 14 | 4 | 4 | NA | NA | a) | 3 | a) |  | 2 |  |
| 15 | 5 | 5 | NA | NA | a) | 3 | a) |  | 2 |  |
| 16 | 6 | 6 | NA | NA | a) | 2 | a) |  | 2 |  |
| 17 | 7 | 7 | NA | NA | a) | 1 | 0 |  | 2 |  |
| 18 | 8 | 8 | NA | NA | 0 | 0 | 0 |  | 2 |  |
| 19 | 6 | 2 | NA | 3 | b) | 2 | c) |  | 1 |  |
| 20 | 6 | 3 | NA | 3 | b) | 2 | c) |  | 1 |  |
| 21 | 6 | 4 | NA | 3 | b) | 2 | c) |  | 1 |  |
| 22 | 6 | 4 | NA | 2 | b) | 2 | c) |  | 1 |  |
| 23 | 6 | 6 | NA | 2 | b) | 2 | c) |  | 1 |  |
| 24 | 8 | 2 | NA | 3 | b) | 2 | d) |  | 1 |  |
| 25 | 8 | 3 | NA | 3 | b) | 2 | d) |  | 1 |  |
| 26 | 8 | 4 | NA | 3 | b) | 2 | d) |  | 1 |  |
| 27 | 8 | 4 | NA | 2 | b) | 2 | d) |  | 1 |  |
| 28 | 8 | 6 | NA | 2 | b) | 2 | d) |  | 1 |  |
| 29 | 8 | 8 | NA | 2 | b) | 2 | d) |  | 1 |  |
| 30 | 5 | 1 | 6 | 2 | 1 | 1 | 1 |  | 1 |  |
| 31 | 5 | 2 | 6 | 2 | 1 | 1 | 1 |  | 1 |  |
| 32 | 5 | 3 | 6 | 2 | 1 | 1 | 1 |  | 1 |  |
| 33 | 5 | 4 | 6 | 2 | 1 | 1 | 1 |  | 1 |  |
| 34 | 5 | 5 | 6 | 2 | 1 | 1 | 1 |  | 1 |  |
| 35 | 5 | 1 | 6 | 2 | 1 | 1+to | 1 |  | 1 |  |
| 36 | 5 | 2 | 6 | 2 | 1 | 1+to | 1 |  | 1 |  |
| 37 | 5 | 3 | 6 | 2 | 1 | 1+to | 1 |  | 1 |  |
| 38 | 5 | 4 | 6 | 2 | 1 | 1+to | 1 |  | 1 |  |
| 39 | 5 | 5 | 6 | 2 | 1 | 1+to | 1 |  | 1 |  |
| 40 | 6 | 1 | 7 | 1 | 1 | 1 | to |  | 1 |  |
| 41 | 6 | 2 | 7 | 1 | 1 | 1 | to |  | 1 |  |
| 42 | 6 | 3 | 7 | 1 | 1 | 1 | to |  | 1 |  |
| 43 | 6 | 4 | 7 | 1 | 1 | 1 | to |  | 1 |  |
| 44 | 6 | 5 | 7 | 1 | 1 | 1 | to |  | 1 |  |
| 45 | 6 | 6 | 7 | 1 | 1 | 1 | to |  | 1 |  |

a)	= 1 with frequency hopping.

= 0 without frequency hopping.

b)	= 1 with frequency hopping or change from Rx to Tx.

= 0 without frequency hopping and no change from Rx to Tx.

c)	= 1 with frequency hopping or change from Tx to Rx.

= 0 without frequency hopping and no change from Tx to Rx.

d)	= 1 change from Tx to Rx, or, with frequency hopping and the MS is either assigned a downlink dual carrier or DLMC configuration, or, the MS does not support fast downlink frequency switching (see 3GPP TS 24.008).

= 0 no change from Tx to Rx and either without frequency hopping or the MS is not assigned a downlink dual carrier configuration and supports fast downlink frequency switching (see 3GPP TS 24.008).

to	= 31 symbol periods (this can be provided by a TA offset, i.e. a minimum TA value).

Type 1 MS are not required to transmit and receive at the same time.

Type 2 MS are required to be able to transmit and receive at the same time.

For HSCSD, only multislot classes 1 - 18 are recognised. An MS with a higher multislot class number shall indicate a suitable multislot class less than 19 for HSCSD applications (see 3GPP TS 44.018).

EC-EGPRS operation is only defined for Type 1 MS, with only ‘Rx’ and ‘Tx’ applicable in Table B.1 for the respective multislot class. I.e. ‘Sum’, ‘Tra’, ‘Trb’, ‘Tta’, ‘Ttb’ do not apply.

Rx:

Rx describes the maximum number of receive timeslots that the MS can use per TDMA frame. The MS must be able to support all integer values of receive TS from 0 to Rx (depending on the services supported by the MS). The receive TS need not be contiguous. For type 1 MS, the receive TS shall be assigned within window of size Rx, and no transmit TS shall occur between receive TS within a TDMA frame.

Tx:

Tx describes the maximum number of transmit timeslots that the MS can use per TDMA frame. The MS must be able to support all integer values of transmit TS from 0 to Tx (depending on the services supported by the MS). The transmit TS need not be contiguous. For type 1 MS, the transmit TS shall be assigned within window of size Tx, and no receive TS shall occur between transmit TS within a TDMA frame.

Sum:

Sum is the total number of uplink (u) and downlink (d) TS that can actually be used by the MS per TDMA frame. The MS must be able to support all combinations of integer values of d <= Rx and u <= Tx TS where 1 <= d + u <= Sum (depending on the services supported by the MS). Sum is not applicable to all classes.

Tta:

Tta relates to the time needed for the MS to perform adjacent cell signal level measurement and get ready to transmit.

For type 1 MS it is the minimum number of timeslots that will be allowed between the end of the previous transmit or receive TS and the next transmit TS when measurement is to be performed between. It should be noted that, in practice, the minimum time allowed may be reduced by amount of timing advance.

For type 1 MS that supports extended TA, the parameter Tta is increased by 1 if TA > 63 and there is a change from RX to TX.

For type 2 MS it is not applicable.

For circuit switched multislot configurations as defined in subclause 6.4.2.1, Tta is not applicable.

Ttb:

Ttb relates to the time needed for the MS to get ready to transmit. This minimum requirement will only be used when adjacent cell power measurements are not required by the service selected.

For type 1 MS it is the minimum number of timeslots that will be allowed between the end of the last previous receive TS and the first next transmit TS or between the previous transmit TS and the next transmit TS when the frequency is changed in between. It should be noted that, in practice, the minimum time allowed may be reduced by the amount of the timing advance.

For type 1 MS that supports extended TA, the parameter Ttb = 2 if TA > 63 and there is a change from RX to TX.

For type 2 MS it is the minimum number of timeslots that will be allowed between the end of the last transmit burst in a TDMA frame and the first transmit burst in the next TDMA frame.

Tra:

Tra relates to the time needed for the MS to perform adjacent cell signal level measurement and get ready to receive.

For type 1 MS it is the minimum number of timeslots that will be allowed between the previous transmit or receive TS and the next receive TS when measurement is to be performed between.

For type 2 MS it is the minimum number of timeslots that will be allowed between the end of the last receive burst in a TDMA frame and the first receive burst in the next TDMA frame.

An MS, except for multislot class 30 – 45, shall be able to decode SCH from a neighbour cell, independent of its relative timing, using an idle frame in combination with Tra from the preceding frame.

Trb:

Trb relates to the time needed for the MS to get ready to receive. This minimum requirement will only be used when adjacent cell power measurements are not required by the service selected.

For type 1 MS it is the minimum number of timeslots that will be allowed between the previous transmit TS and the next receive TS or between the previous receive TS and the next receive TS when the frequency is changed in between.

For type 2 MS it is the minimum number of timeslots that will be allowed between the end of the last receive burst in a TDMA frame and the first receive burst in the next TDMA frame.

End of modifications