3GPP TSG-GERAN Ad Hoc on EC-GSM and eDRX 	GPE150036

# Stockholm, Sweden, 12th-13th of October 2015

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 45.004 | CR | 0020 | rev | - | Current version: | 12.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EC-EGPRS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | G1 |  |  |  |  |  |  |  |  |  |
| Work item code: | CIoT_EC_GSM |  |  |  |  |  |  | Date: |  | 2015-10-06 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | To introduce EC-EGPRS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of EC-EGPRS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EC-EGPRS will not be introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1.1, 2.6, 2.7, 3, 3.6, 3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  | TS 43.013 0135 TS 43.022 0035 TS 43.064 CR 0089,0090,0091,0092 TS 44.018 CR 1025,1026 TS 44.060 CR 1609,1610,1611,1612 TS 45.001 CR 0079,0080 TS 45.002 CR 0183,0184,0185,0186 TS 45.003 CR 0135 TS 45.005 CR 0578 TS 45.008 CR 0626,0627 |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 51.021 TS 51.010 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** First Change ***

# 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 45.001: “Physical Layer on the Radio Path (General Description)”.

[3]	3GPP TS 45.002: “Multiplexing and multiple access on the radio path”.

[4]	3GPP TS 45.005: “Radio transmission and reception”.

[5]	3GPP TS 45.010: “Radio subsystem synchronization”.

[6]	3GPP TS 44.060: “Radio Link Control/ Medium Access Control (RLC/MAC) protocol”.

[7]	3GPP TS 43.064: “General Packet Radio Service (GPRS)”.

[8]	3GPP TS 45.003: “Channel Coding”.

***Next Change ***

## 2.6	Modulation

The modulated RF carrier, except for start and stop of the TDMA burst may therefore be expressed as:

where Ec is the energy per modulating bit, f0 is the centre frequency and 0 is a random phase and is constant during one burst.

For EC-EGPRS, see 3GPP TS 43.064, 0 for all blind physical layer repetitions of the same burst, see 3GPP TS 45.003, shall be the same as 0 for the first transmission of that burst.

***Next Change ***

## 2.7	Overlaid CDMA

In the case of OLCDMA for EC-EGPRS the modulated RF carrier may be expressed with an additional term representing the OLCDMA code:

where OC equals 0 or 1, in accordance to the applied OLCDMA code, see 3GPP TS 45.002.

***Next Change ***

## 3.6	Modulation

The modulated RF carrier during the useful part of the burst is therefore:

where Es is the energy per modulating symbol, f0 is the centre frequency and 0 is a random phase and is constant during one burst.

For EC-EGPRS, see 3GPP TS 43.064, 0 for all blind physical layer repetitions of the same burst, see 3GPP TS 45.003, shall be the same as 0 for the first transmission of that burst.

***Next Change ***

## 3.7	Overlaid CDMA

In the case of OLCDMA for EC-EGPRS the modulated RF carrier may be expressed with an additional term representing the OLCDMA code:

where OC equals 0 or 1, in accordance to the applied OLCDMA code, see 3GPP TS 45.002.