Short Sync Procedure for PEO and EC-EGPRS Devices

# Introduction

A device that supports the PEO/EC-EGPRS feature and is in a cell that supports the PEO/EC-EGPRS feature will, after successfully negotiating the use of eDRX/PSM with the network using NAS signaling, use new idle mode procedures that remove the current requirements for periodically updating neighbor cell measurements. Such a device will instead only be required to be synchronized at the time of communication with the network.

# Reduced set of idle mode procedures/tasks (PEO and EC-EGPRS)

For some PEO and EC-EGPRS device implementations new idle mode procedures can involve what is called a ‘short sync procedure’. The procedure is ‘short’ in the sense that in most cases it can be limited to performing frequency and time domain correction tasks which are necessary due to oscillator drift that occurs when a device is in deep sleep. The procedure can be implemented periodically, or may only be performed shortly before a reception or transmission event.

The intention is that when a device wakes from its power saving state (eDRX or PSM) to perform the short sync procedure it will read, for devices using PEO, the FCCH and SCH to perform short sync whereas a device that uses the EC-EGPRS feature reads the FCCH and EC-SCH to perform short sync. The specific methods a PEO/EC-EGPRS device uses for reading FCCH/SCH/EC-SCH is considered to be implementation specific. Other tasks that could be performed either during or independent of the short sync procedure are BSIC verification (using the SCH or EC-SCH or confirming the TSC using a normal burst) and evaluation of the C3 criterion (see section 3) for ensuring suitability of the serving cell.

- The specific method used for determining how often BSIC verification and C3 criterion evaluation is performed is implementation specific.

- Due to the reduced mobility expected for a device that uses the PEO/EC-EGPRS feature the point(s) in time when BSIC verification and C3 criterion evaluations are performed can be independent of the points in time when the short sync procedure is performed i.e. the frequency of performing the short sync procedure may be tied to oscillator drift whereas a decision to perform BSIC verification and C3 criterion evaluation can be considered as independent of oscillator drift.

- A device that uses the PEO/EC-EGPRS feature with eDRX should perform BSIC verification and C3 evaluation sufficiently in advance of each PCH/EC-PCH reception event so that, in the event that cell reselection becomes necessary, there will be enough time to complete the cell reselection prior to that reception or transmission event. However, BSIC verification and C3 criterion evaluation does not need to be performed more often than once per minute.

- A device that uses the PEO/EC-EGPRS feature with PSM shall perform BSIC verification and C3 criterion evaluation whenever it wakes up to perform an uplink data transmission.

- Synchronizing to a non-serving cell BCCH carrier, reading the BCCH data thereon and evaluation of its C2 criterion is only required when the serving cell BCCH carrier is no longer considered to be suitable.

- The maximum time allowed for synchronization to a BCCH carrier and the maximum time allowed to read the BCCH data thereon is relaxed (compared to legacy operation) for the case of EC-EGPRS devices that are in CC2 or higher.

# Enhanced Cell Re-selection (PEO and EC-EGPRS)

A new parameter called “C1_OFFSET” can be sent in SI13 or SI2n to modify the legacy C1 pathloss criterion used by a PEO/EC-EGPRS device for determining the suitability of its serving cell. This parameter is provides the following benefits:

- It takes into account the possibility that the evaluation of C1 according to legacy procedures may be oriented towards CS domain operation.

- Cells that support EC-EGPRS may set this parameter to reflect the range of extended coverage that they support.

For cells that support PEO the range of C1_OFFSET is [-15, -14…-1, 0, 1, 2…15] where negative values allow the pathloss criteria to fail sooner (the device leaves the cell earlier) whereas positive values allow the pathloss criteria to fail later (the device hangs onto the cell longer). If C1_OFFSET is not included in SI then the device assumes C1_OFFSET = 0 and simply uses the legacy pathloss criterion C1 > 0 to determine the suitability of the serving cell.

For cells that support EC-EGPRS the range of C1_OFFSET is [-15, -14…-1, 0, 1, 2…15, 16, …35] which allows an extra 20 dB margin for devices in the worst coverage class to still consider the serving cell as suitable.

Taking into account C1_OFFSET, the new pathloss criteria for the serving cell consists of evaluating C3 where:

- For PEO and EC-EGPRS Devices:

  - If C3 = C1 + C1_OFFSET > 0 then the serving cell is considered suitable

  - If C3 = C1 + C1_OFFSET ≤ 0 then the serving cell might not be suitable and neighbor cell measurements, as per legacy procedure, are triggered with possible cell reselection.

- In order to evaluate the C1 criterion a device needs to perform at least 5 RSSI measurements within a time interval no less than 5 seconds long to establish a running average = RLA_C.

C1 = (A - Max(B,0))

where

A = RLA_C - RXLEV_ACCESS_MIN
B = MS_TXPWR_MAX_CCH - P

- The specific method used by a PEO/EC-EGPRS device to perform the RSSI measurements is implementation specific where the only requirement is that they be made within a time interval no less than 5 seconds long and be evenly spaced.

As an additional safety measure, a PEO/EC-EGPRS device can check for a relative reduction in C3 when considering any two successive C3 evaluations. A new parameter called C3_DELTA (sent in SI13) can specify a threshold value to use for determining whether the change in two successive evaluations of C3 is large enough to cause the device to trigger neighbor cell measurements and a possible cell re-selection as follows:

- C3T1 – C3T2 > C3_DELTA then neighbor cell measurements  are triggered even if C3 > 0

In addition, an EC-EGPRS device that detects a change (deterioration) of either its downlink or uplink coverage class may choose to respond (based on implementation) by triggering neighbor cell measurements in an attempt to determine if a better cell is available.

# Detecting Changes to System Information (PEO)

A device that has enabled PEO shall have read the SYSTEM INFORMATION TYPE 13 message no longer than 24 hours prior to attempting any given packet access to determine the status of the BCCH_CHANGE_MARK field therein.

- In addition, if the PEO BCCH CHANGE MARK field of a received PCH/AGCH message indicates a change of system information broadcast in the cell then the device shall re-acquire the SYSTEM INFORMATION TYPE 13 message.

- To help avoid the case where a change in PEO BCCH CHANGE MARK is detected when a device reads its paging subchannel it may acquire an AGCH/PCH block (for the purpose of checking PEO BCCH CHANGE MARK) before the next instance of its paging subchannel.

- A PEO device that is subject to implicit reject will also check the value of the PEO BCCH CHANGE MARK field in the AGCH/PCH block that is acquired prior to attempting system access. If a change of PEO BCCH CHANGE MARK is detected then it reads SI13 regardless if the implicit reject flag indicates its system access is denied or not.

- A PEO device that is not subject to implicit reject will still acquire an AGCH/PCH block prior to attempting system access but only for the purpose of reading the PEO BCCH CHANGE MARK field. If a change of PEO BCCH CHANGE MARK is detected then it reads SI13 prior to performing the system access.

- Considering the time spanned by the longest eDRX cycle, changes to the PEO BCCH CHANGE MARK field should not occur more frequently than 3 times within a 52 minute time period

# Detecting Changes to System Information (EC-EGPRS)

A MS that has enabled EC-EGPRS shall have read the EC-SI1 message no longer than 24 hours prior to attempting any given packet access to determine to determine the status of the BCCH_CHANGE_MARK field therein.

- In addition, if the mobile station detects a change to the EC-EGPRS BCCH CHANGE MARK field when reading the EC-SCH then it shall also re-acquire EC-SI1 (see sub-clause 3.5.2a).

- To help avoid the case where a MS detects a change in EC-EGPRS BCCH CHANGE MARK when waking up to read its paging subchannel it may acquire EC-SCH before the next instance of its paging subchannel.

- An EC-EGPRS device that is subject to implicit reject will examine the EC-EGPRS BCCH CHANGE MARK field sent as part of the EC-SCH prior to attempting system access.  If a change of EC-EGPRS BCCH CHANGE MARK is detected it shall first read the EC-SI message set regardless if the implicit reject flag indicates its system access is denied or not.

- An EC-EGPRS device that is not subject to implicit reject will still examine the EC-EGPRS BCCH CHANGE MARK field sent as part of the EC-SCH acquire an AGCH/PCH block prior to attempting system access for the purpose of reading the EC-EGPRS BCCH CHANGE MARK field. If a change of EC-EGPRS BCCH CHANGE MARK is detected then it reads the EC-SI message set prior to performing the system access.

- Considering the time spanned by the longest eDRX cycle, changes to the EC-EGPRS BCCH CHANGE MARK field should not occur more frequently than 3 times within a 52 minute time period.

# Conclusion:

CRs produced in support of the PEO and EC-EGPRS feature are targeting the introduction of device operation as described above.