3GPP TSG GERAN#57	GP-130206

Vienna, Austria	Agenda Item 7.2.5.3.5

February 25th – March 1st, 2013

Source: Renesas Mobile Europe Ltd

# On MFBI and EARFCN extension

# Introduction

Two LSs have been received from RAN2; one on EARFCN extension [1] and one on MFBI for overlapping (E-)UTRAN bands [2]. A discussion paper [3] also addresses the MFBI topic with two alternatives to either do nothing, or follow along the lines of what RAN2 did.

This discussion paper addresses GERAN considerations for the above and proposes a way forward.

NOTE: MFBI and EARFCN extension are not linked per se.

# EARFCN Extension

[1] indicates that the number spaces of value ranges of both the band number and the EARFCN have been increased by a factor of 4 i.e. from 64 to 256 for the band number and from 65535 to 262143 for the EARFCN.

These changes could not be introduced in Release 8, but could only be introduced:

- In Release 9 for E-UTRAN

- In Release 11 for UTRAN, in a release independent manner

It should be noted that no Frequency Band Indicator is used in GERAN for E-UTRA hence the impact on band number is transparent to GERAN.

The EARFCN defined in [4] identifies a carrier frequency in uplink and downlink as depicted in the table below where NDL indicates the EARFCN of DL carriers and NUL the EARFCN of UL uplink carriers. The yellow shading points to FDD bands (UL and DL spectrums divided in frequency domain i.e. duplex spacing) and the blue shading to TDD bands (same spectrum for UL and DL, divided in time domain) – while FDD UL/DL requires 2x EARFCNs, TDD UL/DL requires 1x EARFCN. As can be seen, an EARFCN is uniquely associated with a carrier frequency and a given band.

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 600 – 1199 | 1850 | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4949 | 1427.9 | 22750 | 22750 – 22949 |
| 12 | 729 | 5010 | 5010 - 5179 | 699 | 23010 | 23010 - 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 | 23280 | 23280 – 23379 |
| … |  |  |  |  |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 – 6449 | 832 | 24150 | 24150 – 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 – 7399 | 3410 | 24600 | 24600 – 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 - 8039 | 1626.5 | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 - 8689 | 1850 | 26040 | 26040 – 26689 |
| 26 | 859 | 8690 | 8690 - 9039 | 814 | 26690 | 26690 – 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 | 27040 | 27040 – 27209 |
| 282 | 758 | 9210 | 9210 – 9659 | 703 | 27210 | 27210 – 27659 |
| 292 | 717 | 9660 | 9660 – 9769 | N/A |  |  |
| … |  |  |  |  |  |  |
| 33 | 1900 | 36000 | 36000 – 36199 | 1900 | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 | 2010 | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 | 1850 | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 | 1930 | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 | 1910 | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 | 2570 | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 | 1880 | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 | 2300 | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 –41589 | 2496 | 39650 | 39650 –41589 |
| 42 | 3400 | 41590 | 41590 – 43589 | 3400 | 41590 | 41590 – 43589 |
| 43 | 3600 | 43590 | 43590 – 45589 | 3600 | 43590 | 43590 – 45589 |
| 44 | 703 | 45590 | 45590 – 46589 | 703 | 45590 | 45590 – 46589 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. |  |  |  |  |  |  |

Table 1. E-UTRAN Channel Numbers [4]

In GERAN specifications, only the EARFCN of DL carriers need to be identified for it is these carriers that are measured for mobility purpose. As a result, half of the FDD EARFCNs address space is unused in GERAN; the NUL range is free for use. Also it is worth noting that the range 46590-65535 is yet to be used today.

As a way forward in GERAN, it is then proposed to not extend the EARFCN addressing space but instead, whenever news bands are introduced, to use a local EARFCN mapping in GERAN networks and MSs that can exploit the unused FDD uplink EARFCNs (NUL). This approach avoids any signalling change in GERAN, whilst allowing the future introduction of new EUTRA bands in a true release-independent manner.

# Multiple Frequency Band Indicators

[2] indicates a MS that supports a band which overlaps with other bands is allowed to roam to those overlapping bands. The table below illustrates for LTE the example of band 26 overlapping with bands 5, 18 and 19 which for instance allows a MS supporting band 26 only to camp onto any of bands 5, 18 or 19 (there are other possibilities as well).

| Bands | Uplink (MHz) |  |  |  |  |  |  | Downlink (MHz) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band 26 | 814 |  |  |  |  | 849 |  | 859 |  |  |  |  | 894 |
| Band 18 |  | 815 |  | 830 |  |  |  |  | 860 |  | 875 |  |  |
| Band 5 |  |  | 824 |  |  | 849 |  |  |  | 869 |  |  | 894 |
| Band 19 |  |  |  | 830 | 845 |  |  |  |  |  | 875 | 890 |  |

Table 2. Example of band overlap (E-UTRA FDD Bands 26 with bands 5, 18 and 19)

As already highlighted in §2, only DL [E/U]ARFCN are applicable for mobility purpose. As can be seen in Table 1 above for EARFCNs, and in the tables below for UARFCNs, an [E/U]ARFCN is uniquely associated with a given carrier frequency and a given band. The same carrier frequency in overlapping bands needs thus as many [E/U]ARFCNs as there are overlapping bands. Assuming overlapping Bands M and N, a MS supporting Band M only (resp. Band N only) would normally not understand EARFCNs of Band N (resp. Band M). However the overlap implies that such MS could camp on a non-supported band.

RAN2 decided to introduce an MFBI list for each [E/U]ARFCN, as it was thought to be more signalling efficient than expanding the number of [E/U]ARFCNs that can be signalled. This list allows a MS not understanding an EARFCN to check the MFBI list for supported band(s) and perform an EARFCN translation accordingly. As rightly identified in [3] a similar approach in GERAN could yield severe signalling impact that may be considered unjustified. It should also be noted that network sharing aspects under discussion in FULL_MOCN_GERAN are not accounted for in [3] – thus the impact could be greater than anticipated in [3].

Without signalling an MFBI list per [E/U]ARFCN, extra [E/U]ARFCNs can be signalled within the limits set by the standard (e.g. 8 EARFCNs for EUTRA). It is assumed that these limits will not be changed in GERAN: the flexibility is, as for [E/U]TRAN, “limited” by the number of “legacy” [E/U]ARFCN that can be signalled.

A possible way forward is a MS-centric solution without any signalling changes whereby a MS supporting a given band could understand the [E/U]ARFCNs of the applicable carriers in the applicable overlapping bands when such exist any (as required by RAN2 solution already, a MS would not ignore an EARFCN in a band it does not support, so such functionality would already exist). The impact of this approach vs. an explicit MFBI list is the lack of a priori knowledge of the supported bands of neighboring (E-)UTRAN cells while in GERAN.

- In idle mode, or generally for autonomous cell reselection (i.e. including in packet transfer mode) this may initially yield an unnecessary failed reselection to a cell when the MS cannot support this cell (due to lack of MFBI support in the cell), however this is not deemed to be a significant problem for the band support (or lack thereof) is expected to be geographically ~static across wide areas (e.g. PLMN). A MS would also update its “MFBI”-related information whenever visiting an (E-)UTRA cell.

- In dedicated mode, dual transfer mode and packet transfer mode (PS Handover) the target cell decides whether or not to proceed with a handover, no real issue is foreseen from a service interruption standpoint. However, unnecessary measurements and reporting can be triggered given the BSS does not know which UTRA/E-UTRA bands are supported by the MS. This could be addressed, if necessary, either by some MFBI indication in Measurement Information / Packet Measurement Order or by some band support indication in CM3/MS RAC IE.

- In packet transfer mode (NC2 no PS Handover), cell reselection failure can occur, resulting in the MS sending a Packet Cell Change Failure back to the BSS in the old cell (with Cause ‘0000’ frequency not implemented) which allows the BSS to act accordingly. The MS could also behave in a similar way as for autonomous reselection (i.e. assume the band support to be static) to prevent further measurement and reporting of cells that are not supported.

|  | Uplink (UL) |  | Downlink (DL) |  |
| --- | --- | --- | --- | --- |
| Band | General | Additional | General | Additional |
| I | 9612 to 9888 | - | 10562 to 10838 | - |
| II | 9262 to 9538 | 12, 37, 62, 87, 112, 137, 162, 187, 212, 237, 262, 287 | 9662 to 9938 | 412, 437, 462, 487, 512, 537, 562, 587, 612, 637, 662, 687 |
| III | 937 to 1288 | - | 1162 to 1513 | - |
| IV | 1312 to 1513 | 1662, 1687, 1712, 1737, 1762, 1787, 1812, 1837, 1862 | 1537 to 1738 | 1887, 1912, 1937, 1962, 1987, 2012, 2037, 2062, 2087 |
| V | 4132 to 4233 | 782, 787, 807, 812, 837, 862 | 4357 to 4458 | 1007, 1012, 1032, 1037, 1062, 1087 |
| VI | 4162 to 4188 | 812, 837 | 4387 to 4413 | 1037, 1062 |
| VII | 2012 to 2338 | 2362, 2387, 2412, 2437, 2462, 2487, 2512, 2537, 2562, 2587, 2612, 2637, 2662, 2687 | 2237 to 2563 | 2587, 2612, 2637, 2662, 2687, 2712, 2737, 2762, 2787, 2812, 2837, 2862, 2887, 2912 |
| VIII | 2712 to 2863 | - | 2937 to 3088 | - |
| IX | 8762 to 8912 | - | 9237 to 9387 | - |
| X | 2887 to 3163 | 3187, 3212, 3237, 3262, 3287, 3312, 3337, 3362, 3387, 3412, 3437, 3462 | 3112 to 3388 | 3412, 3437, 3462, 3487, 3512, 3537, 3562, 3587, 3612, 3637, 3662, 3687 |
| XI | 3487 to 3562 | - | 3712 to 3787 | - |
| XII | 3617 to 3678 | 3707, 3732, 3737, 3762, 3767 | 3842 to 3903 | 3932, 3957, 3962, 3987, 3992 |
| XIII | 3792 to 3818 | 3842, 3867 | 4017 to 4043 | 4067, 4092 |
| XIV | 3892 to 3918 | 3942, 3967 | 4117 to 4143 | 4167, 4192 |
| XIX | 312 to 363 | 387, 412, 437 | 712 to 763 | 787, 812, 837 |
| XX | 4287 to 4413 | - | 4512 to 4638 | - |
| XXI | 462 to 512 | - | 862 to 912 | - |
| XXII | 4437 to 4813 | - | 4662 to 5038 | - |
| XXV | 4887 to 5188 | 6067, 6092, 6117, 6142, 6167, 6192, 6217, 6242, 6267, 6292, 6317, 6342, 6367 | 5112 to 5413 | 6292, 6317, 6342, 6367, 6392, 6417, 6442, 6467, 6492, 6517, 6542, 6567, 6592 |
| XXVI | 5537 to 5688 | 5712, 5737, 5762, 5767, 5787, 5792, 5812, 5817, 5837, 5842, 5862 | 5762 to 5913 | 5937, 5962, 5987, 5992, 6012, 6017, 6037, 6042, 6062, 6067, 6087 |

Table 3. UTRAN Channel Numbers (FDD) [5]

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission | Additional UARFCN Uplink and Downlink transmission |
| --- | --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9512 to 9588 10062 to 10113 | - |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9262 to 9538 9662 to 9938 | - |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9562 to 9638 | - |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12862 to 13088 | 2112, 2137, 2162, 2187, 2212, 2237, 2262, 2287, 2312, 2337 |

Table 4. UTRAN Channel Numbers (TDD – 3.84Mcps chip rate) [6]

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission |
| --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9504 to 9596 10054 to 10121 |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9254 to 9546 9654 to 9946 |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9554 to 9646 |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12854 to 13096 |
| For operation in frequency band as defined in subclause 5.2 (e) | 2300-2400 MHz | 11504 to 11996 |
| For operation in frequency band as defined in subclause 5.2 (f) | 1880-1920 MHz | 9404 to 9596 |

Table 5. UTRAN Channel Numbers (TDD – 1.28Mcps chip rate) [6]

| Frequency Band | Frequency Range | UARFCN Uplink and Downlink transmission | Additional UARFCN Uplink and Downlink transmission |
| --- | --- | --- | --- |
| For operation in frequency band as defined in subclause 5.2 (a) | 1900-1920 MHz 2010-2025 MHz | 9512 to 9588 10062 to 10113 | - |
| For operation in frequency band as defined in subclause 5.2 (b) | 1850-1910 MHz 1930-1990 MHz | 9262 to 9538 9662 to 9938 | - |
| For operation in frequency band as defined in subclause 5.2 (c) | 1910-1930 MHz | 9562 to 9638 | - |
| For operation in frequency band as defined in subclause 5.2 (d) | 2570-2620 MHz | 12874 to 13076 | - |

Table 6. UTRAN Channel Numbers (TDD – 7.68Mcps chip rate) [6]

# Conclusions

This paper has addressed the two LSs from RAN2 on EARFCN numbering space and on support of multiple frequency band indicators. It is proposed:

- to not extend the EARFCN addressing space but instead, whenever new bands are introduced, to use a local EARFCN mapping in GERAN networks and MSs that can exploit the unused FDD uplink EARFCNs. This approach avoids any signalling change in GERAN, whilst allowing the introduction of new EUTRA bands in a true release-independent manner.

- to not broadcast an MFBI list, but to assume a mobile station supporting given [E/U]TRAN bands would understand the [E/U]ARFCNs of applicable carriers in the unsupported overlapping bands if any.

  - If deemed necessary, unnecessary measurement and reporting could be avoided in dedicated mode, dual transfer mode and packet transfer mode (when handover is used) by including an MFBI indication in Measurement Information / Packet Measurement Order or by some band support indication in CM3/MS RAC IE.

# REFERENCES

- GP-130086, Reply LS on extending E-UTRA band number and EARFCN numbering space, RAN2

- GP-130088, LS on support of multiple frequency band indicators in GERAN, RAN2

- GP-130151, MFBI Impact on GERAN, Huawei

- 3GPP TS 36.101, E-UTRA: User Equipment (UE) radio transmission and reception

- 3GPP TS 25.101, UTRA: User Equipment (UE) radio transmission and reception (FDD)

- 3GPP TS 25.102, UTRA: User Equipment (UE) radio transmission and reception (TDD)

UL EARFCN are not deemed needed  (Duplex distance can be used and “RACH” configuration can be obtained once in the target cell)

Band 19 is for Japan, hence not applicable from GERAN interworking perspective but is shown here for illustration.

Note that outside Japan, and DL UARFCN of Band V and Band VI can be considered as being from Band V (inside Japan itself is not applicable otherwise from GERAN perspective). This is shaded in grey in Table 2.