Narrow Band Hybrid Modulation

for Cellular IoT

# Introduction

Machine type communication (MTC) i.e. Machine to Machine communication is a rapidly growing market segment for cellular technologies requiring extended coverage, long battery life, low device complexity and capacity for large number of connected devices. The extended coverage requirement is seen as the most important item for improvement with the target of up to 20 dB improvement over Rel99 GPRS and preferably without impact to existing BSS HW.

At GERAN#62 a new study item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was agreed ([1]). This includes the study of both non-backward compatible concepts (such as the clean slate proposal [2]) as well as backward compatible evolved GERAN concepts. This paper introduces a backward compatible candidate technique to improve coverage substantially without HW impact to BSS together with other benefits due to its evolutionary approach.

# options for increasing coverage studied so far

To meet the objectives set for the study item, there are already a number of techniques proposed in [2], [3] and [4]. The limitations of these candidate techniques were already highlighted in previous discussions. Use of message repetition can improve coverage, but, due to large number of repetitions in case of extended coverage scenario, battery consumption as well as transmission delay will increase significantly. The clean slate approach with reduced channel bandwidth would introduce incompatibility with current modulators fed by modulating bits, e.g. new symbols may need to be created as complex-valued samples instead.

# narrow band hybrid modulation concept

This paper proposes a narrow band hybrid modulation method that exploits existing modulators with a precoder to produce a signal with fractional bandwidth of symbol rate. The precoder produces sequences of N symbols, with each sequence having a single non-zero bin in frequency domain as an output of N-point FFT, resulting in a bandwidth of 1/N times the normal symbol rate. The narrow band signal, i.e. the sequence of normal symbols, may carry information on frequency offset and signal phase as FSK or BPSK respectively.

After the precoding, the signal is passed to the legacy GMSK or 8PSK modulator with legacy pulse shaping, as depicted in Figure 1.

Figure 1 Pre-coding for Narrow Band Hybrid Modulation

This precoding concept may be applied for new type of TCH and RACH to achieve efficient multiplexing with legacy GPRS devices and for new type of BCCH, CCCH and SCH dedicated for new MTC devices, e.g., in a dedicated timeslot (see sections 3.2 and 3.7).

## Precoding options

Precoding involves data bits to be mapped to a series of 8 GMSK or 8PSK symbols with total length of 8 x 3.69 = 29.52 µs. The data bits can be grouped into 1, 2 or 3 data bits depending on the hybrid modulation options. Four options are listed here.

### BPSK with precoded GMSK

| Data bits | Precoded Bits | Resulting phase | Resulting FSK tone |
| --- | --- | --- | --- |
| 0 | 00000000 | +1 | +68 kHzNote |
| 1 | 11111111 | -1 | +68 kHz |

Note: 68 kHz corresponds to the resulting GMSK symbol frequency (=270.833/4 kHz

### 2-FSK+BPSK with precoded GMSK

| Data bits | Precoded Bits | Resulting phase | Resulting FSK tone |
| --- | --- | --- | --- |
| 00 | 00000000 | +1 | +68 kHz |
| 01 | 11111111 | -1 | +68 kHz |
| 10 | 10101010 | +1 | -68 kHz |
| 11 | 01010101 | -1 | -68 kHz |

### 4-FSK with precoded 8PSK

In this case a hybrid modulation symbol is formed from a series of 8 8PSK symbols with symbol rotation of π/2 or 3π/8 radians per normal symbol period. Two data bits are mapped to one of the 4-FSK symbols by this rule

sk=1, ejπl/4,ejπ2l/4,ejπ3l/4,ejπ4l/4,ejπ5l/4,ejπ6l/4,ejπ7l/4 where sk identifies the series of 8 pre-coded symbols. The symbol parameter l is given in the table below.

|  | π/2 rotation |  | 3π/8 rotation |  |
| --- | --- | --- | --- | --- |
| Data bits | Symbol parameter l | Resulting 4-FSK tone | Symbol parameter l | Resulting 4-FSK tone |
| 00 | 0 | +68 kHz | 0 | +51 kHz |
| 01 | -1 | +34 kHz | -1 | +17 kHz |
| 10 | -3 | -34 kHz | -2 | -17 kHz |
| 11 | -4 | -68 kHz | -3 | -51 kHz |

### 4-FSK+BPSK with precoded 8PSK

With BPSK addition, 3 data bits can be used to generate the precoded sequences.

sk=(1-2m)1, ejπl/4,ejπ2l/4,ejπ3l/4,ejπ4l/4,ejπ5l/4,ejπ6l/4,ejπ7l/4 where sk identifies the series of 8 pre-coded symbols and m is the first bit of each group of 3 bits. Symbol parameter l is given in the table below.

| Data bits | First bit of the group m | Resulting phase | π/2 rotation |  | 3π/8 rotation |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Symbol parameter l | Resulting 4-FSK tone | Symbol parameter l | Resulting 4-FSK tone |
| 000 | 0 | 1 | 0 | +68 kHz | 0 | +51 kHz |
| 001 | 0 | 1 | -1 | +34 kHz | -1 | +17 kHz |
| 010 | 0 | 1 | -3 | -34 kHz | -2 | -17 kHz |
| 011 | 0 | 1 | -4 | -68 kHz | -3 | -51 kHz |
| 100 | 1 | -1 | 0 | +68 kHz | 0 | +51 kHz |
| 101 | 1 | -1 | -1 | +34 kHz | -1 | +17 kHz |
| 110 | 1 | -1 | -3 | -34 kHz | -2 | -17 kHz |
| 111 | 1 | -1 | -4 | -68 kHz | -3 | -51 kHz |

## Burst Formats

### Normal burst format

Narrow band sequences with length of 8 symbol periods can fit into current burst structure with minor modifications. Table 1 shows the length of different sections of a burst in normal symbol periods. The lengths are given in number of sequences for the new burst in parenthesis. Stealing flags and spare symbols may be used, e.g.,  for RLC/MAC related signalling.

Table 1. Burst format for normal burst

| Burst Content | Current normal burst | Hybrid modulation burst |
| --- | --- | --- |
| Front tail symbols | 3 | 3 |
| Payload symbols (first part) | 57 | 56 (7 NB sequences of 8 symbols) |
| Spare | - | 1 |
| Stealing flag/MCS indicator | 1 | 1 |
| Training Sequence | 26 | 26 |
| Stealing flag/MCS indicator | 1 | 1 |
| Spare | - | 1 |
| Payload symbols (second part) | 57 | 56 (7 NB sequences of 8 symbols) |
| Tail symbols | 3 | 3 |
| Guard period | 8.25 | 8.25 |

### Random access burst

Narrow band sequences with length of 8 symbol periods can fit into random access burst, if it is extended over 2 time slots. This could be achieved, if the new type of CCCH  slot is located to timeslot 1 next to existing CCCH timeslot.

At the same time length of the training sequence can be up to double size to allow better channel estimation. Optimal length of TSC is FFS. Thus, the new RACH could carry as many payload bits as exiting RACH, but is expected to have up to 9 dB better coverage than legacy RACH as alone and even more by using repetitions.

Table 2. Burst format for random access burst

| Burst Content | Current | Hybrid modulation burst (in 2 timeslots) |
| --- | --- | --- |
| Front tail symbols | 8 | 8 |
| Training Sequence | 41 | 41..89 |
| RACH payload symbols | 36 | 192..144 (24..18 NB sequences of 8 symbols)Note |
| Tail symbols | 3 | 3 |
| Guard period | 68.25 | 68.5 |

Note: It is expected that a single value is implemented, unless there is a good reason to implement an additional value.

### Synchronization burst

Coverage for synchronization burst may be improved with narrow band symbol sequences. With new CCCH structure as depcited in Figure 5 it is possible to carry 200 bits for synchronization in one 51 multiframe period.

Table 3. Burst format for synchronization burst

| Burst Content | Current normal burst | Hybrid modulation burst |
| --- | --- | --- |
| Front tail symbols | 3 | 3 |
| Payload symbols (first part) | 39 | 40 (5 NB sequences of 8 symbols) |
| Training Sequence | 64 | 62 |
| Payload symbols (second part) | 39 | 40 (5 NB sequences of 8 symbols) |
| Tail symbols | 3 | 3 |
| Guard period | 8.25 | 8.25 |

## Layer 1 Parameters for NB PDTCH

Layer 1 parameters for the four options mentioned above are summarised in Table 4 with possible improvement in sensitivity with CS1 of GPRS as a reference.

Impact to Layer 2 is minimised by using coding scheme (CS1´ and CS1’’) for pre-coded GMSK that is slightly modified from existing CS1 with puncturing of 4 additional bits per RLC/MAC block (if tail-biting is used during encoding) or 8 additional bits per RLC/MAC block (if tail-biting is not used). Indeed further modified CS1’’’ with 1/3 rate convolution code is used for pre-coded 8PSK with higher gross bit rate.

TTI is extended from current 20ms to 80ms or 160ms.

Table 4. Layer 1 parameters

| Parameter | GMSK | BPSK | 2-FSK+BPSK | 4-FSK | 4-FSK+BPSK |
| --- | --- | --- | --- | --- | --- |
| Modulator | GMSK | GMSK | GMSK | 8PSK | 8PSK |
| Duration of NB symbol sequence | N.A. | 8 x 3.69 = 29.52 µs | 29.52 µs | 29.52 µs | 29.52 µs |
| Bandwidth for NB symbol sequence | N.A. | 34 kHz | 34 kHz | 34 kHz | 34 kHz |
| NB symbol sequences per burst | N.A. | 14 | 14 | 14 | 14 |
| Number of m-FSK tones | N.A. | 1 | 2 | 4 | 4 |
| Keying of NB symbol sequences | - | BPSK | BPSK | - | BPSK |
| Data bits per NB symbol sequence | N.A. | 1 | 2 | 2 | 3 |
| Gross bit-rate (kbps) | 22.8 | 2.8 | 5.6 | 5.6 | 8.4 |
| TTI | 20 ms | 160 ms | 80 ms | 80 ms | 80 ms |
| Bits per TTI | 456 | 448 | 448 | 448 | 672 |
| Coding scheme | CS1 | CS1’ | CS1’’ | CS1’’ | CS’’’ |
| Block code | FIRE | FIRE | FIRE | FIRE | FIRE |
| Convolution code | 1/2 | 1/2 | 1/2 | 1/2 | 1/3 |
| Maximum number of transmissions with chase combining | N.A. | 4 | 8 | 8 | 8 |
| Effective code rate | 0.40 | 0.41 | 0.41 | 0.41 | 0.27 |
| Peak throughput | 8.0 kbpsNote | 1.0 - 0.25kbps | 2.0 – 0.25 kbps | 2.0 – 0.25 kbps | 2.0 – 0.25 kbps |
| Relative sensitivity at 10% BLER | 0 dB | FFS | FFS | FFS | FFS |
| Relative sensitivity at 200 bps with chase combining | 0 dB | FFS | FFS | FFS | FFS |

Note: Based on 3GPP TS 43.064 excluding RLC header and USF

Figure 2 illustrates an idealised example of spectrogram for the new burst format, where 7 4-FSK symbol sequences are located at each side of the training sequence.

Figure 2 Spectrogram (8 bin FFT) of a burst with 4-FSK modulation (Example)

## Transmitter Structure

An example of the transmitter block diagram is given in Figure 3, where existing functions are shown in light blue and the new function to support this feature is given in dark blue. The precoder performs precoding for payload bits and is located in between channel encoding and burst building.

Figure 3 Example of Transmitter Block Diagram for Narrow Band Hybrid Modulation concept

## Receiver Structure

An example of the receiver structure is given as an illustrative example in Figure 4, where existing functions are shown in light blue and new functions in dark blue.

Figure 4 Example of Receiver Block Diagram for Narrow Band Hybrid Modulation concept

The receiver uses normal equalisation for a burst in order to remove impact of ISI. Equalisation includes channel estimation, synchronisation and pre-filtering, but not bit detection. Separate FFT is performed over each received sequence of 8 symbols, extracted from the equalised burst, followed by detection of both m-FSK and BPSK from output bins of FFT.  Chase combining is performed before detection for cumulating selected FFT bins over defined number of repetitions.

## Chase Combining

Normal RLC/MAC repetition is proposed to be applied to new PDTCH schemes with or without chase combining. Use of chase combining may be based on pre-determined number of repetitions. Complexity is reasonable if only selected bins of complex FFT output are combined.

In this study maximum number of repetitions is determined so that 200bps target throughput is possible to achieve, with some residual BLER.  E.g. CS1’’ with 2.0 kbps peak throughput is repeated 8 times.

ARQ works in addition to the repetition, however, further study is needed to identify modifications in RLC window size due to the increase in TTI.

## CCCH Coverage Improvement with pre-coding

CCCH coverage including synchronization channel can be improved with an IoT dedicated CCCH time slot, applying the pre-coding technique. That new slot appears in TSL 1 right after current basic CCCH timeslot in TSL 0. The new 51 multi frame structure is depicted in Figure 5.

Multiplexing of NB BCCH (BCCH’) and NB CCCH (CCCH’) blocks takes 4 times longer with 2-FSK+BPSK pre-coding option. Existing FCH in TSL 0 is expected to be sufficient.

SCH coding is changed such that T1, T2 and T3’ refer to the first TDMA frame (0) of multi frame-51 and new MTC device then calculate frame numbers for other frames accordingly. This allows faster synchronization in coverage limited cases.

Figure 5 New CCCH timeslot 1 with preceeding legacy CCCH timeslot 0.

# link level Performance evaluation

## Simulation assumptions

Link level simulation assumptions are given in Table 5.

Table 5. Simulation assumptions

| Parameter | Value |
| --- | --- |
| Frequency bands | 900 |
| Interference profile | Sensitivity |
| Propagation conditions | TU |
| Mobile speed | 1.2 km/hr (corresponding to 1 Hz Doppler) |
| Frequency hopping | ideal |
| BTS RF impairments | Typical |
| Back-off (single transmit antenna) | 0 dB for GMSK 3.2 dB for 8-PSK downlink only |
| MS RF impairments | TBD |
| Antenna configuration | BTS: 1Tx, 2Rx MS: 1Tx, 1Rx |
| MCSs | Precoded GMSK CS1’ & CS1’’ Precoded 8-PSK CS1’’ & CS1’’’ CS1 |
| Chase combining | 1, 4 or 8 coding blocks depending on precoded MCS (>200bps) |

## Simulation results

[TBD]

## Link budget estimations

Simple link budget estimations are given in Table 6 based on required Es/N0 that is needed to meet e.g. throughput criteria of 200bps for the PS traffic channel. Sensitivity criteria for common control channels are FFS.

Table 6. Link budgets for NB Hybrid Modulation

| Channel with NB Hybrid Modulation | PDTCH DL CS1’ | PDTCH DL CS1’’ | B/CCCH | SCH | PDTCH UL CS1’ | PDTCH UL CS1’’ | RACH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Transmitter peak power (dBm) | 43 | 43 | 43 | 43 | 33 | 33 | 33 |
| Noise figure (dB) | 9 | 9 | 9 | 9 | 5 | 5 | 5 |
| Symbol Rate (kHz) | 271 | 271 | 271 | 271 | 271 | 271 | 271 |
| Noise level (dBm) | -111 | -111 | -111 | -111 | -115 | -115 | -115 |
| Sensitivity criteria | >200bps | >200bps | FFS | FFS | >200bps | >200bps | FFS |
| Chase combining | 4 | 8 | FFS | FFS | 4 | 8 | FFS |
| Diversity RX | No | No | No | No | Yes | Yes | Yes |
| Es/N0 at sensitivity level(dB)Note | -7 | -7 | FFS | FFS | -11 | -11 | FFS |
| Sensitivity (dBm) | -118 | -118 |  |  | -126 | -126 |  |
| MCL | 161 | 161 |  |  | 159 | 159 |  |

Note: Es/N0 at sensitivity levels are estimated figures assuming chase combining is applied along with narrow band hybrid modulation.

# conclusion

This paper has presented a narrow band hybrid modulation technique to improve the coverage of M2M service. Along with repetition and chase combining this evolutionary scheme is expected to meet the objective of coverage improvement with MCL of nearly 160dB for the PS traffic channel.

Therefore, it is proposed to include this mechanism in the Technical Report of the Cellular IoT study item.

# REFERENCEs.

- GP-140421 - New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Vodafone Group Plc, TSG-GERAN#62

- GP-140322 - Discussion on MTC Evolution for Cellular IoT, Huawei Technologies Co. Ltd, Hi Silicon, TSG-GERAN#62

- GP-140323 - Evaluations on Narrow-band M2M.doc, Huawei Technologies Co. Ltd, Hi Silicon, TSG-GERAN#62

- GP-140297 - GSM optimization for Internet of Things, Ericsson, TSG-GERAN#62