# NewToN – Performance evaluation

# Introduction

A new work item on New Training Sequences for GERAN, acronym NewToN, was approved at GERAN#60, see [1].

The work consists of defining new training sequences for both CS and PS services in GERAN with the aim to reduce the cross correlation between TSCs to primarily allow for a more spectral efficient implementation of synchronized GSM networks.

A performance evaluation framework has been agreed (see reference [2]) to be able to select among different TSC proposals. Currently, there is only one TSC proposal available (see reference [3]), but the framework can also be used to compare a TSC proposal to the legacy TSC sets. In Annex A, a relative performance comparison according to the framework is shown. The final metric (basically a weighted average of the performance with all possible combinations of TSCs for carrier and interferer in various scenarios) for the new TSC set is found to be 0.7 dB better than legacy when including TSC set 2 for GMSK, and 1.5 dB if only TSC set 1 from all modulations are considered.

Whereas this is an attractive improvement, it may not fully reflect the expected gains of NewToN. One important aspect of extending the set of training sequences is that it increases the possibilities of TSC planning so that under-performing TSC combinations can more easily be avoided.

In this contribution, the following aspects of using an extended TSC set are investigated:

- The benefits of extended TSC sets for TSC planning are investigated:

  - In Section 2, the impact of co-TSC interference – interference from an interferer with the same TSC as the wanted signal – is studied on link level based on system level statistics.

  - System level simulations using TSC planning with current and existing TSC sets are evaluated

- The benefit of extended TSCs sets according to the agreed performance framework is presented in Section 4

# Impact of co-TSC interference

## Introduction

Figure 1 illustrates an extreme example of the impact of co-TSC interference. An IRC receiver interfered by a single co-channel interferer has been simulated. The interferer is synchronized to the carrier but has a propagation delay according to the agreed propagation delay model for NewToN.

In the “Co-TSC” case, the interferer always has the same TSC as the carrier, whereas in the “Other TSC” case, the interferer TSC is randomly chosen from the other seven TSCs in GMSK set 1.

At 1% FER, the difference between the two curves is about 18 dB. Even though this is in an extreme scenario, it is obvious that co-TSC interference is very detrimental to IRC. Similar results (not shown here) have been noticed for a SAIC receiver.

Figure 1: FER vs C/I with single CCI.

## System model of co-TSC interference

To assess the impact of TSC planning to reduce co-TSC interference in real networks, the following approach has been taken:

- For a given network, TSC planning is performed seeking to avoid strong co-TSC interference. Two different TSC plans were derived using eight TSCs (corresponding to the legacy case without VAMOS) and 16 TSCs (corresponding e.g. to the case of extended TSC sets), respectively. The TSC planning algorithm is proprietary but should be seen to reflect a realistic TSC planning in the field.

- System simulations are run using the derived TSC plans to get statistics of interference levels and co-TSC probabilities.

- The statistics are used to build an interference model that is used in a link simulator to derive link performance impacts.

### Network configuration

The considered network is a tight reuse network with 100% speech users and with the network load placed at around 2% hard blocking. The configuration is summarized in Table 1.

Table 1: Parameters for the system simulations.

| Parameter | Value |
| --- | --- |
| Cell radius | 300m |
| Frequency re-use | 1/1 |
| #TRX | 3 |
| #Frequencies | 9 |
| Erlang per cell | 14.3 |
| Power control | ON |
| Speech codec | AFS5.90 |
| DTX | ON |
| Speech activity factor | 0.6 |
| #cells in system | 147 |
| Pathloss model | Okumura-Hata |
| Shadow fading | Log-normal, standard deviation = 8 dB |

### Interferer strength

The strength of the carrier and the two strongest CCI interferers, the two strongest ACI+ interferers and the two strongest ACI- interferers are logged for each transmitted burst in the system. The statistics are binned based on the C/Itot  (where Itot is the total interferer energy) before fast fading. For a given C/Itot, the median strength of each of the interferers is stored. This way, a C/I-dependent interferer strength profile is derived.

The results are shown in Figure 2 and Figure 3 for the uplink and for the downlink, respectively. The individual interferer strengths as well as Itot are defined after the RX filter assuming and an ACP of 18 dB.

An interesting observation is that at low C/I levels, the interference is dominated by the strongest CCI (especially for downlink), whereas at higher C/I, the second strongest CCI and the ACIs become increasingly prominent (i.e., closer in strength to the strongest CCI).

Figure 2: Interferer strengths for uplink scenarios.

Figure 3: Interferer strengths for downlink scenarios.

### Co-TSC probability

The co-TSC probabilities for the two strongest CCIs are also derived from the system simulation statistics. The probabilities are calculated per cell. CDFs over all cells are shown in Figure 4 and Figure 5 for uplink and downlink, respectively.

Figure 4: CDFs of co-TSC probability in uplink scenarios.

Figure 5: CDFs of co-TSC probability in downlink scenarios.

To cover a wide range of situations in the network, three different scenarios are considered when deriving the likelihood of co-TSC in the interferer models: the 10th percentile (corresponding to a good cell from a TSC planning perspective), the median (corresponding to a median cell) and the 90th percentile (bad cell). The probabilities are summarized in Table 2.

|  |  |  | 10th percentile | 50th percentile | 90th percentile |
| --- | --- | --- | --- | --- | --- |
| Uplink | 8 TSC plan | 1st CCI | 3.5% | 6.5% | 11% |
|  |  | 2nd CCI | 6.9% | 9.8% | 13% |
|  | 16 TSC plan | 1st CCI | 0.64% | 1.6% | 3.2% |
|  |  | 2nd CCI | 1.7% | 3.2% | 5.7% |
| Downlink | 8 TSC plan | 1st CCI | 1.8% | 5.5% | 13% |
|  |  | 2nd CCI | 4.4% | 9.9% | 16% |
|  | 16 TSC plan | 1st CCI | 0.19% | 1.1% | 4.7% |
|  |  | 2nd CCI | 0.80% | 3.0% | 6.2% |

Table 2: Probabilities of co-TSC.

## Link level simulations

Based on the statistics derived in section 3, an interference model is built and used in link simulations.

### Interference model

The interference model consists of two CCI interferers, two ACI+ interferers and two ACI- interferers. Their relative strengths (before fast fading) are set according to Figure 2 (for uplink simulations) and Figure 3 (for downlink simulations) depending on the C/I.

The CCI interferers randomly use the same TSC as the carrier with probabilities given in Table 2 for a given configuration (in total there are 12 configurations in Table 2 – two link directions, two different TSC plans and three different percentiles). When the co-TSC is not chosen, one of the other TSCs (7 or 15 other, depending on the used TSC plan) is chosen randomly with a uniform distribution. The ACI interferers randomly choose a TSC from all available (8 or 16) TSCs. The carrier always uses TSC 0 from legacy set 1.

In the 16 TSC plan case, the GMSK TSC Set 3 in [3] is used in addition to the legacy GMSK TSC Set 1.

All interferers are GMSK modulated. The NewToN propagation delay models are used.

### Other simulation parameters

Other simulation parameters are listed in Table 3.

Table 3: Parameters for the link simulations.

| Parameter | Value |
| --- | --- |
| Channel model | TU50 noFH |
| Frequency band | 900 MHz |
| Channel coding | AFS5.90 |
| Receiver | DL: SAIC UL: IRC |
| TX impairments | Typical |
| RX impairments | Typical |
| Number of speech frames | 10000 |

### Results and discussion

Plots of class 1A FER versus C/I are shown in Figure 6 and Figure 7 for uplink and downlink, respectively. It is evident that even though the co-TSC probabilities are much smaller than in the 100% co-TSC scenario in Figure 1, they have a significant impact on performance. The difference between a good and bad cell (10th and 90th percentile) is about 1.6-2.5 dB at 1% FER. The gain of using a TSC plan with 16 TSCs instead of eight is 1.7-2.7 dB. This gain is partly due to the reduced co-TSC probability and partly due to the better cross correlation properties of the extended TSC set.

Figure 6: FER vs C/I in uplink scenarios

Figure 7: FER vs C/I in downlink scenarios.

# System level simulations

## Introduction

System level simulations have been carried out using a dynamic system simulator where a link simulator object has been integrated in each radio link to model the link level performance of each user.

Hence, instead of using Link-2-System mappings, which is the conventional method to model radio link performance on system level, the link performance is modeled on IQ-level with demodulators called for each user and each burst.

This allows the evaluation of system performance to fully take into account complex aspects such as TSC allocation and their impact on system capacity.

More details on the methodology and verification of the integrated link simulator is provided in [4].

## Simulation assumptions

The same system level configuration as presented in Table 1 was used, except that AFS12.2 was used for the simulations instead to get a quality limited network below the load of 2 % blocking.

The same TSC planning algorithm was used as described in Section 2.2. Since this planning principle mainly aims at avoiding co-TSC interference there is a need to map a specific TSC value/index to each specific TSC value in the plan. In other words, the TSC planning algorithm will determine for example that e.g. cells [1,15,27,35,52,89,115,132,145] should have the same TSC in order to avoid co-TSC in the network (and similar cell-vectors exist for all 8 or 16 TSCs). It will however, not map a specific TSC to these cells. In order to estimate the impact on the results from different TSC plans, three different, randomly chosen, mapping vectors were generated and simulated. The result for each simulated scenario is an average of these three mapping alternatives.

More details on the methodology and verification of the integrated link simulator is provided in [4].

The two TSC planning scenarios as described in Section 2.3.1 was also evaluated on system level. In addition, the TSC plan of 16 available TSCs, only taken from the proposed NewToN set was also simulated. This scenario would represent a system with a high penetration of NewToN MS where the new set could be used as a baseline in the TSC planning, and the legacy set is only used when allocating users in a VAMOS channel.

The metric on “Happy users” is taken from the MUROS study where a <2% call FER is classified as a “Happy user” when simulating FR channels.

## Results

The results are shown in Figure 8.

Figure 8. System level results with different TSC mapping plans

It can be seen that the system level capacity gains in reference to the legacy 8 TSC planning are quite substantial both for the case of using legacy set 1 together with NewToN set 3, with further gains when adding a complete TSC plan using the NewToN set.

The results in terms of system capacity gains are also summarized in Table 4 at the quality limit of 95% Happy users.

Table 4. System capacity gains with NewToN compared to system performance using legacy set 1.

| System capacity gains [%] |  |
| --- | --- |
| Legacy set 1 + NewToN set 3 | NewToN set 3 + NewToN set 4 |
| 33.6 | 47.4 |

It should be noted that simulation results presented have not reached their full simulation length, and should be considered preliminary. However, considering the significant differences provided in the different scenarios, the conclusions are not expecting to change.

# Performance comparison according to NewToN framework

In Annex A, the performance gain of the proposed TSC set [3] according to the performance evaluation framework is shown. The gain is shown compared to two different references. The first reference is the legacy training sequences for all modulations, excluding GMSK TSC Set 2 (expect for the VAMOS performance, for which both GMSK TSC sets were used). Compared to this reference, the gain is 1.5 dB, when averaged across all scenarios defined in the framework.

The second reference is using all legacy training sequences, i.e., GMSK TSC Set 2 is included. The gain compared to this reference is 0.7 dB.

It can be seen that gains of up to 4.8 dB is observed in the extreme scenario (32QAM carrier, GMSK interferer) while some performance losses are also observed, mainly in scenarios where low weight is given to the interferer scenario, modulation combination according to the agreed framework.

# Conclusions

In this contribution, the impact of co-TSC interference (interference with the same TSC as the wanted signal in a synchronized network) has been investigated. Further, the gains of having a sparser TSC plan (as enabled by e.g. NewToN) have been assessed. It was found that by using 16 TSCs instead of eight in the TSC plan, the probability of strong co-TSC interference can be reduced, resulting in a link level gain of 1.7-2.7 dB.

The new TSC set has also been investigated on system level using a dynamic system level simulator with an integrated link level simulator object in detail modeling the impact of TSCs allocation for each radio link. System capacity gains in the range of 33.6 - 47.4 % were observed compared to a system utilizing TSC set 1 for the TSC plan.

Furthermore, the proposed TSC set was evaluated with the agreed framework to provide on average 1.5 dB and 0.7 dB gains respectively when compared with TSC set 1 and TSC set 1 and TSC set 2. The gains were seen to provide rather large variations depending on scenario with maximum gain at 4.8 dB, but also noting some losses mainly in the less prioritized scenarios according to the agreed framework.

# References

GP-131139, “New Training Sequence Codes for GERAN”, source Telefon AB LM Ericsson, Telecom Italia S.p.A., China Mobile Com. Corporation, NOKIA Corporation, Alcatel-Lucent, Vodafone Group, Com-Research GmbH. GERAN#60

GP-140352, “NewToN – Working assumptions”, source Ericsson. GERAN #62

GP-140587. “Training sequence design for NewToN”, source Ericsson. GERAN#63

GP-101844,	“Link performance methodology for system level evaluations (update of GP-101544)”, source 	Telefon AB LM Ericsson, ST-Ericsson SA. GERAN#48.

# Annex A: Performance comparison according to NewToN performance framework

Figure 8 summarizes the gains of the proposed NewToN TSC sets in [3] compared to legacy TSC Sets, according to the performance evaluation framework in [2].

In the left table, GMSK TSC Set 2 was excluded except for the sensitivity performance with VAMOS, for which GMSK TSC Set 2 was included.

In the right table, GMSK TSC Set 2 is included also in the non-VAMOS simulations.

Figure 9: Performance evaluation of TSC proposal according to the framework, compared to legacy training sequences, using a BTS receiver.