pCR 43.869 Introduction of Reference Cases

Introduction

Background

The objective of uPod study item is to study on power saving for Machine Type Communication (MTC) devices in GERAN. It’s important to evaluate how much enhancement can be reached compared with legacy mechanism. This pCR introduces the reference cases.

Reason for change

We think it is necessary to involve reference cases in the uPod study to avoid ambiguous judgment for candidate solutions.

The functions of reference cases in the uPod study are that:

- provide legacy mechanism for clarification

- provide common parameter for evaluation

- calculate energy consumption as a base line

It is agreed that operating current and voltage involve in the TR. In order to calculate energy consumption, the operation time is proposed to be defined also. And some common parameters should be involved in evaluation assumptions of the TR.

Summary of changes

Section 6.2 is updated by adding operation time and common parameters. Some description about how to use reference cases is added in Section 6.3. And Section 6.4 is added with the introduction of reference case as the baseline to evaluation energy consumption.

PCR

============================== first change ==========================

- 6.2	Evaluation Assumptions

Editor’s notes: this section is to provide the assumptions that used for power consumption evaluation e.g. idle mode procedures, corresponding parameters, energy consumptions etc.

- 6.2.1 Operating voltage and current

Typical values of average current at different operational modes (procedures) are tabulated in Table 2. It has been assumed that the same operating voltage is used in all operational modes and that it is the operating current that is varying based on the specific mode of operation performed. The operating voltage is assumed to be 3.3 V.

Any use-case can be broken down into one or several instances of variable length intervals of the different operational modes:

- Transmission (Tx) - baseband and RF are engaged in data transmission.

- Reception (Rx) - baseband and RF are engaged in data reception.

- Monitoring - RSSI measurements are performed for serving/neighbour cells.

- Light sleep - MS is in wakeup state and waiting to perform impending tasks.

- Deep sleep - there is no activity for a long duration, modem enters into prolonged sleep (instead of staying in light sleep) by cutting down voltages and clocks to all modem sub-modules excepts RAM memory (for refreshment) and timer management (for interrupt generation).

Table 2 Typical values of average current during different Modem operations in an MS.

| Operation | Specification | Operating current (µA) |
| --- | --- | --- |
| Transmission (Tx) | Tx (GMSK) (33 dBm) | xx |
|  | Tx (8-PSK) (27 dBm) | xx |
| Reception (Rx) | Rx with Baseband processing | xx |
|  | Monitoring (RSSI with 32 I,Q samples) | xx |
| Power Saving States | Light sleep | xx |
|  | Deep sleep | xx |
|  | Power off(1) | xx |

Note 1: It should be noted that in this state the modem is completely switched off, however some energy is consumed by the application running able to wake the modem up.

- 6.2.2 Operating time

The typical time values for different operations are provided in Table 3.

Table 3 Typical time values of energy consumption during different MS operations [1]

| Operation | Operation time |
| --- | --- |
| RACH access | 1 burst transmitted |
| Rx - One radio block | 4 bursts received + 28 bursts (in light sleep or BCCH measurement) (Applies also to for example BCCH, PCH blocks) |
| Tx - One radio block (@ 33 dBm output power, GMSK) | 4 bursts transmitted (in Tx GMSK) + 28 bursts (in light sleep or BCCH measurement) |
| Monitor BCCH carrier - one BCCH carrier measurement | 0.5 bursts received. (In case X neighbours are monitored the figure need to be multiplied by X) |
| BSIC Reconfirmation (Rx) - One BSIC reconfirmation | a single SCH burst is read |
| BSIC Identification (Rx + Idle) – One BSIC identification | an average of 32 bursts in light sleep + 1 FCH burst read + 1 SCH burst read |
| Deep sleep | 1000 bursts |

- 6.2.3 Default parameters

The default parameters defined here should be applied in the evaluation of any candidate solutions, except the ones impacted in the optimization, where the proposed values will be applied instead. [3]

Table 4 Typical value for common parameters

| Parameters | Value |
| --- | --- |
| The number of RF channels or BCCH carriers searching for cell selection | 124[note1] or 8 [note2] |
| The number of RF channels or BCCH carriers to be identified before a suitable cell is selected | 10 |
| The number of full set of System Information to be read | 4[note3] |
| the number of non-serving cell BCCH carriers maintained in BA list | 16 |
| The number of new BCCH carriers which become one of the 6 strongest non-serving cell BCCH carriers (performed as part the task of maintaining the BA list) | 1 |

[note 1]: for frequency scanning, only 900M band is considered.

[note 2]: for stored list

[note 3]: SI1,SI2,SI3 and SI 13 are included and only one radio block is assumed to be occupied by each SI.

==============================second change ==========================

- 6.3 Evaluation Metrics

Editor’s notes: this section is to provide the metrics that collected from the evaluation, e.g. total energy consumption, time duration etc.

Energy consumption is the primary issue in the uPod evaluation, which is calculated by Voltage * Current * Operating Time. In order to evaluate the benefit brought by any candidate solutions on GERAN network, both the energy consumption of reference case and candidate solution should be provided for comparison.

The reference case in 6.4 is defined to be used for the calculation as a baseline in the evaluation.

In the evaluation, the network configuration based on different scenarios should be the same for both candidate solution and reference case, except the optimization part which should be highlighted.

===============================last change ===========================

## 6.4 Reference Case Assumptions

For clarification, the reference case will be used for calculation the energy consumption for the typical MS today. That’s the baseline to evaluate energy consumption. When developing the solutions, the reference case will be rebuilt. [2] The same reference case should be used when evaluating both use cases defined in Section 5.

### 6.4.1 Packet Idle Mode

- 6.4.1.1 Typical procedures and sub procedures

The procedures and sub-procedures of idle mode tasks are described as following:

- Measurements for cell selection:

Measurement for cell selection can be based on either frequency scanning or stored list. It’s depending on if the MS has prior knowledge of BCCH carriers.

  - Frequency scanning:

- Receive RF signal and calculate the average receive level for each frequency in the operation bands supported.

- Identify the BCCH carrier by reading frequency correction bursts.

- Read full set of BCCH data of the cell selected

  - Stored list:

- Receive RF signal and calculate the signal level for each frequency in the stored BCCH carrier information (BCCH list)

- Identify the BCCH carrier by reading frequency correction bursts.

- Read full set of BCCH data of the cell selected

- If stored list cell selection is not successful, the MS shall perform the frequency scanning.

- Measurements for cell re-selection

  - measurements and data reading of servicing cell

- Monitor of received signal level of serving cell

- Monitor BCCH data of the serving cell at least every 30 seconds

  - measurements and data reading of neighbour cells

- Monitor all BCCH carriers as indicated by the BA list

- Identify BSIC for 6 strongest neighbor cells

- Reconfirmation BSIC for 6 strongest neighbor cells at least every 30 seconds

- Identify new 6 strongest cells BSIC and BCCH data.

- Read BCCH data for 6 strongest neighbor cells at least every 5 minutes

- PCH reading

  - PCH data reading in every DRX cycle

- 6.4.1.2 Energy consumptions as per procedure

The calculation for total reference case energy consumption is based on different MS operations. The energy consumption for every procedure listed in Table 5 can be calculated as follows based on section 6.2.

Table 5 Energy consumptions for MS in idle mode

| Procedure | sub-procedure | operation | occurrence | Energy consumption for each sub-procedure (uJ) | Energy consumption for procedure (uJ) |
| --- | --- | --- | --- | --- | --- |
| Measurements for cell selection | RF signal receiving | Monitor BCCH carrier | switch on or from non-service area into the service area | TBD | TBD |
|  | Identify BCCH carriers in the list | BSIC Identification |  | TBD |  |
|  | decode the full set of BCCH data of the serving cell | Rx - 4 radio block |  | TBD |  |
| Measurements for cell re-selection | Monitor of received signal level of serving cell | Monitor BCCH carrier | DRX cycle | TBD | TBD |
|  | Monitor BCCH data of the serving cell | Rx - One radio block | once every 30s | TBD |  |
|  | Monitor all BCCH carriers as indicated by the BA list | Monitor BCCH carrier | DRX cycle | TBD |  |
|  | identify BSIC of 6 strongest | BSIC Identification | once | TBD |  |
|  | reconfirm BSIC of 6 strongest | BSIC Reconfirmation | once every 30s | TBD |  |
|  | identify BSIC and decode BCCH data ( only SI3) for the new BCCH carrier which become one of the 6 strongest | BSIC Identification and Rx - One radio block | once every 1h | TBD |  |
|  | decode BCCH data of 6 strongest ( only SI3) | Rx - One radio block | once every 5min | TBD |  |
| PCH reading | read PCH | Rx - One radio block in serving cell | DRX cycle | TBD | TBD |

### 6.4.2 Packet Transfer Mode

Editor’s notes: this chapter is to provide the procedures and energy consumptions when MS is entering the packet transfer mode such as Attach, RAU and normal data transmission.

Reference

[1] GP-140361, “MS energy consumption model”, source Ericsson, GERAN#62.

[2] 3GPP TSG GERAN Telco#2 on uPod, “Reference case definition”, Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd.

[3] GP-140324, “Proposed Working Assumptions on power consumption evaluation in idle mode for MTC devices”, Huawei Technologies Co., Ltd. , HiSilicon Technologies Co., Ltd., GERAN#62.