C-UNB technology for Cellular IoT - Performance evaluation

# Introduction

The Cooperative Ultra Narrow Band (C-UNB) technology is a disruptive approach for addressing the Cellular IoT requirements, as defined in [1]. Contribution [2] gives an overall description of the C-UNB technology. Contribution [3] is a description of the C-UNB physical layer, whereas contribution [4] is for MAC-layer. This document gives performance evaluation of the C-UNB technology with regards to requirements given in [5].

# Capacity

## Methodology

The methodology for capacity evaluation is fully detailed in chapter 3 and annex D, E & F of document [5]. The calculation assumes a coverage of a legacy cellular network with a given density of devices per cell and a given traffic model per device. The following subclauses explain how density and traffic assumptions are adapted to the C-UNB technology.

### Coverage calculation

In 2G cellular networks, coverage is evaluated using a hexagonal grid: each 200kHz channel is dedicated to a given cell (see figure 1a). Capacity metric is defined as spectral efficiency in number of reports/200kHz/hour.

Figure 1a and 1b: Sector coverage in 2G and in C-UNB

In the case of C-UNB technology, the coverage for each 200kHz channel is much larger than a single hexagonal cell (see figure 1b), because of the overlap needed for cooperative reception. This overlap is not mandatory but helps reducing the probability of collisions.

In the following capacity evaluations, we define three levels of overlap:

high: range of a C-UNB site reaches the inter site distance (ISD),

low: range of a C-UNB site is 2/3 of ISD,

medium: range is 3/2 ISD.

### Traffic model

Document [5] evaluates the capacity of candidate technologies with a traffic model of 80% of Mobile Autonomous Report (MAR) and 20% of Network Command. MARs are uplink and Network Commands are downlink. In C-UNB, there is no interaction between uplink capacity and downlink capacity, because downlink operates with time-delayed piggy-backing, therefore without adding extra traffic to the uplink. For the sake of simplicity, the traffic model of MAR is average as 11,2 MARs per day per device, as explained in Table 1.

The size of MARs is a Pareto distribution with parameters given in table E.2.1 of [5]. Capacity evaluation given hereunder use mean value of the MAR size given by the formula ():

(1)

where k is the shape parameter and xm is the minimum application payload size. With values given in table E.2.1 of [5], the mean size of a MAR is about 33.3 bytes.

Table 1: traffic model of MAR per day

| inter-arrival time | ratio | number of MAR per day | average number of MAR per day |
| --- | --- | --- | --- |
| 1 day | 40% | 1 | 11,2 |
| 2 hours | 40% | 2 |  |
| 1 hour | 15% | 24 |  |
| 30mn | 5% | 48 |  |

In C-UNB, the payload is variable size. With a payload size of 20 bytes, the average number of radio packet transmitted by a device per day is:

## Capacity results

Table 2 gives the capacity of C-UNB technology in terms of device per square kilometers. Values ranging from 48 600 to 109 400 are compatible with the requirements of 60 680 devices/km2, as stated in [5]

Table 2: C-UNB capacity

| Overlap | C-UNB sector radius | Capacity (device/km2) |
| --- | --- | --- |
| high | ISD | 48 600 |
| low | 2/3 ISD | 109 400 |
| medium | 3/2 ISD | 64 800 |

# Battery life

Table 5.4-3 in document [5] defines various scenarios for battery life analysis. Different values for transmit power and packet sizes are suggested. Packet sizes include Payload and COAP+DTLS+UDP+IP header overhead. Results in Table 3 assume a C-UNB packet size of 25 bytes. The battery energy is 5 Wh, as stated in [5].

Table 3: battery life of C-UNB devices (in years)

Note: Battery lifes over 15-20 years can hardly be reached with a standard Lithium Polymer cell, but these values are given here to demonstrate the power efficiency of the C-UNB technology.

# Conclusion

This contribution evaluates C-UNB performance in terms of capacity and battery life. Compared to the radio interface of legacy 2G networks, the C-UNB technology is totally disruptive. But the use of innovative features, such as ad'hoc micro-channels and cooperative reception, makes it compliant with the main requirements of Cellular IoT.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT

[3]	GP-150057, "C-UNB technology for Cellular IoT- Physical layer description"

[4]	GP-150058, "C-UNB technology for Cellular IoT - MAC layer description"

[5]	TR 45.820 v0.3.1 (2015-02), "3rd Generation Partnership Project; Technical Specification Group GSM/EDGE Radio Access Network; Cellular System Support Ultra Low Complexity and Low Throughput Internet of Things; (Release 13)"

The formula (1) includes the tail of the distribution. In [5], the tail is cut to 200 bytes. Therefore, formula (1) is an overestimation of the actual mean value.