NB M2M- Evaluation of Transmission Efficiency

Background

As agreed at GERAN#64 meeting in [1], the transmission efficiency will be used as a criterion for the architecture evaluation. In this contribution, the transmission efficiency of Gb based and S1 based architecture is discussed for both cases of Packet Data Transmission and Short Message Service (SMS).

It is an updating of [6]. The major parts updated are in red.

Discussion

Assumption

There are some common assumptions in the evaluation listed in Table 1.

Table 1. Common assumption

|  | Gb based | S1 based |
| --- | --- | --- |
| Common |  |  |
| >Signalling | legacy GPRS signaling | Optimized RRC signaling [2] |
| > Retransmission | None | None |
| > Traffic Model | Mobile Autonomous Reporting (MAR) periodic reports [3] The data size of packet is fixed as 20 bytes and the size of application ACK is 0. |  |
| > Others | The UE is assumed to have attached successfully. The attach/RAU/TAU procedures are not taken into account |  |
| Packet Data Transmission (PDT) |  |  |
| > IP header compression | Option 1: None Option 2: Yes | Option 1:None Option 2: Yes |
| > application ACK | ACK by CoAP layer | ACK by CoAP layer |
| SMS |  |  |
| > Category | SMS over GPRS, see Annex 4.1 | SMS over SGs, see Annex 4.2 &4.3 SMS over IP is not considered |
| > CN required | SGSN | MME, MSC |
| > UE required | none | Combined attach procedure should be supported |

Evaluation methodology

It is required in the TR Mobile Autonomous Reporting (MAR) periodic reports will be used in the evaluation. However, the signalling consumption for period transmission is linear with the consumption for one transmission. The transmission efficiency here is only evaluated during one data transmission procedure.

The coverage condition is not taken into account which should be reflected in the MCS selection on PHY. Consequently, the result is irrelevant with coverage condition.

IP-based Packet Data Transmission

Gb base Architecture

Without IP header compression

Table 2. Signalling Flow for PDT without IP header compression

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request | 0 | 0 | 0 | 4 | 0 | 4 |
| Random Access Response | 0 | 0 | 0 | 6 | 0 | 6 |
| Uplink Data Transmission (User DATA) | 20 | 65 | 12 | 4 Note 1 | 2 Note 2 | 103 |
| Downlink Data Transmission (Application ACK) | 0 Note 3 | 65 | 12 | 0 | 0 | 77 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 0 | 2 |
| Connection Release | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  |  |  |  |  | 194 |
| Note 1: TLLI included in a MAC control element and sent with the data Note 2: The header for the MAC control element taking the TLLI. Note 3: Application ACK by COAP layer |  |  |  |  |  |  |

With IP header compression

Table 3. Signalling Flow for PDT with IP header compression

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request | 0 | 0 | 0 | 4 | 0 | 4 |
| Random Access Response | 0 | 0 | 0 | 6 | 0 | 6 |
| Uplink Data Transmission (User DATA) | 20 | 29 | 12 | 4 Note 1 | 2 Note 2 | 67 |
| Downlink Data Transmission (Application ACK) | 0 Note 3 | 29 | 12 | 0 | 0 | 41 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 3 | 2 |
| Connection Release | 0 | 0 | 0 | 2 | 3 | 2 |
|  |  |  |  |  |  | 122 |
| Note 1: TLLI included in a MAC control element and sent with the data Note 2: The header for the MAC control element taking the TLLI. Note 3: Application ACK by COAP layer |  |  |  |  |  |  |

S1 based Architecture

When the optimization in [2] is used, the UE is always in the RRC_CONNECTED mode. The following period report reuses the existing DRB which is setup in the first data transmission and there is no need for RRC connection setup procedure every time. With this optimization, the consumption of signalling is reduced significantly.

Without IP header compression

Table 4. Signalling Flow for PDT without IP header compression

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request (with C-RNTI) | 0 | 0 | 0 | 4 | 0 | 4 |
| Uplink Data Transmission (User DATA) | 20 | 65 | 4 | 0 | 0 | 89 |
| Downlink Data Transmission (Application ACK) | 0 Note 1 | 65 | 4 | 0 | 0 | 69 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  |  |  |  |  | 164 |
| Note 1: Application ACK by COAP layer |  |  |  |  |  |  |

With IP header compression

Table 5. Signalling Flow for PDT with IP header compression

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request (with C-RNTI) | 0 | 0 | 0 | 4 | 0 | 4 |
| Uplink Data Transmission (User DATA) | 20 | 29 | 4 | 0 | 0 | 53 |
| Downlink Data Transmission (Application ACK) | 0 Note 1 | 29 | 4 | 0 | 0 | 33 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  |  |  |  |  | 92 |
| Note 1: Application ACK by COAP layer |  |  |  |  |  |  |

SMS

Gb base Architecture

Table 6. Signalling Flow for SMS

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request | 0 | 0 | 0 | 4 | 0 | 4 |
| Random Access Response | 0 | 0 | 0 | 6 | 0 | 6 |
| Uplink Data Transmission (CP DATA) | 20 | 9.375 Note 3 | 12 | 4 Note 1 | 2 Note 2 | 47.375 |
| Downlink Data Transmission (CP ACK) | 0 | 2 Note 4 | 12 | 0 | 0 | 14 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 0 | 2 |
| Downlink Data Transmission (CP DATA) | 0 | 3.375 Note 3 | 12 | 0 | 0 | 15.375 |
| Uplink Data Transmission (DL ACK & BSR) | 0 | 0 | 0 | 2 Note 5 | 4 Note 6 | 6 |
| Uplink Data Transmission (CP ACK) | 0 | 2 Note 4 | 12 | 0 | 0 | 14 |
| Connection Release | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  |  |  |  |  | 110.75 |
| Note 1: TLLI included in a MAC control element and sent with the data Note 2: The header for the MAC control element taking the TLLI. Note 3: the header and data for SM-RL and CM sub-layer [3] Note 4: the header and data for CM sub-layer [3] Note 5: Two MAC control elements are included in this uplink transmission. The size of signaling is 1 byte for each Note 6: Two MAC control elements are included in this uplink transmission. The MAC header is 2 bytes for each. |  |  |  |  |  |  |

S1 based Architecture

Table 7. Signalling Flow for SMS

|  | D_application | H_CN | H_access | S_radio | H_signalling | bytes |
| --- | --- | --- | --- | --- | --- | --- |
| Random Access Request (with C-RNTI) | 0 | 0 | 0 | 4 | 0 | 4 |
| Uplink Data Transmission (CP DATA) | 20 | 9.375 Note 1 | 0 | 0 | 10 Note 2 | 39.375 |
| Downlink Data Transmission (CP ACK) | 0 | 2 Note 5 | 0 | 0 | 10 Note 2 | 12 |
| Uplink Data Transmission (DL ACK) | 0 | 0 | 0 | 2 | 0 | 2 |
| Downlink Data Transmission (CP DATA) | 0 | 3.375 Note 1 | 0 | 0 | 10 Note 2 | 13.375 |
| Uplink Data Transmission (DL ACK & BSR ) | 0 | 0 | 0 | 2 Note 3 | 4 Note 4 | 6 |
| Uplink Data Transmission (CP ACK) | 0 | 2 Note 5 | 0 | 0 | 10 Note 2 | 12 |
|  |  |  |  |  |  | 88.75 |
| Note 1: the header and data for SM-RL and CM sub-layer [3] Note 2: The SMS is sent as a RRC signaling in Uplink /DOWNLINK NAS TRANSPORT message. The header for NAS is 2bytes, the header for RRC is 1bytes, the PDCP is 5 bytes and the MAC is 2 bytes. Note 3: Two MAC control elements are included in this uplink transmission. The size of signaling is 1 byte for each Note 4: Two MAC control elements are included in this uplink transmission. The MAC header is 2 bytes for each, Note 5: the header and data for CM sub-layer [3] |  |  |  |  |  |  |

Evaluation

According to the criteria of transmission efficiency in 8.2.1 of [3], the transmission efficiency can be calculated by the formula below,

E_transmission=D_application/ (D_application+H_CN + H_access + S_radio + H_signalling)   (1)

However, before that, it is helpful to evaluate the application transmission efficiency for each case. Here, the application efficiency is calculated isolated with the radio bearer. In other words, no matter which kind of radio access network, the transmission efficiency could not be higher than the application transmission efficiency.

The application transmission efficiency could be calculated by the formula and listed in Table 8.

E_application =D_application/ (D_application+H’_CN)              (2)

- For PDT, H’_CN including the overhead from the application layer to IP layer

- For SMS, H’_CN including the overhead from the application layer to CM sub-layer.

- The other parameters in formula (2) have the same definition within formula (1)

Table 8. Application Transmission Efficiency

|  |  | Application efficiency |
| --- | --- | --- |
| PDT | Without IP header compression | 13.30% |
|  | With IP header compression | 25.64% |
| SMS |  | 54.42% |

From Table 8, it could be found in the IP based PDT, considering the consumption of IP header, the transmission efficiency is lower the SMS. And since the application efficiency is far from 100%, from the view of sourcing company, the normalized transmission efficiency by application efficiency will provide more reliable comparison between Gb based architecture and optimized S1 based than the absolute efficiency calculated by formula (1).

The normalization is proposed by the formula (3) as follows,

E_normalized = E_ transmission / E_ application    (3)

With the application transmission efficiency, the normalized transmission efficiency could be calculated in Table 9. And the optimized S1 based architecture has better performance than Gb based in either PDT or SMS.

Table 9. Normalized Transmission Efficiency over NB M2M

|  |  | Gb based | Optimized S1 based | benefit |
| --- | --- | --- | --- | --- |
| PDT | Without IP header compression | 77.51% | 91.69% | 18.29% |
|  | With IP header compression | 63.94% | 84.79% | 32.61% |
| SMS |  | 33.18% | 41.41% | 24.79% |

It should be highlighted that the evaluation is based on a preliminary design of NB M2M. There will be many aspects that could be optimized later such as the format of signalling with fewer headers and so on. So it makes no sense to compare the transmission efficiency calculated here with other mature system such as GERAN or LTE.

During the discussion in Adhoc meeting, one possible way is to apply the optimization in [2] to Gb based architecture. After the investigation of sourcing company, the efficiency is the same with the one in the optimized S1 based architecture. Table 10 below is provided for reference.

Table 10. Normalized Transmission Efficiency over NB M2M

|  |  | Gb based | Optimization in Gb based | Optimized S1 based |
| --- | --- | --- | --- | --- |
| PDT | Without IP header compression | 77.51% | 91.69% | 91.69% |
|  | With IP header compression | 63.94% | 84.79% | 84.79% |
| SMS |  | 33.18% | 41.41% | 41.41% |

However, from the view of sourcing company, the optimization for the S1 based architecture is requisite since the complex signalling setup procedure really lows down the efficiency. However, the optimization for Gb could be take as an enhancement. Without it, the performance in transmission efficiency of Gb based architecture could still not bad.

Conclusions and Proposals

The transmission efficiency of Gb based architecture and optimized S1 based architecture are evaluated. It is proposed that,

Proposal: Normalization of transmission efficiency by the application efficiency in the evaluation to provide reliable comparison between Gb based and S1based architecture.

And based on the proposal, it can be concluded that:

- In either IP based PDT or SMS, the optimized S1 architecture over NB M2M provides higher transmission efficiency than legacy Gb based architecture.

- When the optimization is applied in both Gb and S1 architecture over NB M2M, the same transmission efficiency could be expected.

Annex

GPRS Mobile Originated Messaging in A/Gb mode [4]

LTE Mobile Originated Messaging [4]

NOTE:	Service Request Procedure may not be initiated if RRC connection already exists

LTE Mobile Originating SMS in active Mode procedure [5]

Figure A.1: Mobile originating SMS in idle mode

Mobile Originating SMS in active Mode procedure is specified by reusing the Mobile Originating SMS in Idle Mode with the following modification:

-	The established signalling connection between the MS/UE and the MME is reused for the transport of the SMS message and the delivery report (i.e. the UE triggered Service Request procedure defined in step 2 is skipped).

References

[1] GP-140857, “NB M2M - Evaluation of Transmission Efficiency”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd, GERAN#64.

[2] GPC150006, “Optimisations for S1-based architecture”, Neul Ltd., 3GPP TSG GERAN adhoc#1 on FS_IoT_LC.

[3] Draft_3GPP TR 45 820 V0 3 0_including agreements up to GERAN#64_clean.

[4] 3GPP TS 24.011 Group Core Network and Terminals; Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface.

[5] 3GPP TS 23.272 Group Services and System Aspects; Circuit Switched (CS) fallback in Evolved Packet System EPS); Stage 2.

[6] GPC150051, “NB M2M- Evaluation of Transmission Efficiency”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd, 3GPP TSG GERAN adhoc#1 on FS_IoT_LC.