NB M2M- Exception Report Latency

Background

At GERAN#62, a new SI was agreed to study Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things [1]. A radio access technology referred to as NB M2M has been introduced as a candidate solution [2].

In this contribution, the exception report latency is analysed for the NB M2M solution, based on the agreed methodology in [3]. Results are presented for 0 dB, 10 dB and 20 dB coverage extension compared with legacy GPRS.

Evaluation

Common Assumptions

The common assumptions used in the evaluation are as follows:

- Packet size for the exception report:

95 bytes = 20 bytes (for application payload) + 65 bytes (higher layer headers assuming no IP header compression) + 4 bytes (for SNDCP) + 6 bytes (for LLC)

In NB M2M, the TLLI (4 bytes) is included in the first uplink packet following RACH when using the Gb interface. In addition, there are MAC overheads and a CRC. Therefore, the total size of the MAC PDU for the exception report, including the CRC, is:

105 bytes = 95 bytes + 4 bytes (for TLLI) + 3 bytes (for MAC header) + 3 bytes (CRC)

- The formula for calculating the data transmission latency is as follows

Latency for DATA transmission = (1) T Synchronization+ (2) T Transmission + (3) T Receiving + (4) T Wait

- TSynchronization: The time for UE to synchronize to the network. This value depends on the coverage condition.

- TTransmission: The transmitting time for any signalling and data.

- TReceiving: The receiving time for any signalling and data. It should be highlighted that scheduling information that must be received by a UE but which is not included in obvious signalling, such as USF, should be taken into account.

- TWait: The time between any transmission and reception, and also the time between two consecutive transmissions or receptions. The waiting time depends on the scheduling mechanisms for a specific solution.

Signalling Flow

For the NB M2M solution, the signalling flow for exception reporting is depicted in Figure 1.

Twait_1 is the time between synchronization being achieved and the start of the RACH transmission. It must be at least the minimum UE transition time between RX and TX since NB M2M assumes half duplex UE operation.

For exception reporting, the common RACH resource will be used for access. The location of the common RACH resource is broadcast in the System Information.

Twait_2 is the time between the end of the RACH transmission and the start of the DCI that addresses the UE to allocate resource for the Random Access Response (RAR) and the uplink report. It must be at least the minimum UE transition time between TX and RX, and also no more than typically two DCI intervals otherwise it will be considered as an access failure. In this analysis, it is assumed that the UE is addressed by the next DCI following the end of the RACH transmission.

Twait_3 is the time between the end of the DCI and the start of the RAR message containing the C-RNTI allocation. Typically the RAR reception will be completed within the same DCI interval as the DCI that contains the RAR allocation. Consequently, Twait_3 equals the length of the DCI interval minus the sum of the lengths of the DCI and the RAR.

Twait_4 is the time between the end of the Random Access Response and the start of uplink report. It must be at least the minimum UE transition time between RX and TX.

Figure 1. Signalling Flow for Exception Report

In the NB M2M solution, the duration of the DCI interval depends on the coverage condition. This allows different coverage classes to use different DCI intervals in order to optimise UE latency, UE battery life, and network capacity.

Similarly, the durations of the DCI, RACH, RAR and UL report depend on the MCS and CBS indexes that are selected for each burst type according to the coverage condition and the number of bits that need to be transmitted in each case [4][5]. The selections of MCS and CBS index for each burst type and for each coverage condition are shown in the Annex.

Latency Results

Normal Coverage (Coupling loss = 144 dB)

For the normal coverage condition, the typical length of the DCI interval is 160ms (2 frames). The selected MCS and CBS index for each burst to achieve this coverage is shown in the Annex. The synchronisation time breakdown is also shown in the Annex. The resulting latency for exception reporting is shown in Table 1.

Table 1. Latency for Exception Report in Normal Coverage

| Normal coverage | Number of repetitions | Time per repetition (ms) | Total time (ms) |
| --- | --- | --- | --- |
| Synchronization | - | - | 400 |
| Twait_1 | - | - | 40 |
| Channel Request | 1 | 40 | 40 |
| Twait_2 | - | - | 80 |
| DCI for RAR and UL Allocation | 1 | 80 | 80 |
| Twait_3 | - | - | 30 |
| RAR | 1 | 20 | 20 |
| Twait_4 | - | - | 40 |
| UL DATA | 1 | 240 | 240 |
| Total latency |  |  | 970 ms |

10 dB Extended Coverage (Coupling loss = 154 dB)

For the 10 dB extended coverage condition, the typical length of the DCI interval is 640ms (8 frames). The selected MCS and CBS index for each burst to achieve this coverage is shown in the Annex. The synchronisation time breakdown is also shown in the Annex. The resulting latency for exception reporting is shown in Table 2.

Table 2. Latency for Exception Report in 10 dB extended Coverage

| 10 dB extended coverage | Number of repetitions | Time per repetition (ms) | Total time (ms) |
| --- | --- | --- | --- |
| Synchronization | - | - | 640 |
| Twait_1 | - | - | 40 |
| Channel Request | 3 | 40 | 120 |
| Twait_2 | - | - | 320 |
| DCI for RAR and UL Allocation | 1 | 320 | 320 |
| Twait_3 | - | - | 120 |
| RAR | 1 | 80 | 80 |
| Twait_4 | - | - | 40 |
| UL DATA | 1 | 920 | 920 |
| Total latency |  |  | 2600 ms |

20 dB Extended Coverage (Coupling loss = 164 dB)

For the 20 dB extended coverage condition, the typical length of the DCI interval is 2560ms (32 frames). The selected MCS and CBS index for each burst to achieve this coverage is shown in the Annex. The synchronisation time breakdown is also shown in the Annex. The resulting latency for exception reporting is shown in Table 3.

Table 3. Latency for Exception Report in 20 dB extended Coverage

| 20 dB extended coverage | Number of repetitions | Time per repetition (ms) | Total time (ms) |
| --- | --- | --- | --- |
| Synchronization | - | - | 1120 |
| Twait_1 | - | - | 80 |
| Channel Request | 8 | 40 | 320 |
| Twait_2 | - | - | 1280 |
| DCI for RAR and UL Allocation | 4 | 320 | 1280 |
| Twait_3 | - | - | 480 |
| RAR | 4 | 80 | 320 |
| Twait_4 | - | - | 40 |
| UL DATA | 3 | 920 | 2760 |
| Total latency |  |  | 7680ms |

Conclusion

In this contribution, the exception report latency has been analysed for the NB M2M solution. Results are presented for 0 dB, 10 dB and 20 dB coverage extension compared with legacy GPRS. The results indicate that the 10 second latency target for exception reporting can be achieved even with 20 dB coverage extension.

References

  - GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, VODAFONE Group Plc., GERAN#62.

  - Draft_3GPP TR 45 820 V0 3 0_including agreements up to GERAN#64_clean.

  - FS_IoT_LC_Telco#8 “Evaluation of Latency for MAR Exception Report”, HuaWei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd.

  - GPC150022, ” NB M2M - Design of Modulation and Coding Schemes”, HuaWei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd., 3GPP TSG GERAN1 Adhoc#1 on FS_IoT_LC.

  - GP-150085, “NB M2M - Optimized Uplink Pilot Design and CBS Tables”, HuaWei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd., 3GPP TSG GERAN#65.

  - GPC-150033, “NB M2M – Evaluations of Cell Search”, Huawei Technologies Co., Ltd., HiSilicon Technologies Co., Ltd, Cellular IoT Ad-Hoc#1.

  - GP-150073, “ NB M2M – Battery Life Analysis”, Neul Ltd., 3GPP TSG GERAN#65.

Annex

Table 4 shows the selected MCS and CBS index [4][5] for each burst type used for exception reporting under each coverage condition. Class-B uplink modulation (GMSK) is assumed [2]. The corresponding total duration of each burst is also shown (including any repetitions). Further details may be found in [7].

Table 4. Selection of MCS and CBS indexes for each burst type

|  |  | Coupling loss = 144 dB |  |  | Coupling loss = 154 dB |  |  | Coupling loss = 164 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Burst type | PHY burst size | MCS Index | CBS index | Duration (ms) | MCS index | CBS index | Duration (ms) | MCS index | CBS index | Duration (ms) |
| DL DCI | UE scheduled (48 bytes) | 5 | 7 | 80 | 3 | 7 | 320 | 1 | 7 | 1280 |
| DL data | RAR (9 bytes) | 5 | 1 | 20 | 3 | 1 | 80 | 1 | 1 | 320 |
| UL random access | RACH (5 bytes) | 5 | 0 | 40 | 3 | 0 | 120 | 1 | 0 | 320 |
| UL data (105 bytes) | Short report (95+10 = 105 bytes) | 7 | 11 | 120 | 5 | 22 | 920 | 3 | 22 | 2760 |

Table 5 shows the synchronization time breakdown under each coverage condition. The number of frames required for synchronization is based on two interferers and 90% confidence of timing accuracy to ±1/8 Tb and CFO accuracy to ±45 Hz [6] (neither of which are hard limits for subsequent decoding, so the 90% threshold is a conservative lower limit on successful reception of subsequent bursts).

System information reading is not included, in accordance with [2].

Table 5. Synchronization time breakdown

|  | Latency (in 80ms frames) |  |  |
| --- | --- | --- | --- |
|  | Coupling loss = 144 dB | Coupling loss = 154 dB | Coupling loss = 164 dB |
| PSS | 2 | 4 | 9 |
| SSS | 1 | 1 | 1 |
| FIIS | 2 | 3 | 4 |
| Total frames | 5 | 8 | 14 |
| Total latency | 400 ms | 640 ms | 1120 ms |