uPoD Paging Group Determination

# Introduction

How to realize extended DRX cycles for GSM was described in [1] at GERAN#64. This was further discussed within [2] at CIoT AdHoc #1 in the context of the cellular IoT study including extended coverage operation, and is now presented again within the context of uPoD where the determination of paging groups on the PCH channel is outlined for MTC devices.

# Paging Group Determination

When sending a paging request to a BSS, the SGSN includes an indication of the eDRX cycle and IMSI associated with the target device thereby allowing the BSS to determine the next occurrence of the nominal paging group for that device within its eDRX cycle as follows:

-	N is the number of paging groups within a given eDRX cycle and is determined based on EXTENDED_DRX_MFRMS and PCH_BLKS_MFRM where:

- EXTENDED_DRX_MFRMS is the number of 51-multiframes per eDRX cycle determined as per Table 1 below.

- PCH_BLKS_MFRM is a legacy parameter and indicates the number of PCH blocks (i.e. the number of 4 bursts blocks) per 51-multiframe. For uPoD this value can range from 1 to 9.

-	The set of eDRX cycle lengths identified by Table 1 is selected such that each member of the set occurs an integral number of times within the full TDMA FN space.

-	N  = PCH_BLKS_MFRM x EXTENDED_DRX_MFRMS.

-	Nominal paging block = mod (IMSI, N).

Table 1. Set of eDRX Cycles Supported

| eDRX Cycle Value (EXTENDED_DRX) | Target eDRX Cycle Length | Number of 51-MF per eDRX Cycle (EXTENDED_DRX_MFRMS) | eDRX Cycles per TDMA FN Space |
| --- | --- | --- | --- |
| 0 | ~24.5 seconds | 104 | 512 |
| 1 | ~49 seconds | 208 | 256 |
| 2 | ~1.63 minutes | 416 | 128 |
| 3 | ~3.25 minutes | 832 | 64 |
| 4 | ~6.5 minutes | 1664 | 32 |
| 5 | ~13 minutes | 3328 | 16 |
| 6 | ~26 minutes | 6656 | 8 |
| 7 | ~52 minutes | 13312 | 4 |
| Note 1: 53248 51-multiframes occur with the TDMA FN space (2715648 TDMA frames) Note 2: All remaining EXTENDED_DRX values are reserved |  |  |  |

Example 1:

- IMSI = 00000000  10001001  00110000  00000001 = 4796417, EXTENDED_DRX_MFRMS = 6656 (i.e. the eDRX cycle ~ 26 minutes) and PCH_BLKS_MFRM = 8.

- N = 8 * 6656 = 53248

- Nominal Paging Group = mod (IMSI, 53248) = 4097 which occurs in the 4098th PCH block of the eDRX cycle (i.e. in the 2nd PCH block in 51-multiframe #513.

# Conclusion

In light of the discussion herein the uPoD TR should be updated to indicate uPoD devices shall make use of the set of eDRX cycles in Table 1 along with the nominal paging group determination method described above (see the proposed pCR in [3]).

# References

GP-140894, “Supporting eDRX for uPoD”, source Ericsson LM. TSG GERAN #64

GPC150086, “EC-GSM - Paging Group Determination”, source Ericsson LM. GERAN Ad Hoc#1 on FS_IoT_LC.

[3] GP-150130,  “pCR - uPoD Paging Group Determination”, source Ericsson LM, TSG GERAN #65