# pCR, Identifying uPoD devices

# Introduction

As discussed in [1] the legacy code point ‘100’ can be re-used for uniquely identifying RACH requests from MTC devices that want to extend their battery lifetime. A corresponding pCR is needed to capture this functionality in the uPoD TR.

# pCR to uPoD TR

The following change to the uPoD TR (GP-140968_Technique Report for FS_uPoD-v030) is proposed to allow a BSS to identify an access request from a device requiring Power Efficient Operation.

First modification (modified sub-clause)

## 6.5	One Phase Access Requests from MTC Devices

A MTC device attempting system access in a cell that supports power efficient operation shall send an EGPRS Packet Channel Request indicating “PEO One Phase Access Request” as shown in Table 6.5-1 below. This allows the BSS to manage such devices with the knowledge that they may support a different set of features compared to legacy devices or even other types of MTC devices.

Table 6.5-1: EGPRS PACKET CHANNEL REQUEST message content

< EGPRS Packet channel request message content > ::= < One Phase Access Request : 0 < MultislotClass : bit (5) > < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < PEO One Phase Access Request : 100 < Spare : bit (4) > < Priority : bit (1) > < RandomBits : bit (3) > > ¦ < One Phase Access Request by Reduced Latency MS: 101 < MultislotClassGroup : bit (3) > < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Two Phase Access Request : 110000 < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Signalling : 110011 < RandomBits : bit (5) > > ¦ < One phase Access Request in RLC unack mode : 110101 < RandomBits : bit (5) > > ¦ < Dedicated Channel Request : 110110 < RandomBits : bit (5) > > ¦ < Emergency call : 110111 < RandomBits : bit (5) > > ¦ < Two Phase Access Request by IPA capable MS: 111000 < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Signalling by IPA capable MS : 111001 < RandomBits : bit (5) > > ;

End of modifications

# References

[1] GP-150159, “Identifying uPoD Devices”, Ericsson LM, TSG GERAN #65.