# EC-GSM, PACCH Header Optimization

# Introduction

At GERAN#62 a new feasibility study named Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things  [1] (WI code: FS_IoT_LC)  was approved.

In [2] the general RLC procedures applicable to GSM were reviewed and a proposal for a modified RLC data block header applicable to EC-GSM was described in an effort to reduce the scope/complexity for EC-GSM devices. Similarly, in this contribution a modified EC-PACCH block header is proposed to reduce the scope/complexity for EC-GSM devices.

# Uplink RLC/MAC control block

The legacy uplink RLC/MAC control block together with its MAC header is formatted as shown in figure 1. Each field is considered with respect to its applicability to EC-GSM and a modified uplink RLC/MAC control block for EC-GSM is shown in Figure 2.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload Type |  |  |  | spare |  |  | R | MAC header |
|  |  |  |  |  |  |  |  | Octet 1 |
| Control Message Contents |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | Octet 23 |

Figure 1: Legacy Uplink RLC/MAC control block together with its MAC header

- Payload Type field: This field is not needed for EC-GSM since stealing flags indicating CS-1 will be uniquely interpreted as a EC-PACCH block by the BSS.

- Retry field: This field can be maintained for EC-GSM.

| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| spare |  |  |  |  |  |  | R | MAC header |
|  |  |  |  |  |  |  |  | Octet 1 |
| Control Message Contents |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | Octet 23 |

Figure 2: EC-GSM Uplink RLC/MAC control block together with its MAC header

# Downlink RLC/MAC control block

The legacy downlink RLC/MAC control block together with its MAC header is formatted as shown in figure 1. Each field is considered with respect to its applicability to EC-GSM and a modified downlink RLC/MAC control block for EC-GSM is shown in Figure 4.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Payload Type |  | RRBP |  | S/P |  | USF |  | MAC header |
| RBSN | RTI |  |  |  |  | FS | AC | Octet 1 (optional) |
| PR |  | TFI |  |  |  |  | D | Octet 2 (optional) |
| RBSNe |  |  | FSe | spare |  |  |  | Octet 2/3 (optional) see note |
|  |  |  |  |  |  |  |  | Octet M |
| Control Message Contents |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  | Octet 22 |
| NOTE: This optional octet is included in case of extended RLC/MAC control message segmentation. Its presence is indicated through the combination of RBSN bit and FS bit equal to (RBSN=’1’ and FS=’0’) |  |  |  |  |  |  |  |  |

Figure 3: Legacy Downlink RLC/MAC control block together with its MAC header

- Payload Type field: Since stealing flags indicating CS-1 will be uniquely interpreted as a EC-PACCH block by a CIoT device this field is not needed.

- RRBP, S/P fields: These two fields are merged to create a single 2 bit polling (P) field used to indicate when a device has been polled and if polled where it is to begin transmitting the corresponding UL EC-PACCH message (as in [2]). However, further investigation of a suitable size for P is foreseen.

- USF field: This field can be maintained as per legacy operation.

- RBSN field: This field can be maintained as per legacy operation since a DL EC-PACCH message will require a maximum of two EC-PACCH blocks.

- RTI field: This field can be deleted since EC-PACCH messages will be 2 blocks long at most and therefore any given EC-PACCH message in progress must be completed before a new one is started.

- FS field: The functionality of this field can be maintained to allow a device to determine when a 1 block or 2 block DL EC-PACCH message is sent but is renamed Final Block Indicator (FBI).

- Address Control (AC) field: This field can be deleted since the presence of an octet including TFI will always be needed for EC-GSM.

- Power Reduction (PR), TFI fields: These fields can be maintained as per legacy operation.

- D field: This field is not needed since the establishment of a DL TBF will always result in the release of an UL TBF (and vice versa).

- RBSNe, FSe fields: These fields can be deleted as they are only needed for extended segmentation feature of the legacy DL PACCH wherein up to 9 PCCH blocks can comprise a single PACCH message. With this deletion the entire optional octet 2/3 can be removed from the header.

| 8 | 7 |  | 6 |  | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| spare |  | P |  |  | PR |  |  | USF |  | MAC header |
| RBSN |  | spare |  | TFI |  |  |  |  | FBI | Octet 1 |
|  |  |  |  |  |  |  |  |  |  | Octet 2 |
| Control Message Contents |  |  |  |  |  |  |  |  |  | … |
|  |  |  |  |  |  |  |  |  |  | Octet 9 |

Figure 4: EC-GSM Downlink RLC/MAC control block together with its MAC header

# Conclusions

Proposed modifications to the EC-PACCH header are provided above and reflect the following EC-GSM principles of operation:

- UL EC-PACCH messages shall be sent using a single EC-PACCH block.

- DL EC-PACCH messages shall be sent using up to 2 EC-PACCH blocks.

- The TFI field shall be present in all DL EC-PACCH blocks.

An accompanying pCR to introduce the EC-PACCH block formats applicable to EC-GSM is provided in [3].

# References

GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things (FS_IoT_LC) (revision of GP-140418)”, source VODAFONE Group Plc. GERAN#62.

GPC150083, “EC-GSM, RLC Procedures”, source Ericsson, GERAN Ad Hoc#1 on FS_IoT_LC.

GP-150176, “pCR for EC-GSM PACCH Header Optimization”, source Ericsson, GERAN#65.