# EC-GSM, EC-SCH design, performance and mapping

# Introduction

In this contribution the Synchronization Channel (SCH) performance in extended coverage is investigated for stationary devices. The target of the investigation is to support “extended coverage of 20 dB compared to legacy GPRS (Non EGPRS)” [1] for the Extended Converge for GSM (EC-GSM) solution, earlier referred to as GSM Evolution, using repetition based transmission schemes. To distinguish the Synchronization Channel in EC-GSM mode of operation from the regular SCH, the EC-GSM SCH will from now on be referred to as the EC-SCH.

In its current form the SCH conveys a Reduced TDMA frame Number (RFN) that signals the position of the SCH burst in the 51-multiframe structure. This means that the content of the SCH changes for each transmission. As the repetition based schemes investigated in this contribution requires that the content of the EC-SCH remains constant over the repetition period a redesign of the RFN information element is presented as part of the investigations.

This contribution is an update of [11] presented at the 1st GERAN1 Adhoc on FS_IoT_LC. Major updates have in the following been highlighted using red font color.

# Coverage improvment

## Maximum Coupling Loss

One of the aims of the feasibility study FS_IoT_LC [1] is to improve coverage by 20 dB compared to legacy GPRS.  At the GERAN#64 a working assumption was agreed on a legacy GPRS MCL reference value of 144 dB given a MS Noise figure of 5 dB.  Adding 20 dB of coverage extension to the MCL of 144 dB leads to a coverage requirement of 164 dB. The target of this investigation is therefore to explore the feasibility of extending EC-SCH coverage beyond a MCL of 164 dB.

## Repetition based transmission

To achieve a coverage improvement beyond 20 dB it is obvious that the receiving end must cope with negative signal to noise ratios (SNR). Without redefining the channel coding of the EC-SCH the channel can be made more robust trough repetition based transmission. It will be shown that 14 blind transmissions are needed to reach beyond the ambitious target of 20 dB coverage enhancement. At the transmitter side the repetition, or blind transmission, is straightforward, the BTS sends the same EC-SCH single burst radio block over the air on the defined EC-SCH positions in the 51-multiframe. At the receiving side the device is in this investigation blindly accumulates partitions of the repeated bursts in the IQ domain coherently, and sends the aggregated signal from each partition to the equalizer that produces soft bits. The soft bits from all partitions are combined and forwarded to the channel decoder.

Important to remember is that there will be devices requiring less than the full set of blind transmissions to read the EC-SCH. However since the EC-SCH is a broadcast channel it must be designed to support devices in the worst expected coverage. So an aspect to take note of is that while the BSS will convey the EC-SCH over e.g. 14 blind transmission, the devices in a network will be able to decode the EC-SCH after anywhere from one up to 14 blind transmission. It can further be noted that if the device is not in extended coverage the legacy procedure for FCCH and SCH on TS0 on the BCCH carrier can be followed to achieve cell synchronization.

## Frequency synchronization

In [4] it is shown that a device in extended coverage with high likelihood will meet the frequency accuracy requirement of 0.1 ppm [9] already after reading the FCCH for coverage extensions up to 20 dB.

The EC-SCH is used to further improve the frequency estimate from the FCCH, to secure that the 0.1 ppm is met. To study the feasibility of estimating the EC-SCH carrier frequency a low complexity method for frequency tracking has been investigated as part of this contribution. The method is based on the observation that a frequency offset leads to a phase drift in the receiver that can be estimated by studying the received samples in consecutive bursts.

# EC-SCH definition

## Mapping on 51-multiframe structure

The legacy SCH is defined as a single burst radio block, which is mapped on time slot (TS) 0 and TDMA frame 1, 11, 21, 32 and 41 in the 51-multiframe on of the Broadcast control channel (BCCH) frequency [3].

It will be shown in section 4.2 that 14 blind transmissions are needed to achieve coverage improvement of 20 dB. To not disturb GSM legacy operation it is proposed to map the EC-SCH for extended coverage onto TS1. At the same time it is indicated in [4] that operation of the FCCH is feasible already with today’s mapping on TS 0, and no support for FCCH is expected to be needed on TS 1. So a straightforward design of the EC-SCH mapping on TS 1 would be to use a compact mapping where the complete set of EC-SCH blocks is transmitted over the first seven TDMA frames in two consecutive 51-multiframes, as depicted in Figure 1.

Figure 1 Compact mapping of EC-SCH on 51-multiframe on TS1.

## Design of EC-SCH

The SCH contains per its current definition a 19 bit RFN defined as follows [3];

“T1		(11 bits)	range 0 to 2047	= FN div ( 26 x 51)

T2		(5 bits)	range 0 to 25		= FN mod 26

T3' 	(3 bits)	range 0 to 4		= (T3 - 1) div 10

where

T3		(6 bits)	range 0 to 50		= FN mod 51”.

The device interprets the conveyed FN according to [9]

FN	 =	51 x((T3-T2) mod 26) + T3 + 51 x 26 x T1

where

T3 = (10 x T3') + 1.

T1 defines the position of the GSM superframe within the hyperframe. (T3-T2)mod(26) defines the position of the 51-multiframe within the T1 superframe. T3 finally determines the frame position within the 51-multiframe.

Primarily a device needs to acquire the RFN to identify the occurrence of its paging group [3]. When considering that the set of extended DRX (eDRX) cycles proposed for EC-GSM at maximum covers 13312 51-multiframes (~52 minute eDRX cycle), or 1/4th of the entire TDMA FN space (i.e. a quarter hyperframe), it is realized that for EC-GSM it is sufficient to convey T1’ covering a range of 0 to 511 superframes;

T1	‘	(9 bits)	range 0 to 511	= FN div ( 26 x 51 x 4)

Note that there are 512 superframes per quarter hyperframe. Immediately after accessing the network, the device will receive an immediate assignment containing an indication of what quarter hyperframe the last part of the assignment message is received in, hence filling in the missing two bits not provided by the 9 bit T1’ field required to determine the position of a GSM superframe within a hyperframe as per the legacy T1 field [13].

Then if considering the design of the EC-SCH and that a single instance of the EC-SCH is transmitted over two consecutive 51-multiframes T2 can be modified to signal on which pair of 51-multiframes in the superframe the instance of the EC-SCH is transmitted. T2 to be included in the EC-SCH can hence be redefined as;

T2’		(4 bits)	range 0 to 12		= (FN –T1’*26*51) div 102

Devices requiring seven or less blind transmissions need to be able to identify if the EC-SCH is transmitted over an even or an odd numbered 51-multiframe in order to minimize the monitoring time of the EC-SCH. Devices requiring eight or more blind transmissions need an indication of between which 51-multiframe borders the EC-SCH information is changing to avoid blindly decoding two different EC-SCH radio blocks, i.e. EC-SCH blocks not belonging to the same pair of 51-multiframes. It is therefore proposed to modulate the EC-SCH using a modified GMSK modulation with a negative modulation index h equal to -½ on odd numbered 51-multiframes. This can be compared with today’s GMSK implementation where a modulation index of ½ is used [5]. By detecting the modulation index, using de-rotation of /2 or -/2, a device can determine if a EC-SCH burst is transmitted over an even or odd numbered 51-multiframe, exemplified in Figure 2.

Figure 2 Modulating EC-SCH with GMSK using a negative modulation index on odd 51-multiframes.

From the BTS perspective implementation of using negative modulation index is of low complexity, and can be achieved e.g. through applying the complex conjugate to the output of an already existing GMSK modulator.

T3’ is finally proposed not to be included in the EC-SCH content, as a device in extended coverage is expected to detect which TDMA frame number (FN) an EC-SCH instance in the 51-multiframes is configured on when considering  the asymmetric distribution of FCCH and EC-SCH blocks over the 51-multiframe on TS 0 and 1, respectively.

With these new definitions for the content of the EC-SCH a device will be able to determine the TDMA FN within a quarter hyperframe, denoted QHFN, as;

QHFN	 =	(51 x 26 x T1’) + (2 x 51 x T2’ + 51 x T2’’) + T3’’

where

T1	‘		(9 bits)	range 0 to 511	= FN div ( 26 x 51 x 4)

T2’		(4 bits)	range 0 to 12		= (FN –T1’*26*51) div 102

T2’’ is signaled trough the detected GMSK modulation index (gmi) and defined as

T2’’(gmi = ½) = 0, for even 51-multiframes,

T2’’(gmi = -½) = 1 for odd 51-multiframes.

T3’’ will be determined through the mapping of the FCCH onto specific TDMA frames within 51-multiframes sent on TS0 and will be an element in the set {0, 10, 20, 30, 40}, see [4].

With this redefinition the EC-SCH RFN will be signaled using 13 bits, effectively creating six spare bits compared to the current SCH RFN definition. By using these spare bits, it is proposed to support the signaling of the single bit (EC-)CCCH TS0 and TS1 mapping information [14], the two bit Implicit Reject Status information applicable to CIoT devices [15], and the two bit the BCCH_CHANGE_MARK information in the EC-SCH [6]. In case of the BCCH_CHANGE_MARK two bits of information would probably be sufficient given the compressed system information proposed in [10] for Internet of Things.  With the signaling of Implicit Reject within the EC-SCH a devices do not have to read the Paging Channel (EC-PCH) or Access Grant Channel (EC-AGCH) to realize that it is restricted from network access, and with the signaling of BCCH_CHANGE_MARK information within the EC-SCH the device do not have to read the EC-BCCH to understand that the system information must be reread. This is preferred since neither the EC-PCH nor the EC-AGCH are dimensioned for the worse coverage class expected, but rather the coverage class of the devices to which the block is addressed. Hence, this information will be at hand already when synchronizing to the cell. After this redefinition of the EC-SCH content a single spare bit remains for future use.

# Simulations

EC-SCH simulation has been carried out for stationary devices using 14 blind transmissions using the frame mappings presented in section 3.1. The below sections highlight some important simulation assumptions and results.

## Simulation assumptions

To facilitate link level simulations the required coverage improvement has been translated into an Es/N0 requirement using the following MCL calculation as baseline;

MCL=PBTS-N0+NFMS+EsN0dB

Where;

- The base station output power BTS=43 dBm

- The  mobile noise figure NFMS=5 dB

- The thermal noise is defined as N0=-174+10log10BWsim dBm, where the simulator bandwidth BWsim=13*106/48 kHz.

Requiring the MCL to reach 164 dB implies that the device shall be able to decode the SCH at an Es/N0 of;

EsN0=43+119.7-5-164=- 6.3 dB

One can either use the simulator bandwidth, i.e. the bandwidth over which the used simulator defines Es/N0, or the mobile receiver bandwidth to calculate the thermal noise. If the latter definition is used then the definition of Es/N0, or rather the SNR after receive filtering needs to be corrected accordingly.

Other simulation settings are presented in below table. The assumed frequency offset was simulated as normally distributed with a mean of 8 Hz and a standard deviation of 45 Hz, based on the results in [4].

Table 1.  Simulation assumptions.

| Parameter | Value | Comment |
| --- | --- | --- |
| MCS | EC-SCH | Redefinition of RFN assumed. |
| Channel | TU [2] | According to agreed assumption in FS_LC_MTC_LTE feasibility study. |
| Mobile speed | 1.2 km/h | Assumed to model stationary mobile in somewhat mobile environment. Corresponds to max Doppler shift of 1Hz. |
| Band | GSM 900 |  |
| Impairments | Typical RX and TX. Frequency offset following a normal distribution N(8,45) Hz. | See e.g. [7] for details on typical settings. Frequency offset chosen based on results in [4]. |
| AFC | Range of possible frequency offsets detected [-100,100] Hz |  |
| Number of repetitions | 14 | For mapping onto the 51-multiframe, see Figure 1 |
| Blind modulation detection | On | Detection of GMSK modulation index. |

## Simulated results

Results are shown in Figure 3 where the MCL requirement of 164 dB is equivalent to an Es/N0 of -6.3 dB. At 14 blind transmissions an Es/N0 of -6.9 dB, is achieved at 10% Data BLER. Blind IQ accumulation was done over groups of seven EC-SCH bursts, followed by equalization and soft combining. This confirms the feasibility of network synchronization in case of the EC-GSM track in the FS_IoT_LC study.

Figure 3. SCH BLER performance for 14 blind transmissions.

In parallel to the equalization the mobile receiver continuously estimates the EC-SCH carrier frequency over the full set of repetitions. Figure 4 contains a PDF over the frequency estimation error logged during a simulation where the Es/N0 was set to -6.3 dB to correspond to a MCL of 164 dB. It is seen that the frequency accuracy after reading the EC-SCH can be assumed to be of an accuracy ±10 Hz with a very high probability, and that a normal distribution characterized by a 3 Hz standard deviation can be used to model the accuracy. Accurate frequency estimate after synchronization is important as the error in the estimated frequency can be mapped to a frequency offset in the mobile transmitter which must be dealt with by the BTS during later packet uplink transfer, at least if some level of coherent accumulation of IQ samples is assumed to be performed.

This confirms that further investigations on uplink performance in the GSM evolution track in the FS_IoT_LC study can be performed with stringent assumption on typical UL carrier frequency offsets.

Figure 4 Frequency estimation error after reading EC-SCH at Es/N0 -6.3 dB.

# Summary

This contribution has shown that network synchronization at a MCL of 164 dB is achievable in case of the EC-GSM candidate in the FS_IoT_LC study. During the synchronization, frequency accuracy within ±10 Hz is achievable with high probability, which is important to take into account when studying UL performance in the FS_IoT_LC study.

The mentioned coverage extension is achieved through up to 14 blind transmissions of the EC-SCH over two consecutive 51-multiframes configured on TS1 of the BCCH carrier. To support the repeated transmission scheme the information content and the modulation of the EC-SCH have been proposed to be redesigned. The EC-SCH block definition in terms of channel coding used and block size is however left as currently defined for the SCH.

# References

GP-140241, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, GERAN#62, source VODAFONE Group Plc.

TR 36.888, v12.0.0, “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”, 3GPP RAN1

TS 45.002, v12.0.0, “Multiplexing and multiple access on the radio path, 3GG GERAN1

GPC150066, “EC-GSM, FCCH overview”, GERAN1 Adhoc#1 on FS_IoT_LC, source Ericsson

TS 45.004, v11.0.0, Modulation, 3GPP GERAN1

TS 44.018, v12.0.0, Radio Resource Control, 3GPP GERAN2

TR 45.860, v11.5.0, “Signal Precoding Enhancements for EGPRS2 DL”. 3GPP GERAN1

GP-140608, “Radio propagation channel for cellular IoT”, GERAN#63, source Ericsson

TS 45.010, v12.0.0, Radio Subsystem Synchronization, 3GPP GERAN1

GP-140603, “GSM Evolution for cellular IoT - BCCH overview”, GERAN#63, source Ericsson

GPC150054, “EC GSM, EC-SCH”, 1st GERAN1 FS_IoT_LC Adhoc, source Ericsson

GP-150133, “EC-GSM, Paging group determination”, GERAN#65, source Ericsson

GP-150167, “EC-GSM, AGCH assignment messages”, GERAN#65, source Ericsson

GP-150137, “EC-GSM, CCCH mapping on TS0 and TS1”, GERAN#65, source Ericsson

GP-150177, “EC-GSM, Overload control”, GERAN#65, source Ericsson