# Pseudo CR 43.869 – pCR - Energy Consumption Evaluation for RAU procedure

# 1	Introduction

## 1.1	Background Information

A study on “Power Saving for MTC Devices” was agreed in GERAN #60 [1]. In order to compare different solution proposals the individual contributions from various activities such as data transfer, Routing Area update etc. needs to evaluated and agreed.

## 1.2	Reason for change

The energy consumption for doing a RAU procedure has been evaluated and needs to be included in the TR.

## 1.3	Summary of change

The TR is updated to include the Energy consumption evaluation for RAU procedure [2].

## 1.4	References

[1]	GP-131136, “New SI Proposal: Study of Power Saving for MTC Devices”, China Mobile Communication Corporation, GERAN #60

[2]	GP-150161, “Energy Consumption Evaluation for RAU procedure”, source Ericsson LM, GERAN#65

# pCR to 3GPP TR 43.869-v0.3.0

First modification (modified sub-clause)

### 6.2.1 		Operating voltage and current

Typical values of average current at different operational modes (procedures) are tabulated in Table 3. It has been assumed that the same operating voltage is used in all operational modes and that it is the operating current that is varying based on the specific mode of operation in effect. The operating voltage is assumed to be 3.3 V.

Any use-case can be broken down into one or several instances of variable length intervals of the different operational modes:

- Transmission (Tx) - baseband and RF are engaged in data transmission.

- Reception (Rx) - baseband and RF are engaged in data reception.

- Monitoring - RSSI measurements are performed for serving/neighbour cells.

- Light sleep - MS is in wakeup state and waiting to perform impending tasks. This mode may be employed by applying Dynamic Voltage and Frequency Scaling (DVFS) schemes.

- Deep sleep - there is no activity for a long duration, modem enters into prolonged sleep (instead of staying in light sleep) by cutting down voltages and clocks to all modem sub-modules excepts RAM memory (for refreshment) and timer management (for interrupt generation).

Table 3 Typical values of average current during different Modem operations in an MS.

| Operation | Specification | Operating current (µA) |
| --- | --- | --- |
| Transmission (Tx) | Tx (GMSK) (33 dBm) | 1 393,528 |
|  | Tx (8-PSK) (27 dBm) | 350,000 |
| Reception (Rx) | Rx with Baseband processing | 30,000 |
|  | Monitoring (RSSI with 32 I,Q samples) | 20,000 |
| Power Saving States | Light sleep | 1,000 |
|  | Deep sleep | 1 |
|  | Power off(1) | 0.01 |

Note 1: It should be noted that in this state the modem is completely switched off, however some energy is consumed by the application running that is able to wake the modem up.

Second modification (modified sub-clause)

## 6.5.2	Energy Consumption for RAU procedure

One RAU can be broken down into several basic operations as specified in Table 6.5-a. For practical reasons a 33dBM transmit power is assumed in these calculations.

Figure 6.5-a provides a sequence of message exchanges for RAU procedure for the case where a 1 phase access is used for the initial access.

Figure 6.5-a: RAU  procedure

Details of the Energy Consumed for individual sub process with time taken for each is depicted in Table 6.5-a below.  The below table assumes the max TX power for transmission (33 dBm)

Table 6.5-a: Energy Consumption for RAU

| Sub Procedure | Energy Consumed (uJ) | Time taken(ms) |
| --- | --- | --- |
| RACH | 2652.411 | 0.577 |
| AGCH Idle(Full CCCH) (light sleep for 80 bursts) Note: practical BSS response time is no faster than 50ms. | 148.5 | 45 |
| Immediate Assignment (4 AGCH bursts) | 228.492 | 2.308 |
| AGCH Idle (Light sleep for 7 bursts after each of 4 AGCH bursts = 28 bursts) | 53.312 | 16.156 |
| PDCH Idle (light sleep for 28 bursts of TS not used to receive 1st USF) | 53.312 | 16.156 |
| USF Monitoring for sending RAU Req (1st block) using 4 bursts | 228.492 | 2.308 |
| RAU Req (1st RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions = 24 bursts). 1 TS is used for USF reception and 1 TS used for MS transmission every DL frame. | 45.7 | 13.848 |
| USF monitoring for RAU Req (2nd block) | 228.492 | 2.308 |
| RAU Req (2nd RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| USF monitoring for RAU Req (3rd block) | 228.492 | 2.308 |
| RAU Req (3rd RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| USF monitoring for RAU Req (4th block) | 228.492 | 2.308 |
| RAU Req (4th RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| DL L2 ACK on PACCH (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) Note: PUAN is assumed to Ack all 4 data blocks. | 53.312 | 16.156 |
| Idle (light sleep on 7 TS per frame for 100 TDMA frames after sending the RAU Request until the DL Assignment on PACCH) Note: Allows ~462ms for RAU Accept to arrive at BSS. | 1332.54 | 403.9 |
| DL PACCH monitoring on DL TS corresponding to assigned UL TS until beginning of the DL Assignment (100 bursts) | 5712.3 | 57.7 |
| DL PACCH DL Assignment (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) | 53.312 | 16.156 |
| RAU Accept (1st RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PDCH burst receptions = 28 bursts) | 53.312 | 16.156 |
| RAU Accept (2nd RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| RAU Accept (3rd RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| RAU Accept (4th RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| PDCH Idle (Light sleep while waiting to send UL L2 Ack after RRBP=13). Note: MS in light sleep for 7 of 8 DL TS over 12 TDMA frames and for all 8 TS on the 13th TDMA frame) | 175.177 | 53.084 |
| USF/TFI monitoring during this time. | 685.476 | 6.924 |
| UL L2 ACK on PACCH (4 bursts Tx). Note: PDAN sent according to RRBP. | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of 4 UL PACCH burst transmissions = 24 bursts). 1 TS used for USF decoding and 1 TS used for MS transmission per TDMA frame | 45.7 | 13.848 |
| USF Monitoring for RAU Complete Note: USF assumed to be received while sending PDAN. | 228.492 | 2.308 |
| RAU Complete (4 bursts TX) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of 4 PDCH burst transmissions = 24 bursts) | 45.7 | 13.848 |
| PDCH Idle for around 10 TDMA frames after transmission of 4th RAU Complete burst (until reception of PUAN excluding the time for TFI/USF monitoring) | 133.254 | 40.38 |
| TFI/USF monitoring till the reception of DL L2 ACK on PACCH | 571.23 | 5.77 |
| DL L2 ACK on PACCH (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) | 53.312 | 16.156 |

Total Energy Consumed for the whole procedure (completed in around 890 ms) is 78.7mJ. It should be noted that cell reselection measurements are not included.

The above data is taken from a network log and the configuration is as follows.

- MS would never move out of Packet transfer state throughout the duration of RAU procedure.

- No UL or DL dummy control block sent or received. In addition it is assumed that Release 6 Extended UL TBF is used both by MS and network(i.e after CV=0 is acknowledged by network, MS would not send any UL Dummy control block for matching USF)

- CS1 used for the NAS messages sent and received

The plot of Power consumed vs Time is depicted in Figure 6.5-b below.

Figure 6.5-b: Power Consumption sequence for RAU

The Energy Consumption for Measurements are ignored as per 3GPP TS45.002 since measurements are done only when MS is doing some activity (TX or RX). The rationale is that the energy/power consumed for measurements during this procedure in Packet transfer mode is not that significant.

End of modifications