# Consolidated Ericsson uPoD pCRs

# Introduction

The set of Ericsson uPoD pCRs agreed in GP-150130, GP-150245, GP-150242, GP-150243 and GP-150206 have been consolidated into a single uPoD pCR as shown below.

First modification (modified sub-clause)

## 7.1		Extended DRX

Support for uPoD Use Case 1 (Network Triggered Reporting) requires that MTC devices support a power saving mechanism for which reachability can be based on either eDRX or PSM.  The desired periodicity of reachability along with the expected rate of device triggering will be used to determine the method of reachability used. The use of eDRX based reachability is intended to target operational scenarios for which battery lifetimes can be extended beyond what would be possible using PSM based reachability.

### 7.1.1		Paging Group Determination

When sending a paging request to a BSS, the SGSN includes an indication of the eDRX cycle and IMSI associated with the target device thereby allowing the BSS to determine the next occurrence of the nominal paging group for that device within its eDRX cycle as follows:

-	N is the number of paging groups within a given eDRX cycle and is determined based on EXTENDED_DRX_MFRMS and PCH_BLKS_MFRM where:

- EXTENDED_DRX_MFRMS is the number of 51-multiframes per eDRX cycle determined as per Table 7.1-1 below.

- PCH_BLKS_MFRM is a legacy parameter and indicates the number of PCH blocks (i.e. the number of 4 bursts blocks) per 51-multiframe. For uPoD this value can range from 1 to 9.

-	The set of eDRX cycle lengths identified by Table 7.1-1 is selected such that each member of the set occurs an integral number of times within the full TDMA FN space.

-	N  = PCH_BLKS_MFRM x EXTENDED_DRX_MFRMS.

-	Nominal paging block = mod (IMSI, N).

Table 7.1-1. Set of eDRX Cycles Supported

| eDRX Cycle Value (EXTENDED_DRX) | Target eDRX Cycle Length | Number of 51-MF per eDRX Cycle (EXTENDED_DRX_MFRMS) | eDRX Cycles per TDMA FN Space |
| --- | --- | --- | --- |
| 0 | ~24.5 seconds | 104 | 512 |
| 1 | ~49 seconds | 208 | 256 |
| 2 | ~1.63 minutes | 416 | 128 |
| 3 | ~3.25 minutes | 832 | 64 |
| 4 | ~6.5 minutes | 1664 | 32 |
| 5 | ~13 minutes | 3328 | 16 |
| 6 | ~26 minutes | 6656 | 8 |
| 7 | ~52 minutes | 13312 | 4 |
| Note 1: 53248 51-multiframes occur with the TDMA FN space (2715648 TDMA frames) Note 2: All remaining EXTENDED_DRX values are reserved |  |  |  |

Second modification (modified sub-clause)

## 7.2		One Phase Access Requests from MTC Devices

A MTC device attempting system access in a cell that supports power efficient operation shall send an EGPRS Packet Channel Request using the code point ‘100’  as shown, for example, in Table 7.2-1 below. This allows the BSS to manage such devices with the knowledge that they may support a different set of features compared to legacy devices or even other types of MTC devices. In addition, the use of this code point allows the BSS to realize it has received an access request from a device requiring power efficient operation in which case uplink TBF establishment shall be performed using a one phase access (i.e. the use of the legacy One Phase Access Request code does not mandate that the BSS make use of a one phase access).

Table 7.2-1: EGPRS PACKET CHANNEL REQUEST message content

< EGPRS Packet channel request message content > ::= < One Phase Access Request : 0 < MultislotClass : bit (5) > < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < PEO One Phase Access Request : 100 < Spare : bit (4) > < Priority : bit (1) > < RandomBits : bit (3) > > ¦ < One Phase Access Request by Reduced Latency MS: 101 < MultislotClassGroup : bit (3) > < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Two Phase Access Request : 110000 < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Signalling : 110011 < RandomBits : bit (5) > > ¦ < One phase Access Request in RLC unack mode : 110101 < RandomBits : bit (5) > > ¦ < Dedicated Channel Request : 110110 < RandomBits : bit (5) > > ¦ < Emergency call : 110111 < RandomBits : bit (5) > > ¦ < Two Phase Access Request by IPA capable MS: 111000 < Priority : bit (2) > < RandomBits : bit (3) > > ¦ < Signalling by IPA capable MS : 111001 < RandomBits : bit (5) > > ;

Third modification (modified sub-clause)

### 6.2.1 		Operating voltage and current

Typical values of average current at different operational modes (procedures) are tabulated in Table 3. It has been assumed that the same operating voltage is used in all operational modes and that it is the operating current that is varying based on the specific mode of operation in effect. The operating voltage is assumed to be 3.3 V.

Any use-case can be broken down into one or several instances of variable length intervals of the different operational modes:

- Transmission (Tx) - baseband and RF are engaged in data transmission.

- Reception (Rx) - baseband and RF are engaged in data reception.

- Monitoring - RSSI measurements are performed for serving/neighbour cells.

- Light sleep - MS is in wakeup state and waiting to perform impending tasks. This mode may be employed by applying Dynamic Voltage and Frequency Scaling (DVFS) schemes.

- Deep sleep - there is no activity for a long duration, modem enters into prolonged sleep (instead of staying in light sleep) by cutting down voltages and clocks to all modem sub-modules excepts RAM memory (for refreshment) and timer management (for interrupt generation).

Table 6.2-1 Typical values of average current during different Modem operations in an MS.

| Operation | Specification | Operating current (µA) |
| --- | --- | --- |
| Transmission (Tx) | Tx (GMSK) (33 dBm) | 1 393,528 |
|  | Tx (8-PSK) (27 dBm) | 350,000 |
| Reception (Rx) | Rx with Baseband processing | 30,000 |
|  | Monitoring (RSSI with 32 I,Q samples) | 20,000 |
| Power Saving States | Light sleep | 1,000 |
|  | Deep sleep | 1 |
|  | Power off(1) | 0.01 |

Note 1: It should be noted that in this state the modem is completely switched off, however some energy is consumed by the application running that is able to wake the modem up.

Fourth modification (added sub-clause)

## 6.5.3	Energy Consumption for RAU procedure

One RAU can be broken down into several basic operations as specified in Table 6.5-a. For practical reasons a 33dBM transmit power is assumed in these calculations.

Figure 6.5-a provides a sequence of message exchanges for RAU procedure for the case where a one phase access is used for the initial access.

Figure 6.5-a: RAU  procedure

Details of the Energy Consumed for individual sub process with time taken for each is depicted in Table 6.5-1 below.  The below table assumes the max TX power for transmission (33 dBm)

Table 6.5-a: Energy Consumption for RAU

| Sub Procedure | Energy Consumed (uJ) | Time taken(ms) |
| --- | --- | --- |
| RACH | 2652.411 | 0.577 |
| AGCH Idle(Full CCCH) (light sleep for 80 bursts) Note: practical BSS response time is no faster than 50ms. | 148.5 | 45 |
| Immediate Assignment (4 AGCH bursts) | 228.492 | 2.308 |
| AGCH Idle (Light sleep for 7 bursts after each of 4 AGCH bursts = 28 bursts) | 53.312 | 16.156 |
| PDCH Idle (light sleep for 28 bursts of TS not used to receive 1st USF) | 53.312 | 16.156 |
| USF Monitoring for sending RAU Req (1st block) using 4 bursts | 228.492 | 2.308 |
| RAU Req (1st RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions = 24 bursts). 1 TS is used for USF reception and 1 TS used for MS transmission every DL frame. | 45.7 | 13.848 |
| USF monitoring for RAU Req (2nd block) | 228.492 | 2.308 |
| RAU Req (2nd RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| USF monitoring for RAU Req (3rd block) | 228.492 | 2.308 |
| RAU Req (3rd RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| USF monitoring for RAU Req (4th block) | 228.492 | 2.308 |
| RAU Req (4th RLC data block) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of the 4 PDCH burst transmissions) | 45.7 | 13.848 |
| DL L2 ACK on PACCH (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) Note: PUAN is assumed to Ack all 4 data blocks. | 53.312 | 16.156 |
| Idle (light sleep on 7 TS per frame for 100 TDMA frames after sending the RAU Request until the DL Assignment on PACCH) Note: Allows ~462ms for RAU Accept to arrive at BSS. | 1332.54 | 403.9 |
| DL PACCH monitoring on DL TS corresponding to assigned UL TS until beginning of the DL Assignment (100 bursts) | 5712.3 | 57.7 |
| DL PACCH DL Assignment (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) | 53.312 | 16.156 |
| RAU Accept (1st RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PDCH burst receptions = 28 bursts) | 53.312 | 16.156 |
| RAU Accept (2nd RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| RAU Accept (3rd RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| RAU Accept (4th RLC data block) | 228.492 | 2.308 |
| PDCH Idle (Light sleep) | 53.312 | 16.156 |
| PDCH Idle (Light sleep while waiting to send UL L2 Ack after RRBP=13). Note: MS in light sleep for 7 of 8 DL TS over 12 TDMA frames and for all 8 TS on the 13th TDMA frame) | 175.177 | 53.084 |
| USF/TFI monitoring during this time. | 685.476 | 6.924 |
| UL L2 ACK on PACCH (4 bursts Tx). Note: PDAN sent according to RRBP. | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of 4 UL PACCH burst transmissions = 24 bursts). 1 TS used for USF decoding and 1 TS used for MS transmission per TDMA frame | 45.7 | 13.848 |
| USF Monitoring for RAU Complete Note: USF assumed to be received while sending PDAN. | 228.492 | 2.308 |
| RAU Complete (4 bursts TX) | 10609.65 | 2.308 |
| PDCH Idle (Light sleep for 6 bursts after each of 4 PDCH burst transmissions = 24 bursts) | 45.7 | 13.848 |
| PDCH Idle for around 10 TDMA frames after transmission of 4th RAU Complete burst (until reception of PUAN excluding the time for TFI/USF monitoring) | 133.254 | 40.38 |
| TFI/USF monitoring till the reception of DL L2 ACK on PACCH | 571.23 | 5.77 |
| DL L2 ACK on PACCH (4 bursts Rx) | 228.492 | 2.308 |
| PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH burst receptions = 28 bursts) | 53.312 | 16.156 |

Total Energy Consumed for the whole procedure (completed in around 890 ms) is 78.7mJ. It should be noted that cell reselection measurements are not included.

The above data is taken from a network log and the configuration is as follows.

- MS would never move out of Packet transfer state throughout the duration of RAU procedure.

- No UL or DL dummy control block sent or received. In addition it is assumed that Release 6 Extended UL TBF is used both by MS and network(i.e after CV=0 is acknowledged by network, MS would not send any UL Dummy control block for matching USF)

- CS1 used for the NAS messages sent and received

The plot of Power consumed vs Time is depicted in Figure 6.5-b below.

Figure 6.5-b: Power Consumption sequence for RAU

The Energy Consumption for Measurements are ignored as per 3GPP TS45.002 since measurements are done only when MS is doing some activity (TX or RX). The rationale is that the energy/power consumed for measurements during this procedure in Packet transfer mode is not that significant.

Fifth modification (modified sub-clause)

## 6.1	Traffic model Attributes and Scenarios

Editor’s notes: this section is to describe the traffic model attributes used to identify use case scenarios of interest and to help for the evaluation of candidate GERAN enhancements applicable to use case scenarios of interest.

For traffic model definitions the following attributes are to be used to identify specific use case scenarios of interest:

- Mobility: this attribute defines the mobility behaviour of the MS which may have no mobility (stationary), medium mobility or high mobility (use case 1 and 2).

- MO Data frequency: the frequency at which mobile originated packet data transmissions are to be made for a mobile station (use case 2).

- MT triggering frequency: the frequency at which triggers are sent for a mobile station that operates according to use case 1 (i.e. page reception triggers the transmission of a report, a downlink TBF is not established). This may occur less frequently than allowed for by the periodicity of paging group monitoring.

- Latency: determines the minimum time length required from the point where uplink data becomes available in a MTC device to the point where uplink data transmission begins (use case 2), or how much time it takes for a MTC device to receive a page (trigger) from the point where a SGSN first determines a report is to be triggered (use case 1)

- UL/DL data size: Uplink or downlink packet data size transmitted or received by MTC devices where it is assumed the packet data is to be transmitted/received in consecutive radio blocks with MCS-2 (use case 1 and 2).

- Ready timer: Used in the MS and the network to control the cell updating procedure. While Ready timer is running the MS will monitor the CCCH as well as do necessary cell and/or routing area updates (T3314) (use case 1 and 2)..

- Non-DRX timer: Time duration where MS needs to continuously monitor AGCH/PCH blocks immediately after completing the transmission of a report (part of ready timer period) (use case 1 and 2).

Based on the use cases defined in section 5, and the above considered attributes, traffic model specific attributes are shown in Table 1 for the Network Triggered Traffic Model and in Table 2 for the Autonomous Reporting traffic model:

Table 6.1-1: Network Triggered traffic model

| Attribute | Value | Comment |
| --- | --- | --- |
| RA update periodic reporting interval | 1 day | Time between periodic RA updates (for the stationary case). If other values are needed is FFS |
| MT triggering frequency: | 5 min 1 hour 10 hours 1 day | The frequency at which triggers are sent for a mobile station that operates according to use case 1 (Network triggered reporting) |
| Packet trigger size | 60 byte (radio interface) | Size of the application payload used to trigger report |
| Mobility | Stationary | No cell change while in power saving state (i.e. only periodic RAU). |
|  | Medium mobility | FFS |
|  | High mobility | FFS |
| Uplink data size | 120 byte (radio interface) | Report to server |
| Downlink data size | 80byte (radio interface) | Report Ack (TCP or application Ack) |
| Ready timer | 20 sec | Used in the MS and the network to control the cell updating procedure. While Ready timer is running the MS will monitor the CCCH as well as do necessary cell and/or routing area updates (T3314) |
| Non-DRX timer | 2 sec | Time duration where MS needs to continuously monitor CCCH’s (part of ready timer period) |
| Active timer | 0 | If other values are needed is FFS |

Note that the packet size for the trigger sent in the NTR traffic model is based on the assumption that UDP is used

Table 6.1-2: Autonomous Reporting traffic model

| Attribute | Value | Comment |
| --- | --- | --- |
| RA update periodic reporting interval | N/A | Not applicable to this traffic model with the assumption of periodic reporting and corresponding reporting interval shorter than Periodic RAU timer. |
| MO Data frequency: | 5 min 1 hour 10 hours 1 day | Periodic reporting is assumed and defines the time between reports to the network |
| Mobility | Stationary | No cell change while in power saving state |
|  | Medium mobility | FFS |
|  | High mobility | FFS |
| Uplink data size | 120 byte (radio interface) | Report to server |
| Downlink data size | 80 byte (radio interface) | Report Ack (TCP or application Ack |
| Ready timer | 20 sec | Used in the MS and the network to control the cell updating procedure. While Ready timer is running the MS will monitor the CCCH as well as do necessary cell and/or routing area updates (T3314) |
| Non-DRX timer | 2 sec | Time duration where MS needs to continuously monitor CCCH’s (part of ready timer period) |
| Active timer | 0 | If other values are need is FFS |

Sixth modification (added sub-clause)

## 6.5.2	Energy Consumption for Data Transfer

#### 6.5.2.1 Energy Consumption for Network triggered Traffic model

In table 6.1-a the assumptions on the data traffic for the network triggered reporting Traffic model are summarized. To transmit one UL Report, we used 120 bytes of UL and 80 bytes DL Data. In addition to this we used a DL trigger of size 60 bytes, all as in table 6.1-a.

Figure 6.5-x: Data exchanges for a network triggered Report

Details of the energy consumed for individual sub process with time taken for each is depicted in Table 6.5-y below.  The calculated energy consumption in Table 6.5-y assumes the max TX power for transmission (33 dBm) and MCS-1 being used throughout.

Table 6.5-y: Energy Consumption for Data Transmission Network triggered USE Case

|  | Sub Procedure | Energy Consumed (uJ) | Time taken(ms) |
| --- | --- | --- | --- |
| 1 | PAGE | 228.492 | 2.308 |
| 2 | Idle time in between monitoring page(28 bursts) | 53.312 | 16.156 |
| 3 | Idle Light sleep(for 15.96 ms) | 52.67 | 15.96 |
| 4 | RACH | 2653.417 | 0.577 |
| 5 | AGCH Idle(Full CCCH) (light sleep for 80 bursts). Note: practical BSS response time is no faster than 50ms. | 148.5 | 45 |
| 6 | Immediate Assignment(4 bursts) | 228.492 | 2.308 |
| 7 | AGCH Idle (Light sleep for 7 bursts after each of 4 AGCH bursts = 28 bursts | 53.301 | 16.152 |
| 8 | PDCH Idle (light sleep for 28 bursts of TS not used to receive 1st USF) | 53.301 | 16.152 |
| 9 | USF Monitoring for sending page Response using 4 bursts | 228.492 | 2.308 |
| 10 | Page Response (Dummy LLC) | 10613.667 | 2.308 |
| 11 | PDCH Idle (Light sleep Light sleep for 7 bursts after each of 4 PDCH bursts = 28 bursts) | 53.312 | 16.156 |
| 12 | Idle for 10 ms | 33 | 10 |
| 12 | Packet TS Reconfigure | 228.492 | 2.308 |
| 13 | PDCH Idle (Light sleep for 28 bursts of TS not used for the reception of PTR) | 53.312 | 16.156 |
| 14 | Trigger Packet DL (3 RLC PDU’s) | 685.476 | 6.924 |
| 15 | PDCH Idle (Light sleep in between receiving these 3 RLC PDU’s) | 159.936 | 48.468 |
| 16 | PDCH Idle (Light sleep while waiting to send UL L2 Ack after 13 TDMA frames as indicated by the RRBP field). Note: MS in light sleep for 7 of 8 DL TS over 12 TDMA frames and for all 8 TS on the 13th TDMA frame) | 175.1739 | 53.083 |
| 17 | TFI monitoring during this time. | 685.476 | 6.924 |
| 18 | UL L2 ACK (4 bursts Tx). Note: PDAN sent according to RRBP. | 10613.667 | 2.308 |
| 19 | PDCH Idle (Light sleep for 6 bursts after each of 4 UL PACCH bursts = 24 bursts). 1 TS used for USF decoding and 1 TS used for MS transmission per TDMA frame | 45.7 | 13.848 |
| 20 | TFI monitoring . TFI/USF monitoring during UL L2 ack transmission | 228.492 | 2.308 |
| 21 | UL RLC PDUs (6 RLC PDU’s for entire report) | 63682 | 13.8848 |
| 22 | PDCH Idle (Light sleep for 6 bursts after each of 4 PDCH burst transmissions = 24 bursts for each UL RLC PDU, applies to each of the 6 UL RLC PDUs) | 274.19 | 83.088 |
| 23 | DL TFI/USF monitoring during the transmission of 6 UL PDU’s (1 slot per frame) | 1370.95 | 13.848 |
| 24 | PDCH Idle for around 10 TDMA frames after transmission of 6th RL PDU (until reception of PUAN excluding the time for TFI/USF monitoring) | 133.254 | 40.38 |
| 25 | TFI/USF monitoring till the reception of DL L2 ACK on PACCH | 571.23 | 5.77 |
| 26 | DL L2 ACK (4 bursts Rx) | 228.492 | 2.308 |
| 27 | PDCH Idle (Light sleep for 7 bursts after each of 4 DL PACCH bursts = 28 bursts) | 53.312 | 16.156 |
| 28 | Idle (light sleep 7 of 8 TS for 100 TDMA frames until the DL Application Ack/TCP Ack) | 1332.54 | 403.8 |
| 29 | All USF/TFI monitoring for 1 of 8 TS for 100 TDMA frames until the DL Application Ack/TCP Ack | 5712.3 | 57.7 |
| 30 | DL TCP Ack/Application Ack (4 DL RLC PDU’s) | 913.968 | 9.232 |
| 31 | PDCH Idle (Light sleep for 7 bursts after each of 4 PDCH burst receptions for each DL RLC PDU = 28 bursts) * 4 occurrences | 213.248 | 64.624 |
| 32 | PDCH Idle (Light sleep while waiting to send UL L2 Ack after 13 TDMA frames as indicated by the RRBP field). Note: MS in light sleep for 7 of 8 DL TS over 12 TDMA frames and for all 8 TS on the 13th TDMA frame) | 175.1739 | 53.083 |
| 33 | TFI monitoring during this time. | 685.476 | 6.924 |
| 34 | UL L2 ACK (4 bursts Tx) Note: PDAN sent according to RRBP. | 10613.667 | 2.308 |
| 35 | PDCH Idle (Light sleep for 7 bursts after each of 4 UL PACCH bursts = 28 bursts) | 53.312 | 16.156 |

Total Energy Consumed for the sending a network triggered report (completed in around1.1 s) is113.2 mJ. The configuration assumption for the energy consumption assumes the following:

- The UL transfer rate is 120 bytes and DL is 80 bytes per report in addition to 60 bytes of trigger.

- UE would never move out of Packet transfer state throughout the duration of procedure and T3192 value=0.

- No UL or DL dummy control block sent or received

- MCS-1 used for all data transfer UL and DL.

- The DL trigger uses UDP or similar connectionless protocol

- During the Idle time of 100 TDMA frames after sending the TCP Packets ,it is assumed that DL PACCH is monitored every frame, but for practical simplicity the power consumption is shown at the end of sleep period continuously

The resources assigned by an Immediate Assignment message are only sufficient for sending a page response and as such a subsequent Packet Timeslot Reconfigure message is sent to provide DL TBF resources appropriate for sending a trigger to a MTC device and UL TBF resources appropriate for receiving a report from a MTC device.

The plot of Power consumed Vs Time is depicted in Figure 2 below.

Figure 6.5-y: Power Consumption sequence for Data Transfer (Network triggered use case)

The Energy Consumption for Measurements are ignored as per 3GPP TS45.002. Measurements are done only when MS is doing some activity (TX or RX). The rationale is that the energy/power consumed for measurements during this procedure in Packet transfer mode is not that significant.

#### 6.5.2.2 	Energy Consumption for Mobile Autonomous Reporting

In table 6.5-z the assumptions on the data traffic for the Mobile Autonomous Traffic model are summarized. For Mobile Autonomous report, Report gets reported either periodically or based on an internal trigger.

The sequence of steps is explained in figure below.

Figure 6.5-z: Mobile Autonomous reporting procedure

One data transfer procedure using Mobile Autonomous reporting can be broken down into several basic operations as specified in Table 6.5-d with the assumption of 120 bytes UL data and 80 Bytes DL data (For TCP ACK or application level Acks).  Details of the Energy Consumed for individual sub process with time taken for each are also included in the table. The below table assumes the max TX power for transmission (33 dBm) and MCS-1 being used throughout.

Table 6.5-d: Energy Consumption for Mobile Autonomous Reporting

|  | Sub Procedure | Energy Consumed (µJ) | Time taken(ms) |
| --- | --- | --- | --- |
| 1 | RACH | 2653.417 | 0.577 |
| 2 | AGCH Idle(Full CCCH) (light sleep for 80 bursts) | 148.5 | 45 |
| 3 | Immediate Assignment(4 bursts) | 228.492 | 2.308 |
| 4 | AGCH Idle (Light sleep for 7 bursts after each of 4 AGCH bursts = 28 bursts | 53.312 | 16.156 |
| 5 | PDCH Idle (light sleep for 1 frame 28in between monitoring 1st USF) | 53.312 | 16.152 |
| 6 | USF Monitoring for sending 1st UL PDU | 228.492 | 2.308 |
| 7 | Packet TS Reconfigure(establishing 1 TS in both UL and DL TBF) | 228.492 | 2.308 |
| 8 | PDCH Idle (Light sleep in between reception of PTR) | 53.312 | 16.156 |
| 9 | UL PDU (6 PDU’s) | 63682 | 13.8848 |
| 10 | PDCH Idle (Lightsleep for 6 bursts after each of 4 PDCH burst transmissions = 24 bursts for each UL RLC PDU, applies to each of the 6 UL RLC PDUs) | 274.19 | 83.088 |
| 11 | DL TFI monitoring during the transmission of 6 UL PDU’s | 1370.95 | 13.848 |
| 12 | PDCH Idle for around 10 TDMA frames after transmission of 6th RLC PDU (until reception of PUAN excluding the time for TFI/USF monitoring) | 133.254 | 40.38 |
| 13 | TFI/USF monitoring till the reception of DL L2 ACK on PACCH | 571.23 | 5.77 |
| 14 | DL PACCH ACK (4 bursts Rx) | 228.492 | 2.308 |
| 15 | PDCH Idle (Light sleep for 7 bursts after each of 4 DL PACCH bursts = 28 bursts)* 3 Occurances | 53.312 | 16.156 |
| 16 | Idle (light sleep for 100 TDMA frames till getting Application/TCP Acks) | 1332.54 | 403.8 |
| 17 | All USF/TFI monitoring till the DL Application/TCP acks | 5712.3 | 57.7 |
| 18 | DL TCP ACK /Application Acks 4 RLC PDU’s) | 913.968 | 9.232 |
| 19 | PDCH Idle (Light sleep for 7 bursts after each of 4 PACCH bursts = 28 bursts) * 4 occurrences | 213.248 | 64.624 |
| 20 | PDCH Idle (Light sleep while waiting to send UL L2 Ack after 13 TDMA frames as indicated by the RRBP field). Note: MS in light sleep for 7 of 8 DL TS over 12 TDMA frames and for all 8 TS on the 13th TDMA frame) | 175.1739 | 53.083 |
| 21 | TFI monitoring for 12 frames | 685.476 | 6.924 |
| 22 | UL L2ACK (4 bursts Tx) Note: PDAN sent according to RRBP | 10613.417 | 2.308 |
| 23 | PDCH Idle (Light sleep for 7 bursts after each of 4 DL PACCH bursts = 28 bursts) | 53.312 | 16.156 |

Total Energy Consumed for the sending a Mobile Autonomous report (completed in around 880 ms) is 89.6 mJ. The configuration assumption for the figure below is same as that reported for Network triggered reporting.

## Figure 6.5-e.  Power Consumption for Mobile Autonomous Reporting

Seventh modification (added sub-clause)

### 7.1.2	eDRX vs. PSM evaluation

The eDRX functionality will for a given reachability almost always consume less than the energy consumed by using PSM. This due to the energy saved when the device only needs to listen to paging instead of transmitting a keep-alive message or transmitting a RAU on a periodic basis.

In Figure 7.1-1 battery lifetime versus reachability is plotted when the shorter sync procedure is used. It can be seen that the lowest possible reachability periodicity for PSM that allows for realizing target battery lifetime of 10 years is 45 minutes when at the same time only sending a report at a maximum rate of approximately once per hour. Longer battery lifetime can be achieved in the PSM case by increasing the reachability periodicity and increasing the triggering interval.  As can be seen for PSM in Figure 7.1-1 the lowest reachability periodicity allowed to still meet the target battery lifetime of 10 years is approximately 45 minutes irrespective of how much the triggering interval is increased beyond approximately 45 minutes. However, when using eDRX it is possible to achieve 10 year battery lifetime with a reachability periodicity down to approximately 90 seconds by increasing the triggering interval (see Figure 7.1-1). As an example, when using eDRX it is possible to use a reachability periodicity of every 5 minutes with a triggering interval of once every other hour  and still meet the target of 10 years battery lifetime(green dashed curve).

Figure 7.1-1: Battery lifetime versus reachability using the short sync (100% short sync success rate).

To evaluate the robustness of the shorter sync procedure a failure of the shorter synchronization procedure is taken into account. The evaluation is done for a failure rate of five percent. When the shorter sync fails the long sync procedure will have to be used and more energy is consumed before the device can receive a page (eDRX) or transmit an access request (PSM). It can be seen that both power saving modes are affected by the failure rate. For PSM most energy is consumed by its reachability, i.e. sending “keep alive” or RAU updates, to trigger the Ready State procedure and the curves in Figure 7.1-2 gives slightly worse performance than without sync failure as can be seen in Figure 7.1-1. For eDRX a small degradation in battery lifetime can be seen when comparing between Figure 7.1-1 and Figure 7.1-2. In the eDRX case the energy consumed by being reachable is due to sync and listening for a page. The failure rate of the shorter sync procedure will therefore have a greater impact on eDRX when compared to PSM. Never the less, in the case we want to reach the device more often than its triggering interval, eDRX will always outperform PSM even when a failure rate of the sync procedure is added, see Figure 7.1-2.

Figure 7.1-2 Battery lifetime versus reachability using the short sync (95% short sync success rate)

End of modifications

# References

[1] GP-150127, “uPoD Paging Group Determination”, source Ericsson LM, TSG GERAN #65.