Pseudo CR 45-820

C-UNB System overview

# Introduction

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62 [1]. This study covers both GSM/GPRS evolution and clean slate concepts. This pCR deals with the clean slate concept named C-UNB [2].

# Change proposal

This pCR deals with text proposal for insertion in section 7.2 on clean slate concept #2.

More precisely, this pCR is for section 7.2.1 on system overview.

# Reference

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT

---------------------------------------------------------------------------------------------------------

beginning of text proposal for insertion in TR 45.820

--------------------------

### 7.2.1 General description

The legacy GSM/GPRS network is mainly designed to address human-to-human services such as voice calls or Internet browsing. With the rapid growth of CIoT devices, cellular networks have to cope with a new kind of service where massive number of devices have very low throughput. Along with this new requirement, CIoT devices are often deployed in buildings or in buried environment, hence requiring better radio coverage.

The C-UNB concept is a clean slate approach that relies on a disruptive radio access network, designed to address CIoT traffic and coverage with very low complexity in the device modem. The C-UNB core technologies are: random transmission for uplink, ultra narrow-band modulation, and cooperative reception.

As the throughput of CIoT devices is very low, there is no need for high-speed modulation rate. C-UNB implements uplink and downlink data rates of a few 100s Hertz, which are called micro-channels here after. This ultra-narrow band transmission allow high link budget without adding complexity in the transmitter or the receiver.

C-UNB implement random access for uplink transmission: a device willing to transmit a radio packet selects three "adhoc micro-channel" and transmits its radio packet, one time on each selected radio ad'hoc micro-channel. When doing that, the device is not attached to any given base station nor synchronised with any timeslot defined by the network. Hence power consumption in the device is largely reduced.

As random transmissions create collisions, the C-UNB concept manages them by spreading the UL transmission over time, frequency and space. Time spreading is obtained because devices are not synchronised. Frequency spreading is given by the large number of ultra-narrow band channels available in a 200Khz resource block. Given the bandwidth of about 160Hz per micro-channel. In a 200kHz block, more than 1000 micro-channel can exist.

Space spreading is obtained through the base stations themselves: they listen the full bandwidth continuously and decode all radio packets. Therefore, one given radio packet may be received and decoded by more than one base station. If there is a collision detected by a station, the probability to have a collision detected by another base station located in a different place is very low. This is called "co-operative reception".

--------------------------

end of text proposal for inclusion in TR 45.820

---------------------------------------------------------------------------------------------------------