Pseudo CR 45-820

C-UNB Physical layer description

# Introduction

A study on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things was approved at GERAN#62 [1]. This item covers both GSM/GPRS evolution and clean slate concepts. This pCR deals with the clean slate concept named C-UNB [2].

# Change proposal

This pCR deals with text proposal for inclusion in section 7.2 on clean slate concept #2.

More precisely, this pCR is for section 7.2.2 on Downlink physical layer design and section 7.2.3 on Uplink physical layer design.

# Reference

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT

---------------------------------------------------------------------------------------------------------

beginning of text proposal for inclusion in TR 45.820

--------------------------

### 7.2.2 Downlink physical layer design

#### 7.2.2.1 Bit rate

The downlink bit rate is set to 600bps in order to reduce the reception time in UEs.

#### 7.2.2.2 Modulation

The proposed modulation for C-UNB downlink transmission is 2GFSK, BT = 1.0, F = +/- 800Hz. It is chosen because GFSK demodulators are commonly available in low cost sub-gigahertz radio chipsets. The bandwidth is 600Hz.

#### 7.2.2.3 Transmission power

The sensitivity of GFSK receivers in UEs is slightly lower than BPSK receivers in base stations, therefore the transmission power on each ad'hoc micro-channel can be set up to +34dBm. In practice, actual transmission power in downlink is adjusted according to uplink-received signal strength.

#### 7.2.2.4 Downlink frequency

The center frequency of each ad'hoc micro-channel is chosen within the legacy 200kHz band corresponding to the uplink frequency band. The precise center frequency of a given ad'hoc micro-channel for downlink is not known a priori. It is calculated for each downlink message; the precise value is derived from actual uplink transmission frequency (see figure 1).

Figure 1: Relationship between frequencies of corresponding UL and DL micro-channels

#### 7.2.2.5 Downlink MCL

Table 2 gives the maximum coupling loss (MCL) of the downlink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 of TR 36.888 [4].

Table 2: Maximum Coupling Loss in C-UNB downlink

| Logical channel name | C-UNB downlink |
| --- | --- |
| Data rate (bps) | 600 |
| Transmitter |  |
| (1) Tx power (dBm) | +34 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 7 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 600 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -139 |
| (7) Required SINR (dB) | 9 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -130 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

### 7.2.3 Uplink physical layer design

#### 7.2.3.1 Bit rate

The requirements for improved coverage and ultra low power UEs imply many tradeoffs when designing the air interface. In C-UNB, the tradeoff consists in using extremely low data rates. These data rates allow ultra narrow band transmission, and hence very good reception sensitivity, with no need of complex error correction codes. Here, it is proposed to use a rate of 160 bps, which is compliant with the requirement stated in [3].

#### 7.2.3.2 Modulation

The modulation scheme is pure D-BPSK. The binary phase shift keying is very robust against noise and hence gives very good sensitivity with limited complexity. BPSK is natively available in several off-the-shelf sub-gigahertz radio chipsets. In the other chipsets, it is easily implemented with modified FSK modulation.

#### 7.2.3.3 Transmission power

The transmission power is a key parameter in IoT UEs because it sets most part of the battery life. Considering the very good selectivity and dynamic of C-UNB receivers in base stations, it is proposed to keep the power transmission constant (no power management depending on the actual link budget between a UE and a base station). Il is proposed to set the power transmission at +24dBm in C-UNB UEs.

#### 7.2.3.4 Transmission frequency

In Cellular IoT, the objective is to re-use the 200kHz legacy frequency bands for dedicated IoT traffic. In the case of C-UNB technology, the legacy frequency band is divided in "micro-channels" of 160Hz. The center frequency of each micro-channel is not precisely defined in the specifications, because C-UNB base stations listen for the full 200kHz bandwidth continuously. Within a 200kHz band, they are able to cope with micro-channels used by different UEs, even if the center frequencies drift a bit. This feature allows using standard crystal oscillator in C-UNB modem instead of costly TCXO. These micro-channels are named "ad'hoc micro-channels" in C-UNB technology.

#### 7.2.3.5 Uplink MCL

Table 1 gives the maximum coupling loss (MCL) of the uplink transmission in C-UNB. Calculation is based on methodology given in subclause 5.1 of TR 36.888 [4].

Table 1: Maximum Coupling Loss in C-UNB uplink

| Logical channel name | C-UNB uplink |
| --- | --- |
| Data rate (bps) | 160 |
| Transmitter |  |
| (1) Tx power (dBm) | +24 |
| Receiver |  |
| (2) Thermal noise density (dBm/Hz) | -174 |
| (3) Receiver noise figure (dB) | 3 |
| (4) Interference margin (dB) | 0 |
| (5) Occupied channel bandwidth (Hz) | 160 |
| (6) Effective noise power = (2) + (3) + (4) + 10 log ((5)) (dBm) | -148 |
| (7) Required SINR (dB) | 8 |
| (8) Receiver sensitivity = (6) + (7) (dBm) | -140 |
| (9) Rx processing gain | 0 |
| (10) MCL = (1) (8) + (9) (dB) | +164 |

--------------------------

end of text proposal for inclusion in TR 45.820

---------------------------------------------------------------------------------------------------------