C-UNB for Cellular IoT

-

Radio Resource Management

# Scope

At GERAN#62, a new feasibility study on Cellular Internet of Things (CIoT) was approved [1]. One of the proposals in the Cooperative Ultra Narrow Band (C-UNB). Overall description of the C-UNB radio access technology can be found in [2]. This document describes the radio resource management in C-UNB. It is a revision of document [3].

# Radio design principles

CIoT assumes that one or more 200kHz frequency blocks from the legacy GSM service are re-allocated for the Internet of things. In a given country, it is expected that each MNO is free to allocate one or many frequency blocks out of its spectrum allocation given by the local regulator. Therefore, frequency blocks for Cellular IoT are chosen and planned using common cellular radio planning and re-use patterns.

In the case of C-UNB, block allocation is a bit different because all C-UNB base stations have to listen on the same 200kHz blocks. As the C-UNB devices are not attached nor synchronized to the network, they all transmit on the same frequency block and receive in the corresponding block. This feature, already described in [2], improves the robustness of UL transmissions because it creates multiple receptions, acting as a kind of space diversity. As a consequence, there is no need for a frequency re-use pattern because all base stations use the same block for C-UNB (see figure 1)

Figure 1: Frequency allocation in GSM cluster (a) and in C-UNB (b)

# Beacon channel

## Beacon channel frequency and content

In C-UNB, devices have to be aware of the 200kHz block allocated to the CIoT service by their respective MNOs. Frequency block allocation, which is specific to each MNO, is obtained by listening for the C-UNB beacon channel.

C-UNB beacon channel is transmitted by all base stations. It uses a UNB micro-channel in the center of the 200kHz block reserved for CIoT (see figure 2). It contains network parameters such as PLMN, time reference, local loop-back delay, wake-up packet periodic transmission time, … It is assumed that all these system parameters can sent in one single radio packet, i.e. within about 600ms.

Radio parameters of the beacon channel are the same as other DL micro channels: 600bps and +34dBm (to get the maximum coverage range).

Figure 2: C-UNB beacon channel located in center of CIoT frequency block

## Beacon channel cyclic transmission

In cellular networks, interference between neighboring base stations is managed with frequency re-use patterns, based on hexagonal tiles. This frequency diversity is not appropriated to C-UNB beacon channel because all base stations transmit their beacon channel on the same C-UNB frequency block. C-UNB beacon channel uses time diversity to avoid interference between base stations.

Beacon frames are transmitted in sequence by all base stations of a cluster (see figure 3). The length of the beacon cycle depends of the level of interferences between base stations: the more interference, the larger the cluster and the longer the cycle.

Nevertheless, the maximum delay for a given device for receiving the beacon frame is not the full beacon cycle, because of the large overlap of C-UNB base station coverage (see figure 3).

The synchronization between base stations is based on a common time reference to avoid time shifting between base stations. Beacon transmission occurs in time slots that can be defined with a low precision of about +/-100ms.

Figure 3: Multiple beacon frames reception due to large coverage overlaps

# Paging

In cellular network, paging alerts devices in case of network-triggered DL transmission. Paging implies time synchronization and regular wake-ups of devices, all functions that are quite energy consuming. In C-UNB, it is proposed to have only device-triggered downlink transmission. Network-triggered transmission occurs only when a device wakes up.

Upon wake-up, a C-UNB device transmits an up-link packet to check for downlink messages waiting in the network; then it opens a reception window. If it receives nothing, it enters sleep mode, immediately at the end of the reception window. If there is a DL packet waiting for transmission, it is transmitted by the network (see figure 4) and received by the corresponding device.

The time between to wake-up packets is set by the MNOs. It is a tradeoff between maximum affordable latency in downlink and the reduction in battery life. It acts as eDRX intervals, but with larger values (i.e. up to several hours).

Figure 4: periodic wake-up and DL packet reception

# Cell selection

In legacy GSM/GPRS networks, cell selection/reselection is a major radio feature because it provides radio parameters for the device attachment procedures. C-UNB radio access technology don't need any cell selection, because C-UNB devices are not attached to any base station. In uplink, all base stations, in the communication range of a C-UNB device, can receive the radio packets. In downlink, one of the base stations, which have just received a UL packet from the device, is selected by the network for transmitting the DL radio packets. The selected base station for DL transmission may vary depending on the network load and the overall interference levels received by the network.

Nevertheless, the cell selection feature is still implemented in the C-UNB architecture [4] in order to be compliant with Gb architecture. This is done by the MS-Avatar of each C-UNB device.

In the cell selection process, the MS-Avatar declares only one received BTS (i.e. the closest one) and no neighbor cells (see figure 5). Considering the only cell viewed by the MS-Avatar, the core network can run the attachment procedure and forthcoming mobility management as it is in Gb architecture.

Figure 5: MS-Avatar cell selection and attachment

# Conclusion

This contribution details some aspects of the radio resource management in C-UNB radio access technology (RAT). It shows how C-UNB RAT can support RRM procedures of the Gb architecture, without major impact on the core network.

# References

[1]	GP-140421, “New Study Item on Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”, VODAFONE Group Plc., GERAN#62, 26-29 May'14, Valencia.

[2]	GPC150052, "Cooperative ultra narrow band technology for Cellular IoT", Sigfox, Ad'hoc meeting, 02-05 Feb'15, Sophia Antipolis, France.

[3]	GPC150146, "C-UNB technology of cellular IoT - Radio Resource Management", SIGFOX, GERAN ad'hoc meeting 2, 20_23 April 2015, Sophia Antipolis, France

[4]	GP-150368, "C-UNB architecture", SIGFOX, GERAN#66 meeting, 25-30 May'15, Vilnius, Lithuania.