# Pseudo CR 45.820: NB M2M – Battery Life Analysis

# Introduction

This document provides some new text for the NB M2M candidate solution in the Technical Report relating to battery life analysis. This document is a re-submission of GPC-150136 with no revisions except changing “UE” to “MS”.

# Proposed text for the TR

Start of Changes

## 7. 1 Narrow Band M2M (NB M2M)

### 7.1.6 	Concept Evaluation

### 7.1.6.4 	Energy Consumption Evaluation

This subclause provides an analysis of the MS battery life that can be achieved with the NB M2M solution following the evaluation methodology described in subclause 5.4.

##### 7.1.6.4.1 	Assumptions

The instantaneous power consumption assumptions for the major operating modes of the NB M2M MS are shown in Table 7.1.6.4-1.

Table 7.1.6.4-1: Power consumption assumptions for NB M2M battery life analysis

| Operating mode |  | Power (mW) | Notes |
| --- | --- | --- | --- |
| Transmit (+23 dBm) | Integrated PA | 500 | +23 dBm with 45% PA efficiency for class B (including Tx/Rx switch insertion loss) plus 60 mW for other circuitry. |
|  | External PA | 460 | +23 dBm with 50% PA efficiency for class B (including Tx/Rx switch insertion loss) plus 60 mW for other circuitry. |
| Receive | Synchronization (PSS/SSS) | 70 | Accounts for more complex digital processing during synchronization. |
|  | Normal (non-PSS/SSS) | 60 | Includes digital mixing/decimation to single 15 kHz sub-channel, and subsequent demodulation of this sub-channel. |
| Sleep |  | 3 | Corresponds to maintaining accurate timing by keeping RF frequency reference active. |
| Standby |  | 0.015 | Common assumption. |

The NB M2M protocol flow that is assumed for the battery life analysis is illustrated in Figure 7.1.6.4-1.

Figure 7.1.6.4-1: Protocol flow assumptions for NB M2M battery life analysis

The assumed DCI intervals are 80 ms for normal coverage (144 dB coupling loss), 320 ms for extended coverage (154 dB coupling loss), and 1280 ms for extreme coverage (164 dB coupling loss). Potential re-transmissions of the uplink report are shown, including an additional DCI for the MAC layer ACK associated with the uplink re-transmission.

The average time taken for network synchronization and for system information reading is shown in Table 7.1.6.4-2. It is assumed that the MS has not moved to a different cell sector, and that only SI1 needs to be read (so SI2 to SI4 are assumed to be unchanged since the previous reception). No downlink PSD boosting is assumed for PBSCH.

Table 7.1.6.4-2: Receiver active time for synchronization and SI reading

|  | Coupling loss = 144 dB |  | Coupling loss = 154 dB |  | Coupling loss = 164 dB |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #frames latency | Rx active time (ms) | #frames latency | Rx active time (ms) | #frames latency | Rx active time (ms) |
| PSS & SSS | 2 | 160 | 4 | 320 | 9 | 720 |
| FIIS & SI1 | 5 | 186 | 6 | 224 | 12 | 448 |

The selection of MCS and CBS for each burst type at each coupling loss is shown in Table 7.1.6.4-3, including the resulting burst durations.

Data bursts include a 15 byte overhead in addition to the packet size above SNDCP (4 bytes for SNDCP, 6 bytes for LLC, 2 bytes for MAC header, and 3 bytes for CRC), and the first uplink burst after RACH uses an additional 5 bytes for TLLI.

The DCI burst size is a typical value for a loaded system, and so carries resource allocation information for multiple MSs. No PSD boosting or adaptive power allocation has been assumed on the downlink.

Table 7.1.6.4-3: MCS and CBS selection for each burst type

|  |  | Coupling loss = 144 dB |  |  | Coupling loss = 154 dB |  |  | Coupling loss = 164 dB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Burst type | PHY burst size | MCS Index | CBS index | Duration (ms) | MCS index | CBS index | Duration (ms) | MCS index | CBS index | Duration (ms) |
| DL DCI | DCI (34 to 48 bytes) | 7 | 1 | 30 | 4 | 4 | 120 | 2 | 4 | 480 |
| DL RAR | RAR (9 bytes) | 5 | 1 | 20 | 3 | 1 | 80 | 1 | 1 | 320 |
| DL data | App ACK (29+15 = 44 bytes) | 6 | 3 | 40 | 4 | 7 | 160 | 2 | 7 | 640 |
| UL random access | RACH (5 bytes) | 5 | 0 | 40 | 5 | 0 | 40 | 1 | 0 | 320 |
| UL data (50 bytes) | Short report (50+15+5 = 70 bytes) | 9 | 3 | 40 | 6 | 7 | 320 | 3 | 15 | 1920 |
| UL data (200 bytes) | Long report (200+15+5 = 220 bytes) | 9 | 11 | 120 | 6 | 23 | 960 | 4 | 47 | 3840 |
| UL ACK of DL data | MAC layer ACK (5 bytes) | 8 | 0 | 10 | 5 | 0 | 40 | 1 | 0 | 320 |

The impact of re-transmissions of the uplink reports is included in the battery life analysis by taking account of the simulated BLER for the initial transmission of the uplink report for each scenario. The analysis allows for an additional uplink transmission plus an additional reception of a DCI containing the MAC layer ACK, as illustrated in Figure 7.1.6.4.1-1. This means that the battery life analysis takes account of the average number of retransmissions of the uplink report. The effect of BLER on channels other than PUSCH is not considered.

##### 7.1.6.4.2 	Results

The achievable battery life in years has been estimated as a function of reporting frequency and coupling loss, based on the previously stated assumptions. The results for an integrated PA are summarized in Table 7.1.6.4-4 and for an external PA in Table 7.1.6.4-5. In both cases, the transmit power from the MS is constrained to be +23 dBm (200 mW) to ensure compatibility in terms of peak current with a wider range of battery technologies, and the frequency re-use assumption is compatible with a stand-alone deployment in a system bandwidth for the entire network of just 200 kHz (FDD).

Table 7.1.6.4-4: Battery life estimates with integrated PA

|  | Battery life (years) |  |  |
| --- | --- | --- | --- |
| Packet size, reporting interval | Coupling loss = 144 dB | Coupling loss = 154 dB | Coupling loss = 164 dB |
| 50 bytes, 2 hours | 21.3 | 9.5 | 2.3 |
| 200 bytes, 2 hours | 17.5 | 5.5 | 1.5 |
| 50 bytes, 1 day | 35.7 | 30.5 | 16.6 |
| 200 bytes, 1 day | 34.7 | 25.5 | 12.4 |

Table 7.1.6.4-5: Battery life estimates with external PA

|  | Battery life (years) |  |  |
| --- | --- | --- | --- |
| Packet size, reporting interval | Coupling loss = 144 dB | Coupling loss = 154 dB | Coupling loss = 164 dB |
| 50 bytes, 2 hours | 21.7 | 10.0 | 2.5 |
| 200 bytes, 2 hours | 18.0 | 5.8 | 1.6 |
| 50 bytes, 1 day | 35.8 | 30.8 | 17.2 |
| 200 bytes, 1 day | 34.8 | 26.0 | 13.0 |

##### 7.1.6.4.3 	Conclusions

The achievable battery life for a MS using the NB M2M solution for Cellular IoT has been estimated as a function of reporting frequency and coupling loss.

It is important to note that these battery life estimates are achieved with a system design that has been intentionally constrained in two key respects:

- The NB M2M solution has a frequency re-use assumption that is compatible with a stand-alone deployment in a minimum system bandwidth for the entire IoT network of just 200 kHz (FDD), plus guard bands if needed.

- The NB M2M solution uses a MS transmit power of only +23 dBm (200 mW), resulting in a peak current requirement that is compatible with a wider range of battery technologies, whilst still achieving the 20 dB coverage extension objective.

The key conclusions are as follows:

- For all coupling losses (so up to 20 dB coverage extension compared with legacy GPRS), a 10 year battery life is achievable with a reporting interval of one day for both 50 bytes and 200 bytes application payloads.

- For a coupling loss of 144 dB (so equal to the MCL for legacy GPRS), a 10 year battery life is achievable with a two hour reporting interval for both 50 bytes and 200 bytes application payloads.

- For a coupling loss of 154 dB, a battery life of 9.5 to 10 years can be achieved with a 2 hour reporting interval for a 50 byte application payload. This could be further improved by exploiting adaptive power allocation on the downlink, using frequency hopping to smooth the PSD over time, but it is not clear whether this is allowed by the common assumptions.

- For a coupling loss of 154 dB with 200 byte application payload, or a coupling loss of 164 dB with 50 or 200 byte application payload, a 10 year battery life is not achievable for a 2 hour reporting interval. This is a consequence of the transmit energy per data bit (integrated over the number of repetitions) that is required to overcome the coupling loss and so provide an adequate SNR at the receiver.

- Use of an integrated PA only has a small negative impact on battery life, based on the assumption of a 5% reduction in PA efficiency compared with an external PA.

Further improvements in battery life, especially for the case of high coupling loss, could be obtained if the common assumption that the downlink PSD must not exceed that of legacy GPRS was either relaxed to allow PSD boosting, or defined more precisely to allow adaptive power allocation with frequency hopping.

End of Changes